Implementation scenarios for ARTS

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Eric Kooistra | ASTRON | 2015 |
| Controle / Checked: |  |  |
| Joeri van Leeuwen | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Joeri van Leeuwen | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature Joeri van Leeuwen | ASTRON |  |
| © ASTRON 2015 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Jeanette Bats Joeri van Leeuwen Roy Smits Alessio Sclocco Gert Kruithof Andre Gunst Stefan Wijnholds | Wim van Capellen Hajee Pepping Daniel van der Schuur Alwin Zanting |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2015-08-5 | E. Kooistra | Creation |
| 0.2 | 2015-08-13 | E. Kooistra | - Processed review comments from Stefan and Allesio. - Added Table 6 with preliminary design choices that were made for Arts. - Added Table 12 with an overview of the scenarios and section 2.8 that provides a conclusion - Added scenario 2b with 8 UniBoard2-A10 - Added section 6.3.3 on external memory operation. - Removed cost for HEM, because HEM is only nice to have in the scenarios with UniBoard2. - Updated cost estimate for UniBoard2 with Stratix10 FPGAs. - UniBoard2 can fit 32 GByte DDR4 modules (section 4.4.2, 6.3) - Clarified that SC2 only needs the Apertif X functionality for the central CB (section 6.1). - Added cost estimates for the Arts PL (section 2.6). |

Table of contents:

1	Introduction	7

1.1	Purpose	7

1.2	Scope	7

2	Scenarios	8

2.1	Overview of the Arts data path	8

2.2	Apertif BF	8

2.3	Transpose Tsp	9

2.4	Arts FPGA beam former (BF)	9

2.4.1	Data processing functions	9

2.4.2	Transient data storage	9

2.5	Transpose Tband	10

2.6	Arts pipeline GPU cluster	10

2.7	Hardware scenarios	11

2.7.1	Preliminary design choices	11

2.7.2	Scenarios for the Arts FPGA beamformer	11

2.7.3	Evaluation of the scenarios	13

2.7.4	Arts SC1 hardware infrastructure in scenario 1	15

2.7.5	Arts SC4 hardware infrastructure in scenario 1	15

2.7.6	Arts SC2 and SC4 hardware infrastructure in scenario 4	16

2.8	Conclusion	16

3	System overview	18

3.1	Apertif	18

3.2	Arts	18

3.3	Interfaces	19

3.3.1	Apertif BF output	19

3.3.2	Arts FPGA BF output	21

4	Uniboard hardware	23

4.1	UniBoard and OEB	23

4.2	UniBoard2 and HEM	23

4.3	Control interface and data offload via 1GbE	24

4.4	Comparison	25

4.4.1	Optical links	25

4.4.2	External memory	25

4.4.3	Processing resources	25

4.4.4	Processing load estimates for Apertif X	26

4.4.5	Processing load estimates for Arts SC3	26

4.4.6	Processing load estimate per TAB	27

4.4.7	Hardware cost	27

4.4.8	Power consumption and cost	28

4.5	Conclusion	29

5	GPU cluster workstations	30

5.1	Input data	30

5.2	Transient data buffer	30

5.3	GPU cluster	30

5.3.1	Input data rate per work station	30

5.3.2	Transient data buffer storage	30

5.3.3	Tband data transpose to bring together the 300 MHz band	30

5.3.4	Pipeline (PL) processing	31

6	Critical system requirements and hardware constraints	32

6.1	Commensal modes	32

6.2	Fringe stopping	32

6.3	Transient data buffer	32

6.3.1	CB-444 voltage data	32

6.3.2	TAB-444 integrated power data	33

6.3.3	External memory operation	33

6.3.4	External memory in Arts	33

6.4	Apertif X integation interval transpose Tint in the Apertif BF	34

6.5	Transpose Tsp	35

6.5.1	On UniBoard	35

6.5.2	Load on the UniBoard mesh	35

6.5.3	Using UniBoard2	35

6.6	Channel band width and time resolution of the Stokes beam data	36

6.7	Streaming output full Stokes or output only Stokes I data	36

6.8	Number of bits per sample	36

6.8.1	Data packing and unpacking	37

6.9	Duplicate Apertif BF output	37

6.10	Pass on Apertif BF output in daisy chain	37

6.11	Using the same 16 UniBoards of Apertif X also for Arts	38

6.12	Using 16 dedicated UniBoards for Arts	39

6.13	Using more than 16 dedicated UniBoards for Arts processing	39

6.14	Using more than 4 dedicated UniBoard2s for Arts processing	39

6.15	Output redistribution inside the Arts BF	40

6.15.1	Via 1GbE	40

6.15.2	Via the UniBoard mesh for 10GbE output	40

6.16	Output data reorder	40

6.16.1	In time	40

6.16.2	Per final destination	40

6.17	Unidirectional 10GbE links	41

6.17.1	Using UniBoard2 to convert unidirectional 10GbE to full duplex 10GbE	41

6.17.2	Using UniBoard2 for processing	41

6.18	Transpose Tband	42

6.18.1	TAB-1 for SC1 using 1GbE and a dedicated switch	42

6.18.2	TAB-444 for SC4 via 10GbE	42

6.18.3	CB-12 and TAB-12 for SC2	44

6.18.4	IAB-37 for SC3	44

6.18.5	Transient buffer data readout for SC3 and SC4	44

7	Hardware status	45

7.1	UniBoard	45

7.2	UniBoard2	45

7.3	GPU cluster	45

References:

- “Arts Requirements Specification”, ASTRON-RS-020, J. van Leeuwen

- “Analysis of tied-array beamforming for Arts”, ASTRON-MEM-191, S.J. Wijnholds

- “Impact analysis of change request to use 1 MHz beamlets instead of 0.78125 MHz beamlets in Apertif and Arts”, ASTRON-CR-032, E. Kooistra

- “Apertif Beamformer Output Interface Specification”, ASTRON-SP-061, E. Kooistra

- “Comparison of FPGA, GPU and network switch”, ASTRON-MEM-193, E. Kooistra

Terminology:

ADC		Analogue to Digital Conversion

ADU		Analogue to Digital Unit (board with 8 ADC)
Apertif		APERture Tile In Focus

Arts		Apertif Radio Transient System

beam		Group of beamlets that point in the same direction

beamlet		Beam formed subband, a small beam spanning one subband

BF		BeamFormer

BG		Block Generator

BN		Back Node FPGA on UniBoard

bps		Bits per second

BSN		Block Sequence Number (time stamp)

BW		BandWidth

CB		Compound Beam, formed at dish level over the FPA
channel		Unit frequency band within a beamlet

CoBI		Correlator Backplane Interface board (connects 8 Uniboards with 8 OEB)

DB		Data Buffer

DM		Dispersion Measure

DT		Delay Tracking

FF		Flip Flop

FIR		Finite Impulse Response (digital filter)
FN		Front Node FPGA on UniBoard

FoV		Field of View

FPGA		Field Programmable Gate Array

FS		Fringe Stopping (is DT + PT)

GbE		Gigabit Ethernet

Gbs		Gigabit per second

GPU		Graphics Processing Unit

HEM		HMC Extension Module (provides UniBoard2 with one HMC and extra optical IO per PN)

HMC		Hybrid Memory Cube
IAB		Incoherent array beam, formed by incoherently combining dishes

IO		Input Output

LE		Logic Element

LSbit		Least Significant bit

MAC		Multiply and Accumulate, Medium Access, Monitoring and Control

MM		Memory Mapped (control and status register interface)

MSbit		Most Significant bit

MTps		Mega transfers per second

NC		Not Connected

NIC		Network Interface Card for 10GbE

node		Processing node (PN), typically one FPGA chip
Nof		Number of

OEB		Optical-Electrical Board (provides UniBoard BN with same optical IO as the FN)

PAC		Power and Control board

power beam	Full Stokes power values: I, Q, U, V

PL		Pipeline processing
PN		Processing Node (FN or BN), PN0:3 = FN0:3, PN4:7=BN0:3

PT		Phase Tracking

QSFP		Quad Small Form-factor Pluggable transceiver (to connect a 4*10G link)
RF		Radio Frequency

SC		Science Case

SFP		Small Formfactor Pluggable transceiver (to connect a 10G link)
SP		Signal Path, 1 CB consists of Npol = 2 SP, 1 SP per Apertif BF subrack

SR		Science Requirement

sps		Samples per second

subband	Frequency band, unit output of the filterbank

TAB		Tied array beam, formed by coherently combining dishes
Tant		Transpose to group data from all S = 64 (≥ Nant) antenna elements in the FPA
Tdish		Transpose to group data from all Ndish = 12 dishes
Tpol		Transpose to group data from both Npol = 2 polarizations
Tsp		Transpose to group data from all Nsp = Npol * Ndish signal paths, so combines Tdish and Tpol 
Tband		Transpose to group data from all Nband = 16 bands

Tintegration	Transpose to group data from an integration interval of Nint_x values in time
TFoV		Transpose to group data from all NCB = 37 beams for the full FoV

VLBI		Very Long Baseline Interferometry
voltage beam	Dual polarization sample values with phase information:  Xre, Xim, Yre, Yim

WSRT		Westerbork Synthesis Radio Telescope
X		Correlator

Definitions:

Ncomplex	2		Two part of a complex number, the real and imaginary part 
Npol	2		Number of polarizations, X and Y
NStokes	4		Number of power values in the Stokes vector [I, Q, U, V]
Ndish	12		Number of WSRT dishes in Apertif
Nsp	24		Number of signal paths = Ndish * Npol at the output of the Apertif BF
CBBW	300 MHz	Full bandwidth of the CB and also of the TAB and IAB (SR-0.2)
Bsub	781250 Hz	Subband bandwidth in Apertif BF, = beamlet bandwidth
Nband	16		= nof_fn_bf, Number of bands in the Apertif BF to process the full CBBW
NCB	37		Required number of compound beams
Ngr	12		Required number of TAB grating lobe patterns to cover the full CB (SR-0.41)
NVLBI	12		Required number of TABs in the central CB for VLBI, choose = Ngr (SR-0.23)
KTAB	12		Implemented number of TABs per beamlet (≥ Ngr)

NTAB	444		= NCB*KTAB, number of TABs 
NIAB	37		= NCB, number of IABs 
Nlink	384		= NPN, number of physical 10G output links of the Apertif BF, so 1 link per PN
NPN	384		= Nsp * Nband, total number of parallel processing nodes in the Apertif BF
MPN	128		= Nband * nof_un, total number of parallel processing nodes in the Arts

Muni	16		= Nband = nof_fn_bf, total number UniBoards in the Arts BF and in Apertif X

Muni2	4		Total number UniBoard2 in the Arts BF

Nchan	4		Number of channels per beamlet, for SC3 and SC4
Bchan			= Bsub/Nchan, channel bandwidth within a beamlet, for SC3 and SC4

Nint	≈ 10		Number of Stokes channel power values that are integrated in Arts
TStokes	≈ 50 μs		Minimum required sample period for the Stokes power values
fStokes	≈ 20 kHz	= 1/TStokes, minimum required sample frequency for the Stokes power values
nof_uni		4	Number of UniBoards per polarization and dish in the Apertif BF

nof_bn		4	Number of back node FPGAs (BN) per UniBoard

nof_fn		4	Number of front node FPGAs (FN) per UniBoard

nof_un		8	= nof_fn + nof_bn, number of processing node FPGAs per UniBoard

nof_10g	3	Number of 10G links per FPGA node on UniBoard

nof_pn			Number of processing nodes (BN or FN on UniBoard or PN on UniBoard2)

nof_bn_fb	16	= nof_uni*nof_bn, number of subband filterbank BN per SP in the Apertif BF

nof_fn_bf	16	= nof_uni*nof_fn, number of beamformer FN per SP in the Apertif BF

Nworkstation	24	Number of workstations in the GPU cluster of the Arts PL

Wbeamlet		6	Word width in number of bits of a beamlet voltage sample
Wtab 		6	Word width in number of bits of a TAB voltage sample
Wpower 		8	Word width in number of bits of a IAB or TAB power sample

# Introduction

## Purpose

In chapter 2 this document describes several implementation scenarios for Arts. The scenarios should provide sufficient information to plan the development of Arts. Table 12 provides an overview of the scenarios and section 2.8 contains a conclusion that summarizes it all. Chapter 3 provides a system overview of the four science cases (SC) of Arts [1]. The other chapters provide background information and more details regarding the critical requirements and hardware constraints that influence the implementation options for Arts.

## Scope

Main focus is on the implementation scenarios for the Arts FPGA beamformer, however the Arts pipeline GPU cluster is also discussed, because it influences the overall implementation choices, especially regarding the transient data buffer and the Arts BF output format and network. The UniBoard is already in use for the Apertif BF and for the prototype Apertif X. The CoBI, OEB, UniBoard2 and HEM boards are new developments that are currently being tested or designed (section 7).

# Scenarios

## Overview of the Arts data path

Figure 1 shows the Arts data path and its main interfaces. The subsequent sections describe the functions in the data path and their implementation scenarios.

Figure 1: Arts top level data path

## Apertif BF

Table 1 lists the functions that relate to the Apertif BF and are needed for Arts.

| Function | Description | Remark |
| --- | --- | --- |
| 1 MHz subbands | Subband filterbank with 1 MHz subbands instead of 781250 Hz to fit SC1 and SC2. After the change the 1 MHz subbands will be used by Apertif X and by all Arts SC. | Change Request [3] |
| Smooth delay tracking (DT) | Recalculate subbands for duration of the filter impulse response after each DT step. | Section 6.2. Also applies to the Apertif X. |
| Phase tracking (PT) | Provide fringe stopping (FS) per CB by adding phase tracking (PT) in combination with the true sample delay tracking (DT). | Section 6.2. Also needed for the Apertif X. |
| Bypass Tint | Add programmable bypass of the integrating interval transpose Tint that is used for the Apertif X. | Section 6.4 |
| Parallel CB-444 output | Provide parallel CB-444 offload via second 10G port to dedicated Arts FPGA processing boards. To avoid dependencies with the Apertif X it seems better to use dedicated Apertif BF output fibers for Arts than to let Apertif X pass on the CB-444 data. | Section 6.9, 6.10 |

Table 1: Arts functions that relate to the Apertif BF

## Transpose Tsp

Table 2 lists the functions that relate to the transpose Tsp and are needed for Arts.

| Function | Description | Remark |
| --- | --- | --- |
| Input redistribution via UniBoard mesh |  | Section 6.5. Reuse from Apertif X. |
| Input align on UniBoard2 |  | Section 6.5. Reuse existing DP component. |

Table 2: Arts functions that relate to the CB-444 input transpose Tsp

## Arts FPGA beam former (BF)

### Data processing functions

Table 3 lists the functions that relate to the data processing functions that are needed for Arts.

| Function | SC | Remark |
| --- | --- | --- |
| TAB-1 | 1 | Reuse BF unit from Apertif BF |
| TAB-12 | 2 | Reuse BF unit from Apertif BF |
| TAB-444 | 4 | Reuse BF unit from Apertif BF |
| CB-12 pass on | 2 |  |
| IAB-37 component | 3 | New component |
| Channel filterbank | 3,4 | Reuse from Apertif X |
| IQUV Stokes powers component | 3,4 | New component |
| Integrate powers component | 3,4 | New component |
| Test logic with generators and monitors | All | Reuse BG, DB and BSN monitor |

Table 3: Arts functions that relate to data path processing per SC

### Transient data storage

Table 4 lists the functions that relate to the transient data storage functions that are needed for Arts (section 6.3).

| Function | SC | Description | Remark |
| --- | --- | --- | --- |
| Transient data buffer | 3,4 | Streaming write access to DDR3 memory. When a trigger occurs the write stops. A selection of the DDR3 memory contents (ie. one CB, a few TABs) can then be read via the MM interface or streamed out via direct offload to 1GbE. | New component |
| Try 16 GByte DDR3 | 3,4 | Verify that UniBoard can use two large DDR3 modules per FPGA. This is needed for the 4 TByte voltage buffer scenario 1 and 2. | Reuse UniBoard test design for DDR3 |
| Try 16 GByte DDR4 | 3,4 | Verify that UniBoard2 can use two large DDR4 modules per FPGA. This is needed for the 0.5 TByte power buffer scenario 4. | Reuse UniBoard2 test design for DDR4 |

Table 4: Arts functions that relate to transient data storage

Initially assume that SC3 is not needed because SC4 can run commensal on dedicated hardware in case of scenario 3 or 4 (section 2.7). If SC3 is not needed then the 4 TByte voltage buffer is not needed (section 6.3.1). For SC4 still a 0.8 TByte power data buffer is needed (section 6.3.2, 6.6). Scenario 3 can offer up to 4 TByte. Assume the 0.5 TByte that scenario 4 can offer (section 4.4.2) are also sufficient. If the full Stokes power data for SC4 are transported to the GPU cluster then the power data buffer can be implemented on the GPU cluster (section 6.3.4). Initially assume that only the Stokes I power data are transported (section 6.7) and that the power data buffer needs to be implemented on the DDR3 (scenario 1, 3) or DDR4 (scenario 4) of the FPGA beamformer. After a trigger the transient data buffer can be read out via the 1GbE interface, because only a few TAB need to be read out (section 6.18.5).

## Transpose Tband

Table 5 lists the functions that relate to the transpose Tband and are needed for Arts.

| Function | SC | Remark |
| --- | --- | --- |
| Output reorder and offload via 1GbE | 1, 3 | Section 6.15, 6.16 |
| Output reorder and offload via 10GbE | 2, 4 | Section 6.15, 6.16 |
| Output redistribution via UniBoard mesh | 2, 4 | Section 6.15.2 |
| Try Tx only 10GbE links to NIC and via switch | 2, 4 | Section 6.17 |
| Pass on meta data including flagging | All |  |

Table 5: Arts functions that relate to the Arts BF output transpose Tband

Assume that SC1 will use output via 1GbE (section 6.18.16.18.4). If SC3 is needed then SC3 can also use the 1GbE network for output (section 6.18.4). For SC2 and SC4 output via 10GbE is needed. Initially assume that for SC4 only the Stokes I data are transported (section 6.7, 6.8). This then implies that only one 10GbE output per Uniboard (scenario 1, 3) or per PN (scenario 4) is needed (section 6.18.2, 6.18.3).

The transpose Tband can be done by a 10GbE switch, by one UniBoard2 with Arria10 or by the infiniband switch in the GPU cluster, see section 6.18.2. Assume that a UniBoard2 will be used, see Table 6. The cost of this are then 15 Euro.

## Arts pipeline GPU cluster

The main architecture choices for the GPU cluster (section 5) concern:

- Arts BF output data format specification concerning need for data reordering (section 6.16), need for packing into bytes (section 6.8.1)

- The number of workstations in the Arts PL GPU cluster and the number of 10GbE ports

- Streaming transport full Stokes data or only Stokes I.

- When only Stokes I data is transported then the transient data buffer for SC4 needs to be on the Arts BF

- The CB-444 voltage data transient data buffer for SC3 can only be on the Arts BF

- How much DDR memory does the Arts PL need for processing and what is the access rate per CPU, GPU.

- Having an Infiniband switch in the GPU cluster may be valuable anyway even if it is not used for the transpose Tband.

The cost for the GPU cluster are estimated at 20k for SC1, 20k for SC2 and 240k for SC3 and SC4, so about 10k per workstation assuming Nworkstation = 24.

## Hardware scenarios

### Preliminary design choices

Table 6 lists the preliminary design choices that have been made for Arts.

| Design choice | SC | Description |
| --- | --- | --- |
| Use dedicated CB-444 output from Apertif BF for Arts BF. | All | Advantage is that dedicated Arts BF hardware can operate independently from the Apertif X hardware, see section 6.9 |
| Use 16*1GbE10GbE switch between Arts BF and Arts PL for SC1 | 1 | Section 6.18.1 |
| Implement transient data buffer in Arts BF. | 3,4 | Advantage is that only the stokes I data need to be streaming transported from the Arts BF ot the Arts PL, see section 6.3, 6.7, 6.18.5. |
| Use UniBoard2-Arria10 as a 10GbE switch between Arts BF and Arts PL. | 2,4 | Advantage is that this UniBoard2 can map the unidirectional inputs to full duplex 10GbE outputs so no need for HEM or to investigate Tx only capabilities of switches, it can do additional reording within data packets or between data packets to map 16 inputs ports to 24 output ports, and it can implement the transpose Tband., see section 6.16, 6.17, 6.18.2. The Infiniband switch in the GPU cluster is then not needed (at least not for the transpose Tband). |

Table 6: Preliminary design choices

### Scenarios for the Arts FPGA beamformer

The following tables list several scenarios for the Arts FPGA beamformer.

| Scenario | Reuse 16 UniBoards |
| --- | --- |
| Description | Reuse the 16 UniBoards of Apertif X for all four Arts SC (section 6.11): - SC1 and SC4 fit on the 16 UniBoards, because they are not running simultaneously with Apertif X. - The SC2 with local interferometer data for only the central CB can fit on the 16 UniBoards, see section 6.1 - Commensal SC3 will have to run together with the Apertif X on the same FPGAs. |
| Issues | - SC3 cannot fit together with Apertif X on the UniBoard FPGAs, see section 4.4.5 - Combining Apertif X with Arts SC3 in one FPGA will make the firmware design and maintenance more complicated. |

Table 7: Arts BF scenario 1

| Scenario | Use dedicated 4 UniBoard2s with Arria10 FPGAs (section 4.2) |
| --- | --- |
| Description | Reuse 16 UniBoards for SC1 and SC4 and use dedicated 4 UniBoard2s with Arria10 for SC2 and SC3: - SC1 can remain on the 16 UniBoards, but it could also be ported to the 4 UniBoard2. - SC4 needs to remain on the 16 UniBoards, because regarding processing SC4 does not fit on the 4 UniBoard2 with Arria10 (see section 4.5). |
| Issues | - UniBoard2 only has maximum 1 TByte for transient data storage, whereas SC3 requires 4 TByte for 10 s of CB-444 voltage data. |

Table 8: Arts BF scenario 2

| Scenario | Use dedicated 8 UniBoard2s with Arria10 FPGAs (section 4.2) |
| --- | --- |
| Description | Compared to scenario 2 the SC4 can now run on the UniBoard2. - With SC4 on a dedicated system the SC3 is no longer needed and can be omitted. |
| Issues | - The 4 extra UniBoard2s increase the cost, complexity and the power consumption. |

Table 9: Arts BF scenario 2b

| Scenario | Use dedicated 16 UniBoards (section 4.1, 6.12) |
| --- | --- |
| Description | All Arts SC can run on dedicated 16 UniBoards. - With SC4 on a dedicated system the SC3 is no longer needed and can be omitted. |
| Issues | - Extra cost of about 312 kEuro for UniBoard + OEB hardware, see Table 22. |

Table 10: Arts BF scenario 3

| Scenario | Use dedicated 4 UniBoard2s with Stratix10 FPGAs (section 4.2) |
| --- | --- |
| Description | All Arts SC can run on dedicated 4 UniBoard2s with Stratix10. - SC1 may remain on 16 UniBoard that are reused when the Apertif X is not used, but SC1 could also be ported to the 4 UniBoard2. - Commensal SC2 and SC4 can fit on 4 dedicated UniBoard2 with Stratix10. - With SC4 on a dedicated system the SC3 is no longer needed and can be omitted. |
| Issues | - Extra cost of about ??? kEuro for UniBoard2 (Stratix10) + HEM hardware, see Table 23. - Stratix10 production samples are expected in Q3 2016, section 7.2. However firmware development can already start earlier because Stratix10 can be simulated. |

Table 11: Arts BF scenario 4

### Evaluation of the scenarios

Table 12 summarizes the science capabilities of the scenarios including the hardware cost and the limitations.

| Scenario | Hardware Arts BF | Cost for Arts BF | Science | Limitation |
| --- | --- | --- | --- | --- |
| 1 | Use the 16 UniBoards of Apertif X | 10k (= 1 TByte for SC4, section 4.4.2) | - All Arts SC can run on 16 UniBoards. | - Maximum 4 TByte buffer - No commensal modes together with Apertif X due to lack of FPGA resources (section 4.4.5). |
| 2 | Dedicated 4 Uniboard2-Arria10 | 132k (= 116k from Table 23 + 16k for 0.5 TByte for SC4, section 4.4.2) | - Commensal SC4, but with limited FoV due to less than 12 TABs. | - Maximum 1 TByte buffer - No commensal SC3 due to only 1 TByte (instead of 4 TByte). - For SC4 maximum 7 TABs per CB instead of 12 (section 4.4.6). |
| 2b | Dedicated 8 Uniboard2-Arria10 | 228k (= 212k from Table 24 + 16k for 0.5 TByte for SC4, section 4.4.2) | - Commensal SC4 for maximum 14 TABs - No need for SC3. | - Maximum 2 TByte buffer |
| 3 | Dedicated 16 UniBoards | 322k (= 312k from Table 22 + 10k for 1 TByte, section 4.4.2) | - Commensal SC4 for maximum 20 TABs - No need for SC3, but can fit SC3. | - Maximum 4 TByte buffer |
| 4 | Dedicated 4 Uniboard2-Stratix10 | 252k (= 236k from Table 23 + 16k for 0.5 TByte for SC4, section 4.4.2) | - Commensal SC4 for maximum 58 TABs - No need for SC3. | - Maximum 1 TByte buffer |

Table 12: Overview of the Arts BF scenarios

For all scenarios applies that using less boards means less bandwidth. For example if  instead of 16 Uniboard only 1 UniBoard is used than CBBW/16 = 19 MHz and similar if instead of 4 UniBoard2 only 1 UniBoard2 is used then CBBW/4 = 75 MHz. The firmware validation on hardware can be done with 1 board, because the boards operate independently and do the same. Firmware can already be developed before hardware is available, because it can be verified in simulation.

From a technical point of view scenario 4 is preferred because it easily fits SC4 (section 4.5), but the UniBoard2 with Stratix10 FPGAs is not available yet (section 7.2). A prototype UniBoard2 with Arria10 FPGAs is available now. UniBoard production boards are available now. Therefore first for SC1 and then for SC4 the Arts development can start with scenario 1. Porting SC4 from StratixIV to Arria10 or Stratix10 is relatively small, because the application components are designed in a technology independent way. However SC4 on Uniboard requires some extra development effort to output the data via 10GbE (section 6.5, 6.15.2) that is not needed for UniBoard2.

For SC2 scenario 2 or 4 is preferred, because then the Apertif X can be used for the local interferometer data of the central CB. Whereas using scenario 1 requires adding a central CB correlator to the Arts SC firmware.

Commensal SC3 will not fit together with Apertif X on the 16 UniBoards (section 4.4.5). Therefore dedicated hardware will be needed for commensal Arts. Scenario 2 using 4 UniBoard2 with Arria10 cannot fit the 4 TByte memory required for the transient voltage data buffer for SC3 (section 4.4.2) and it does not have sufficient processing resources to fit SC4 (section 4.4.6). Using 8 UniBoard2 can fit SC4 but comes at a cost regarding hardware, power consumption and development.

The advantage scenario 3 and 4 is that they can fit SC4 and then SC3 and the 4 TByte transient voltage data buffer are no longer needed. The advantage of scenario 4 is that the UniBoard2 hardware is cheaper than the UniBoard hardware for scenario 3 and that scenario 4 can provide many more TABs (section 4.4.6). The energy consumption of scenario 4 using UniBoard2 is also less and may save > 28 kEuro over 5 years (section 4.4.8). With UniBoard2 the firmware architecture does become simpler (section 6.5.3, 6.15.2, 6.17.2), because UniBoard2 has more IO than Uniboard.

### Arts SC1 hardware infrastructure in scenario 1

Figure 2 shows Arts SC1 implemented according to scenario 1 reusing the 16 UniBoards of the Apertif X. Between the Arts BF and Arts PL there needs to be an Ethernet switch to lead the data to a single 10GbE port. This Ethernet switch effectively performs the transpose Tband to bring together the entire CBBW = 300 MHz at a single GPU node (section 6.18.1). For  SC1 one GPU workstation suffices to do the pipeline processing. Later on the GPU workstation may remain standalone or it can become part of the GPU cluster.

Figure 2: Arts SC1 reusing 16 UniBoard from Apertif X and 1GbE switch for Arts BF output to PL

### Arts SC4 hardware infrastructure in scenario 1

Figure 3 shows Arts SC4 implemented according to scenario 1 reusing the 16 UniBoards of the Apertif X. This scheme assumes that only the TAB-444 Stokes I power data is streamed from the Arts BF to the Arts PL. This implies that only one 10GbE port per UniBoard is needed (section 6.7, 6.18.2) and that the UniBoards will need to buffer the TAB-444 Stokes I power data in DDR3 memory (section 6.3, 6.7). Using e.g. 4 GByte DDR3 modules 16 Uniboards can store 1 TByte. The number of 10GbE output links from the Arts BF to the Arts PL is Nband = 16. The GPU cluster has e.g. 24 workstations and a UniBoard2 with Arria10 is used as switch to connect the Arts BF to the Arts PL.

Figure 3: Arts SC4 reusing 16 UniBoards from Apertif X and Stokes I output to PL

### Arts SC2 and SC4 hardware infrastructure in scenario 4

Figure 4 shows Arts SC2 and SC4 implemented according to scenario 4 using 4 dedicated UniBoard2s. By using dedicated hardware for Arts and by using the second output port of the Apertif BF the Arts hardware is independent from the Apertif X hardware. In Figure 4 SC1 has been ported to UniBoard2, however it could also remain on the Apertif X UniBoards to avoid this effort. The single GPU workstation for SC1 may also be part of the GPU cluster. The 10GbE link for SC1 may be connected via the UniBoard2 with Arria10 that is used as a switch to connect the Arts BF to the Arts PL.

Figure 4: Arts SC2 and SC4 using dedicated UniBoard2 with Stratix10 and independent from Apertif X

## Conclusion

Scenario 1 is the cheapest regarding Arts BF hardware cost, but it implies that Arts cannot run commensal with Apertif X. Running Arts commensal requires adding dedicated hardware for the Arts BF as in the other scenarios.

Scenario 2 can fit SC2. Scenario 2 cannot provide sufficient TABs for SC4, but it can provide a development path towards scenario 4 using the one UniBoard2 with Arria10 that we will have in 2016 for initial tests (providing CBBW/4 = 75 MHz and maximum 7 TABs / CB).

Scenario 2b fits SC4 but it is not much cheaper than scenario 4 and provides far less TABs than scenario 4.

Scenario 3 fits SC4 (and SC3) but it is more expensive than scenario 4 and provides far less TABs than scenario 4.

Scenario 4 easily fits SC4 because it can provide about a factor 4 more TABs than required. Being able to run SC4 commensally makes that SC3 is not needed.

The estimated cost for the Arts BF using scenario 4 is 252k (Table 12).

The estimated cost for the Arts PL is 280k (section 2.6).

The network between Arts BF and Arts PL will cost about 20k (1/10GbE switch for SC1 and a Uniboard2 with Arria10 for SC2 and SC4, see Figure 4).

# System overview

## Apertif

Arts [1] implements the tied array and VLBI functionality of Apertif. Figure 5 shows Arts within Apertif. Both the Apertif correlator (X) and Arts use the beam data from the Apertif beamformer (BF).

Figure 5: Top level overview of Apertif with Arts included

## Arts

Within Arts the processing consists of an FPGA beamformer and a GPU pipeline, as shown in Figure 6. Arts

has four science cases (SC) and for all four SC the FPGA beamformer will be implemented on Uniboards and the pipeline processing will be implemented on GPUs. The mapping on FPGAs or GPUs is discussed generally in [5].

Figure 6: Arts FPGA beamformer and GPU pipeline

## Interfaces

### Apertif BF output

The Apertif BF processes each dish and each polarization independently. The beamformed output for one polarization of one dish is called a signal path (SP). Figure 7 shows the Apertif BF subrack that can beamform the NCB=37 compound beams (CB) and 1 CB consists of Npol=2 SP. The subrack consists of 4 Uniboards and Nband=nof_fn_bf=16 outputs. Note that each FN on UniBoard has nof_10G=3 ports available, but for the Apertif BF output only one 10G port is used as shown in Figure 7.

Figure 7: One Apertif BF subrack per signal path with nof_uni=4 UniBoards and Nband=16 FN

In total the Apertif BF consists of Nsp=Npol*Ndish = 2*12 =24 subracks, as shown in Figure 5 and Figure 8.

Figure 8 shows that the Apertif BF output is carried via Nlink = Nsp*Nband = 24 * 16 = 384 10G fibers to the Nband=16 UniBoards that implement the Apertif X.

Figure 8: Apertif BF transpose Tsp interconnect to Apertif X and Arts

Functionally the Apertif BF output load LBF_CB444 consists of NCB*Ndish = 37*12 = 444 compound beams that each have a bandwidth of CBBW=300 MHz. Hence each UniBoard in the Apertif X receives 1/Nband = 1/16 part of the CBBW. Table 13 lists the load definitions for the Apertif BF output with Wbeamlet = 6 bit. The total load LBF_CB444 = 3.2 Tbps and the number of 10G links Nlink=384 are so large that the Apertif X and Arts BF need to be implemented on FPGAs rather than on GPUs.

| Load | Equation | Value | Description |
| --- | --- | --- | --- |
| LBF_SP1 | = CBBW * Ncomplex * Wbeamlet | 3.6 Gbps | Load for 1 SP |
| LBF_SP1_band | = LBF_SP1 / Nband | 225 Mbps | Load for 1 SP per band (= per BF node) |
| LBF_SP37_band | = NCB * LBF_SP1_band | 8.325 Gbps | Load for NCB = 37 SP per band (= per BF node) |
| LBF_CB1 | = Npol * LBF_SP1 | 7.2 Gbps | Load for 1 CB (= 2 SP, Npol = 2) |
| LBF_CB12 | = Ndish * LBF_CB1 = NPN * LBF_SP1_band | 86.4 Gbps | Total load from Ndish = 12 dishes, for 1 CB |
| LBF_CB444 | = NCB * LBF_CB12 = NPN * LBF_SP37_band | 3.2 Tbps | Total load from Ndish = 12 dishes, for NCB= 37 CB |
| LBF_link1 | = LBF_SP1_band | 225 Mbps | Link load for 1 SP per band (= per BF node) |
| LBF_link37 | = LBF_SP37_band | 8.325 Gbps | Link load for NCB= 37 SP per band (= per BF node) |

Table 13: Load definitions for Apertif BF output interface with Wbeamlet = 6 bit

### Arts FPGA BF output

The input CB data width from the Apertif BF is Wbeamlet=6 bits. The Arts BF output can be:

- CB voltage data for SC2

- TAB voltage data for SC1 or SC2

- TAB or IAB power data for SC3 and SC4

For CB voltage data the Arts BFoutput data width remains Wbeamlet=6 bit. For TAB voltage beam data the Arts BF output data width is Wtab=4 bit and at least 5 bit to include strong sources like Cas A [2]. Choose to output Wtab=6  bit to have some more dynamic range. For TAB or IAB power beam data the Arts BF output data is Wpower=8 bit in a semi-floating point format [2]. Table 14 derives the Arts BF output loads given these data widths. The power beam data can be full Stokes I,Q,U,V or only I. The power beam data is integrated over Nint= 10 channel samples.

| Load | SC | Equation | Value | Load for |
| --- | --- | --- | --- | --- |
| LTAB1 | 1 | = CBBW*Npo *Ncomplex* Wtab | 7.2 Gbps | Voltage TAB-1 |
| LTAB1_band | 1 | = LTAB1 / Nband | 450 Mbps | Voltage TAB-1 per PN0 |
| LTAB12 | 2 | = NVLBI * LTAB1 | 86.4 Gbps | Voltage TAB-12 |
| LTAB12_band | 2 | = LTAB12 / Nband | 5.4 Gbps | Voltage TAB-12 per PN0 |
| LCB12 | 2 | = LBF_CB12 | 86.4 Gbps | Voltage CB-12 |
| LCB12_band | 2 | = LCB12 / Nband | 5.4 Gbps | Voltage CB-12 per PN0 |
| LIAB1_stokes | 3 | = CBBW * NStokes * Wpower | 9.6 Gbps | IAB-1 without integration |
| LIAB1_stokes_int | 3 | = LIAB1_stokes / Nint | 0.96 Gbps | IAB-1 with Nint=10 |
| LIAB37_stokes | 3 | = NCB * LIAB1_stokes | 355.2 Gbps | IAB-37 without integration |
| LIAB37_stokes_int | 3 | = NCB * LIAB1_stokes_int | 35.52 Gbps | IAB-37 with Nint=10 |
| LIAB37_stokes_I_int | 3 | = LIAB37_stokes_int / NStokes | 8.88 Gbps | IAB-37-I with Nint=10 |
| LIAB37_stokes_int_band | 3 | = LIAB37_stokes_int / Nband | 2.22 Gbps | IAB-37 with Nint=10 per UNB |
| LIAB37_stokes_I_int_band | 3 | = LIAB37_stokes_I_int / Nband | 0.555 Gbps | IAB-37-I with Nint=10 per UNB |
| LTAB1_stokes_int | 4 | = LIAB1_stokes_int | 0.96 Gbps | Power TAB-1 with Nint=10 |
| LTAB444 | 4 | = NCB * Ngr * LTAB1 | 3.1968 Tbps | Voltage TAB-444 |
| LTAB444_stokes | 4 | = LTAB444* (Wtab/Wpower) | 4.2624 Tbps | Power TAB-444 without integration |
| LTAB444_stokes_int | 4 | = NCB * Ngr * LTAB1_stokes_int | 426.24 Gbps | TAB-444 with Nint=10 |
| LTAB444_stokes_I_int | 4 | = LTAB444_stokes_int / NStokes | 106.56 Gbps | TAB-444-I with Nint=10 |
| LTAB444_stokes_int_band | 4 | = LTAB444_stokes_int / Nband | 26.64 Gbps | TAB-444 with Nint=10 per UNB |
| LTAB444_stokes_I_int_band | 4 | = LTAB444_stokes_I_int / Nband | 6.66 Gbps | TAB-444-I with Nint=10 per UNB |

Table 14: Load definitions for Arts BF output interface (with Wbeamlet = 6 bit, Wtab = 6 bit, Wpower = 8 bit)

# Uniboard hardware

## UniBoard and OEB

The front nodes on UniBoard have optical 10G interfaces and the UniBoard uses an OEB to also provide the back nodes with optical 10G interfaces. In total the UniBoard + OEB then has nof_un * nof_10G = 8*3 = 24 optical 10G interfaces, so just enough to connect to Nsp=24 links from the Apertif BF as shown in Figure 8. One UniBoard can process 1/Nband=1/16 part of the CBBW=300 MHz band. To process the whole CBBW there are Nband = 16 UniBoards as shown in Figure 9.

Figure 9: One UniBoard to process 1/Nband =1/16 part of the CBBW =300 MHz for Nsp=24 signal paths

Figure 9 also shows the full duplex mesh interconnect between the FN and BN on UniBoard. In fact UniBoard and OEB are connected in groups of 8 boards via a backplane called CoBI. Figure 9 does not show the CoBI board.

## UniBoard2 and HEM

The UniBoard2 has nof_pn=4 FPGA and each PN has 24 optical 10G links. Therefore from an IO point of view UniBoard2 is 4 times more powerful than a UniBoard and hence Muni2 = 4 UniBoard2 are sufficient to receive the output from the Apertif BF as shown in Figure 10.

Figure 10: One UniBoard2 to process 4/Nband =1/4 part of the CBBW =300 MHz for Nsp=24 signal paths

The Hybrid Memory Cube (HMC) Extension Board (HEM) for UniBoard2 in Figure 10 is similar to OEB for UniBoard, because it provides 8 extra optical 10G interfaces to the board. In addition HEM also provides extra external memory. This HMC memory is accessed via transceiver links. UniBoard2 itself has two DDR4 memory modules per PN. With HEM the amount of external memory per PN about doubles.

Without HEM the UniBoard2 needs to use unidirectional Tx only links to output 10G data to the GPU PL. With HEM the UniBoard2 has 8 extra full duplex 10G (or 2 40G) links available per PN for output to the GPU PL.

The UniBoard2 also has 12 full duplex links so 120 Gbps, between the PN. The HEM board also closes this full duplex 120 Gbps ring between PN3 and PN0. These inter PN links can be used to exchange (reorder) input data or output data between PN.

## Control interface and data offload via 1GbE

UniBoard in Figure 9 and UniBoard2 in Figure 10 have an onboard 1GbE switch for access via 1GbE. The 1GbE network is primarily used for control but it can also be used to offload streaming data from the PN (or to the PN). Table 15 summarizes the IO capabilities of the 1GbE interface of UniBoard and UniBoard2.

| Board | Number of 1GbE links per PN | Number of external 1GbE ports board | Maximum IO rate per board |
| --- | --- | --- | --- |
| Uniboard | 1 | 4 | 4 Gbps |
| Uniboard2 | 2 | 6 | 6 Gbps |

Table 15: 1GbE IO interface for UniBoard and UniBoard2

## Comparison

### Optical links

Table 16 summarizes section 4.1 and 4.2 regarding the 10G optical IO capabilities of UniBoard = EOB and UniBoard2 + HEM.

|  | UniBoard + OEB | UniBoard2 + HEM |
| --- | --- | --- |
| Number of optical 10G links | 4*3 + 4*3 = 24 | 4*(24 + 8) = 96 + 32 |

Table 16: Number of optical 10G links per board

### External memory

Table 17 shows that 4 UniBoard2 have a factor 8 less external memory than 16 Uniboards (assuming maximum 16 GByte DDR4 modules). The total access rate is only about a factor 3 less, because DDR4 is faster than DDR3.

| External memory type: | DDR3 on UniBoard | DDR4 on UniBoard2 |
| --- | --- | --- |
| Size and access rate: |  |  |
| Memory module size | 16 GByte (max. Micron) | 16 GByte (max. Micron) |
| Memory module access rate | 800 MTps (max. 1066 MTps) | 2400 MTps (max. 2666 MTps) |
| Total number of memory slots 16 UniBoard | 16 * 8 * 2 = 256 | - |
| Total number of memory slots 4 UniBoard2 | - | 4 * 4 * 2 = 32 |
| Total memory size | 256 * 16G = 4 TByte | 32 * 16G = 0.5 TByte 32 * 32G = 1 TByte |
| Total access rate | 256 * 800M * 64b = 13.1 Tbps | 32 * 2400M * 64b = 4.9 Tbps |

Table 17: External memory size and access rate

External memory costs about 37 Euro per 4 GByte DDR3 module and 281 Euro per 16 GByte DDR3 module (July 2015). Assume the cost for DDR3 memory on a GPU workstation are a little less, because we do not have to use the largest modules then and for UniBoard we typically need to use modules from Micron. Assume that 16 GByte DDR3 and 16 GByte DDR4 will both cost about 250 Euro per module, then:

- GPU cluster: 4 TByte = 50 kEuro

- UniBoard : 4 TByte = 256 * 250 Euro = 64 kEuro

- UniBoard:  1 TByte = 256 * 40 Euro = 10 kEuro

- UniBoard2: 0.5 TByte = 32 * 250 Euro = 16 kEuro

### Processing resources

Table 18 shows the logic, multiplier and RAM resources that are available for the different FPGAs that are used on UniBoard and Uniboard2. UniBoard uses 8 Stratix IV FPGAs. UniBoard2 uses 4 Arria10 FPGAs. End 2016 UniBoard2 can be fitted with 4 Stratix10 FPGAs that are pin compatible (1932 pins, 45 x 45 mm).

| FPGA type: | Stratix IV - EP4SGX230 | Arria10 - 10AX115 |  | Stratix10 – 10SG280 |  |
| --- | --- | --- | --- | --- | --- |
| Resource: |  |  | x |  | x |
| ALM 1) | 91200 | 427200 | 4.6 | 933120 | 10.2 |
| Registers (FF) | 182400 | 1708800 | 9.3 | 3732480 | 20.4 |
| Logic Elements (LE) | 228k | 1150k | 5.0 | 2753k | 12.0 |
| M9K | 1235 | - |  | - |  |
| M144K | 22 | - |  | - |  |
| M20K 2) | - | 2713 | 2.2 | 11721 | 9.5 |
| Mbit | 14.283 | 53 | 3.7 | 229 | 16.0 |
| 18x18 multipliers | 1288 | - |  | - |  |
| 18x19 multipliers 3) | - | 3036 | 2.3 | 11520 | 8.9 |
| Clock max 4) | 200 MHz | 200 MHz | 1 | 400 MHz | 2 |

Table 18: FPGA processing resources

Notes for Table 18:

- One ALM (Adaptive Logic Module) on Stratix IV contains 2 flipflops (FF) while on Arria10 and Stratix10 it contraints 4 FF. The ALM can be represented by a certain number of logic elements (LE). The LE figure can be used to compared the general purpose logic resources between Altera FPGAs.

- Compare the number of M20K block RAM with the number of M9K block RAM. The unit block RAM size on Stratix IV is M9K = 9 kBit while on Arria10 and Stratix20 it is M20K = 20 kBit. Often at least one block RAM is needed therefore a worst case comparison compares the number of block RAMs and a best case comparison compares the amount of Mbits.

- Compare the number of 18x19 multipliers with the number of 18x18 multipliers.

- The maximum core clock rate Fmax for Stratix IV and Arria 10 is about 550 MHz but in practise this means that for applications only 200 MHz is easily achievable and more only with extra design effort. For Stratix10 Fmax is about 1 GHz, hence assume a practical clock rate is 400 MHz, so a factor 2 more.

From Table 18 it follows that the Arria10 FPGA is about a factor 2 - 4 more powerful than the Stratix IV FPGA whereas the Stratix10 FPGA is about a factor 16 - 32 more powerful than the Stratix IV.

### Processing load estimates for Apertif X

Table 19 shows an estimate of the number of FPGA resources that will be needed for the Apertif X. The estimate is based on a synthesis result of Apertif X with channel filterbank (using 9b FIR coefficients) and the correlator. For the input reorder section via the mesh (Tsp) the synthesis results from another design were used. The sum of these results is an estimate for the final Apertif X resource usage. The notes for Table 18 also apply to Table 19. The fact that the number of ALMs > 100 % implies that the final synthesis will have to pack the logic more efficiently into the ALMs, which is feasible, because the number of FF is still < 100 %. Nevertheless the Apertif X FPGA is almost full.

| FPGA resources | Stratix IV - EP4SGX230 | Apertif X | Usage |
| --- | --- | --- | --- |
| Logic (ALM) | 91200 | 108k | 120 % |
| Registers (FF) | 182400 | 175k | 96 % |
| Block RAM (M9K) | 1235 | 960 | 78 % |
| Multipliers (DSP 18x18) | 1288 | 1084 | 84% |

Table 19: Estimated FPGA resources for Apertif X

### Processing load estimates for Arts SC3

Table 20 shows an estimate of the extra number of FPGA resources that will be needed for the Arts SC3. The estimate is based on the synthesis results of:

- a BF unit without weights and without BST (to represent the IAB-37)

- the node_unb1_ddr3_reorder function (to represent the transient buffer access control)

- one extra io_ddr component to represent that both DDR3 memory slots are needed

- a mesh reorder function (to represent the output of the Stokes I data via the mesh)

- a tr_10GbE component (to represent the output of the Stokes I data via 10GbE)

| FPGA resources | Stratix IV - EP4SGX230 | Arts SC3 | Usage |
| --- | --- | --- | --- |
| Logic (ALM) | 91200 | 22k | 24 % |
| Registers (FF) | 182400 | 28k | 15 % |
| Block RAM (M9K) | 1235 | 106 | 8 % |
| Multipliers (DSP 18x18) | 1288 | 96 | 7 % |

Table 20: Estimated FPGA resources that are needed extra for commensal Arts SC3

Adding the numbers in Table 19 and Table 20 shows that the Stratix IV FPGA cannot fit Apertif X and SC3 together on the same FPGA. The bottleneck are the logic resources (the number of ALM and FF).

### Processing load estimate per TAB

The channel filterbank requires 288 18bx18b multipliers, assuming 8 FIR filter taps and 9 bit FIR filter coefficients, and Nchan=4 channels. The number of multipliers that is needed to beamformer 1 TAB per PN is Nsp * Pcmult * fdata/fclk = 24 * 4 * 0.5 = 48 18bx18b multipliers. Here Nsp = 24, Pcmult = 4 for one complex multiply and fdata/fclk=100MHz / 200MHz=0.5, so 1 TAB requires 48 multipliers. The maximum available number of TABs KTAB given in Table 21.

| Platform | Number of multipliers per PN | Relative number of PN | Relative fclk | KTAB | KTAB / Ngr |
| --- | --- | --- | --- | --- | --- |
| 16 UniBoard with StratixIV | 1288 | 1 | 1 | (1288-288)*1*1/48 = 20 | 1.66 |
| 4 UniBoard2 with Arria10 | 3036 | 1/8 | 1 | (3036-288)* 0.125*1/48 = 7 | 0.58 |
| 8 UniBoard2 with Arria10 | 3036 | 2/8 | 1 | (3036-288)* 0.25*1/48 = 14 | 1.16 |
| 4 UniBoard2 with Stratix10 | 11520 | 1/8 | 2 | (11520-288)*0.125*2/48 = 58 | 4.83 |

Table 21: Maximum number of TABs

The required number of TABs for SC4 is Ngr=12. Clearly 4 UniBoard2 with Stratix10 can implement SC4. The 16 UniBoard can also implement SC4. Four UniBoard2 with Arria10 running at fclk = 200 MHz can only achieve 7 TABs instead of 12.

### Hardware cost

Table 22 lists the costs of using dedicated hardware using 16 UniBoards in a similar configuration as for the Apertif X.

| Component | Cost per component [Euro] | Number of components | Total cost [Euro] |
| --- | --- | --- | --- |
| UniBoard | 12.5k | 16 | 200k |
| OEB | 2.5k | 16 | 40k |
| PAC | 4k | 2 | 8k |
| Mini-PAC | 1k | 2 | 2k |
| CoBI | 3k | 2 | 6k |
| subrack | 2k | 2 | 4k |
| 48V power supply | 6k | 2 | 12k |
| SFP fiber connector | 50 | 2*384 2) | 40k |
| Total: |  |  | 312k |

Table 22: Cost for Arts BF using dedicated hardware using 16 UniBoards

Table 23 lists the cost when 4 UniBoard2 with Arria10 FPGAs are used.

| Component | Cost per component [Euro] | Number of components | Total cost [Euro] |
| --- | --- | --- | --- |
| UniBoard2 | 15k (using Arria10) | 4 | 60k |
| UniBoard2 | 45k (using Stratix10) | 4 | 180k |
| HEM | 5k | 4 | 20k |
| box | 1k | 4 | 4k |
| 48V power supply | 6k | 2 | 12k |
| SFP fiber connector | 50 | 384 1) | 20k |
| QSFP fiber connector | 200 2) | 96 1) | 20k |
| Total: |  |  | 116k with Arria10 3) |
|  |  |  | 236k with Stratix10 3) |

Table 23: Cost for Arts BF using dedicated hardware using 4 UniBoard2s with Arria10 or Stratix10

Notes for Table 23:

- The number of SFP connectors is 2 * 384 to allow the scheme of Figure 13.

- Assume that 1 QSPF connector for Uniboard2 costs as much as 4 SFP connectors.

- Cost without HEM.

Table 24 lists the cost when 8 UniBoard2 with Arria10 FPGAs are used. The extra 4 UniBoard2s provide extra processing and are connected in a daisy chain to the first 4 UniBoard2s (similar as in Figure 14).

| Component | Cost per component [Euro] | Number of components | Total cost [Euro] |
| --- | --- | --- | --- |
| Complete system of 4 UniBoard2 with Arria10 | 116k | 1 | 116k (from Table 23) |
| UniBoard2 | 15k (using Arria10) | 4 | 60k |
| box | 1k | 4 | 4k |
| 48V power supply | 6k | 2 | 12k |
| QSFP fiber connector | 200 | 96 | 20k |
| Total: |  |  | 212k |

Table 24: Cost for Arts BF using dedicated hardware using 8 UniBoard2s with Arria10 (and no HEM)

### Power consumption and cost

Exact power consumption estimates are difficult to give because they depend on the processing and the number of IO links. However assume that UniBoard takes 250 W and that UniBoard2 with Arria10 or Stratix10 will take 300 W, then 16 Uniboards take 4 kW and 4 UniBoard2 take 1200 W. The difference is 2800 W. Assume Arts is powered continuously for 5 years. A year has ~10k hours and assume that 1 kWh costs 0.20 Euro, then the energy cost savings by using UniBoard2 are 2800 * 10k * 0.00020 * 5 = 28 kEuro. The savings will even be more because the energy for cooling will also be less.

The difference in between using 4 or 8 UniBoard2 with Arria10 is 1200 W. Hence the extra power consumption cost of using 4 more UniBoard2s are 1200 * 10k * 0.00020 * 5 = 12 kEuro.

## Conclusion

A system with 16 UniBoards has 16 * 8 =128 FPGAs. A system with 4 UniBoard2 has 4 * 4 = 16 FPGAs. Hence for the same processing a UniBoard2 FPGA needs to be 8 times as powerful regarding IO (Table 16), external memory (Table 17) and logic, DSP multipliers, on chip RAM (Table 18). SC4 requires nearly all FPGA resources of the 16 UniBoards. SC1, SC2 and SC3 need less FPGA resources, because they make less beams (TAB-1, TAB-12, IAB-37) and because on UniBoard2 they do not need the logic to redistribute the CB data for the Tsp transpose that UniBoard does need. Conclusion:

- 4 UniBoard2 with Stratix10 FPGAs can easily replace 16 UniBoards also for SC4 (Table 21), with 1 TByte they also have sufficient external memory.

- 4 UniBoard2 with Arria10 FPGAs can replace 16 UniBoard from an IO point of view and for SC2 and SC3 this is also enough from a processing point of view. However from a processing point of view 8 UniBoard2 with Arria10 FPGAs are needed for SC4 (assuming fclk=200 MHz).

# GPU cluster workstations

## Input data

From Table 14 is follows that the integrated full Stokes data rate for SC4 that comes from the Arts BF is LTAB444_stokes_int  = 444 (TABs) * 300 MHz  (TAB bandwidth) * 4 (Stokes parameters) * 8 bit (power data)  / 10 (number of samples per integration period to have T=50 us per channel) = 426.24 Gbps = 53.3 GByte/s.

If only the Stokes I data is transported then LTAB444_stokes_I_int = 106.56 Gbps.

## Transient data buffer

If the full Stokes data is transported then the transient data buffer can be implemented in the Arts BF or in the Arts PL. If only the Stokes I data is transported, then the transient data buffer needs to be implemented on the Arts BF.

## GPU cluster

The Arts pipeline (PL) processing will be done on a cluster of workstations with GPUs. Each workstation (machine) in the GPU cluster has a PCI express bus that connects:

- CPUs with DDR3 memory

- GPUs with DDR3 memory

- 1*1GbE interface for control

- 2*10GbE interface for data

- 40G Infiniband interface for IO between the workstations

Assume a cluster with Nworkstation = 24 GPU workstations and that the workstations are interconnected via a 24 port 40G Infiniband switch.

### Input data rate per work station

With 24 workstations and a total data rate of 426.24 Gbps TAB-444 full Stokes data each workstation gets 17.76 Gbps = 2.22 GByte/s, so two 10GbE interfaces should be enough. If only the TAB-444 Stokes I data needs to be transported, then the data rate is a factor Nstokes=4 less, so 4.44 Gbps = 0.555 GByte/s and then one 10 GbE interface per workstation suffices.

### Transient data buffer storage

The TAB-444 full Stokes data needs to be buffered (SR-0.35). If this is not done on the UniBoards in the Arts BF then it needs to be done in the Arts PL. For 15 s this requires in total 15 s * 53.3 GByte/s = 0.8 TByte, so about 40 Gbyte/ workstation and about 53.3 G / 24 = 2.22 GByte/s (write) access rate per workstation.

### Tband data transpose to bring together the 300 MHz band

The data arrives from 16 UniBoards and each UniBoard provides 1/16 th of the 300 MHz band width. To bring these Nband=16 bands together requires either an 10GbE Ethernet switch in between the Arts BF and the Arts PL or a 24 port Infiniband switch in the Arts PL. For SC1 and SC3 it is also feasible to use a 1GbE/10GbE switch.

### Pipeline (PL) processing

The streaming pipeline (PL) processing operates on the TAB-444 Stokes I data. Per workstation the Stokes I input data rate per workstation is 4.44 Gbps = 0.555 GByte/s, this needs to be processed by the GPUs. The GPU processing may also need access to the DDR3 and it will need IO to output results via 1GbE.

# Critical system requirements and hardware constraints

## Commensal modes

Apertif X, Arts SC1 and SC4 do not have to run at the same time. Hence Apertif X, Arts SC1 and SC4 can reuse the same hardware and then run their dedicated application.

Arts SC2 requires local interferometer data from Apertif X (SR-028 [1]), but only for the central CB. Hence for Arts SC2 it is sufficient to implement only the central CB part of the Apertif X. The purpose of Arts SC3 is to run in parallel with Apertif X (SR-038 [1]) hence Arts SC3 requires running in parallel to the full Apertif X.

## Fringe stopping

Fringe stopping (FS) applies to both Apertix X and Arts BF and is a task that can be done in the Apertif BF. Typically fringe stopping is a two-step process that consists of a true sample delay before a filterbank and phase tracking per frequency channel after the filterbank. For the Apertif the true sample delay tracking (DT) occurs on the ADC samples at the input of the Apertif BF followed by phase tracking (PT) of the beamlet data to stop the residual fringe. The phase tracking needs to be done per beamlet, because it depends on the subband frequency and on the CB direction.

The delay tracking involves duplicating an input sample or skipping an input sample. The phase tracking involves phase rotation by a complex multiplication. The phase tracking must be synchronous to the delay tracking, because a delay step causes a phase step. For the Nyquist frequency a delay step of one sample causes a phase step of 180 degrees. The delay step also disturbs the subband filtering if it is not accounted for. To have a smooth delay tracking the ADC data with the extra sample or the skipped sample needs to be reprocessed by the subband filterbank for the duration of the filterbank impulse response. The subband filterbank in the Apertif BF has 16 taps and an FFT size of 1024, so with 8 bit ADC samples this requires storing at least 16 kByte per ADC input.

Currently the delay tracking is implemented in RAM in the Apertif BF, but only accounts for buffering the  number of samples that are expected for the maximum geometrical delay difference between dishes. The phase tracking is not implemented yet in the Apertif BF. The smooth delay tracking to recalculate the subbands after each delay step is also not implemented yet in the Apertif BF. If the internal RAM is limited, then the delay tracking may require using external memory DDR3. After every delay step the data subband filterbank processing needs to catch up with the input data rate of 200 MHz. This implies that for smooth delay tracking the subband filterbank will need to run at slightly more than 200 MHz. An alternative to smooth delay tracking is to flag the 16 beamlet time samples as being disturbed by the delay step. However for each dish the delay step is typically applied at a different instant, so this then causes quite some flagging. It is sufficient to flag the start of a disturbance, it is not necessary to exactly flag all data that will get affected. Without smooth delay tracking it is necessary to have some rudimentary flagging of the delay step instances, to avoid inform the scientists that there may be small artefacts in the data. The smooth delay tracking can then be added in a later stage of the development.

The fringe stopping may best be implemented in the Apertif BF, because that is the central location. When the fringe is stopped then all user applications of the Apertif BF output data can then rely on it.

.

## Transient data buffer

### CB-444 voltage data

Arts SC3 requires a transient data buffer that can store 10 s of CB-444 data (SR-040 [1]). The CB-444 voltage data is only available within the Arts BF so it cannot be stored in the Arts PL. From Table 13 it follows that the Apertif BF output load LBF_CB444 = 3.2 Tbps with Wbeamlet =6 bit. To store 1 s of this CB voltage data requires 0.4 TByte. Hence to store 10 s requires 4 TByte.

### TAB-444 integrated power data

Arts SC4 requires a transient data buffer that can store 15 s of integrated full Stokes TAB-444 data (SR-035 [1]). This data is available in the FPGA beamformer and may be available in the GPU PL provided that the full Stokes data is output to the PL. From Table 14 it follows that the integrated full Stokes data rate is LTAB444_stokes_int = 426.24 Gbps = 53.3 GByte/s with Wpower = 8 bit. To store 1 s of this TAB power data requires 53.3 GByte, so to store 15 s requires 0.8 TByte.

The DM search range in the Arts PL is about 5 s. This DM search range is slided in steps of 1 s. Therefore with 1 s margin before and after the detection the minimum required storage time is 5 + 2*1 = 7 s. This would require about 0.4 TByte.

### External memory operation

During an observation the external memory is used as a circular buffer that is continuously written with the streaming CB or TAB data. When a trigger occurs then the writing stops to freeze the buffer. If the external memory is in the Arts BF then the Arts PL will provide a trigger to the Arts BF probably via the Arts MAC. All CB or TAB data gets written, but only the one CB or a few TAB that had the trigger need to be read. The read access control is typically done via the Arts MAC. Therefore the read data rate is much lower than the write data rate. The read data may be output to the Arts PL via 1GbE or 10GbE (see section 6.18.5).

### External memory in Arts

Table 25 summarizes the external memory that is available within the Arts BF when either UniBoard or UniBoard2 is used and in the Arts PL when CUDA 4230 GPU Workstations are used.

| Platform: | UniBoard | UniBoard2 | CUDA 4230 GPU Workstation |
| --- | --- | --- | --- |
| Number of FPGA per board | 8 | 4 | - |
| Number of DDR3 memory modules per FPGA | 2 | - | - |
| Number of DDR4 memory modules per FPGA | - | 2 | - |
| Number of DDR3 memory modules per workstation |  |  | 12 |
| Number of boards in Arts BF | 16 | 4 | - |
| Number of work stations in Arts BF | - | - | 24 |
| Total number of DDR memory modules in Arts BF | 256 | 32 | 288 |
| Total memory in Arts BF using 16 GByte DDR memory modules | 4 TByte | 0.5 TByte | 4.6 TByte |
| Total memory in Arts BF using 32 GByte DDR memory modules | - | 1 TByte | - |
| Transfer rate per memory module | 800 MTps | 2400 MTps | 1866 MTps |
| Data rate per memory module (64b data, write only, ~80% effective) | 40 Gbps | 120 Gbps | 95 Gbps / 6 |
| Total access rate | 10 Tbps = 1.25 TByte/s | 3.8 Tbps = 0.47 TByte/s | 0.4 Tbps= 0.05 TByte/s |

Table 25: External memory in Arts

Conclusion:

- 16 UniBoard can store 4 Tbyte which is sufficient for the CB voltage data (4 TByte for SC3) or TAB power data (0.8 TByte for SC4).

- 4 UniBoard2 can store 1 TByte which is sufficient for the TAB power data (0.8 TByte for SC4) and when SC4 can run commensal then SC3 is no longer needed. The 32 GByte modules are not available yet from Micron. With 16 Gbyte modules that are available from Micron the 4 UniBoard2 can store 0.5 TByte, which implies storing 0.5/0.8 * 15 s = 9.3 s. This 9.3 s is also sufficient, because the minimum is about 7 s (see section 6.3.2).

- 24 GPU workstations can have sufficient memory to fit the CB voltage data or TAB power data, but the DDR3 access rate store the TAB power data may be an issue. In Table 25 it is assumed that the 12 DDR3 modules per workstation can be accessed via two ports in parallel (12/ 2 = 6).

## Apertif X integation interval transpose Tint in the Apertif BF

As shown in Figure 5 and Figure 8 both Apertif X and Arts BF use the same Apertif BF output. The Apertif BF performs a transpose over Tint = 1 s to reorder the CB data appropriately for the Apertif X. For Arts SC 3 this Tint introduces a delay of 1 s which can be an issue if Arts needs to trigger other systems (SR-034 [1]).

The transpose over Tint also influences the order in which the TAB weights need to be applied. However TAB weights are not needed in SC3. For the other SC1, 2, and 4 that do use TAB weights the transpose over Tint can be bypassed in the Apertif BF, but that requires a new bypass function in the Apertif BF and a second Apertif BF CB-444 output data format. The Apertif BF output format for Apertif X with transpose over Tint is specified in [4]. The Apertif BF output format with transpose over Tint includes an additional interleave factor Ninterleave = 2 to group the nof_un=8 beamlets per subblock. For Apertif BF output without  with transpose over Tint this additional interleave factor Ninterleave = 2 is not needed, because then the data blocks already have the beamlets at the fastest array index.

To avoid dependencies with the Apertif X it is better to bypass the Tint and to use a dedicated Apertif BF output for Arts. In case the commensal SC2 and SC3 run on the same UniBoards as Apertif X then Tint cannot be bypassed, but this scenario is unlikely to fit.

## Transpose Tsp

### On UniBoard

The CB-444 transpose Tsp that brings together all Nsp=24 SP on one node on UniBoard is achieved in two steps. The first step is done by the wiring of the Nlink=384 links in Figure 8. The second step requires an input redistribution function in the FPGAs that uses the mesh interconnect on the UniBoard to keep 1/nof_un= 1/8 part at each node and transport the rest to the other nof_un-1=7 processing nodes (PN). After the complete Tsp transpose each PN can process about NCB/nof_uni = 37/8 ≈ 5 CB for one complete band and all SP as shown in Figure 11. Hence the Tsp in Figure 11 preserves the full band of CBBW/Nband = 300/16 MHz or NFN=24 subbands. An alternative scheme would be to preserve the FoV by letting each PN process all 37 CB, but only 1/nof_un = 1/8 part of the band. For Arts it is necessary to preserve the full band.

Figure 11: Tsp transpose of the CB-444 data to UniBoard via fiber and to each PN via the mesh

### Load on the UniBoard mesh

The CB-444 input load per PN on UniBoard is LBF_CB444 / Muni / nof_un = 3.2 Tbps / 16 / 8 = 25 Gbps. This CB data needs to be redistributed via the mesh: 1/8 remains at this PN and 7/8 is send across. Then 4/8 remains across and 3/8 needs to be send across again, because a BN can reach an FN directly but another BN only via an FN (and similar for FN to BN or FN). Hence the total data rate to the mesh per PN is (7+3)/8 * 25 Gbps = 31.25 Gbps. This load is transported to nof_fn = nof_bn = 4 nodes across so 31.25 Gbps / 4 = 7.8125 Gbps per BN-FN link. The capacity of the mesh is 12 Gbps per BN-FN link.

### Using UniBoard2

The advantage of UniBoard2 is that only the first step done by the wiring of the Nlink=384 links in Figure 8 is already enough to get the Nsp links to one node. No further redistribution functionality is needed which makes the firmware design smaller and simpler. The PN on UniBoard2 only needs to align the Nsp=24 inputs. On UniBoard2 one PN does what one UniBoard does, in total there are Nband=16 PN and each PN processes all 37 CB for one band and all SP as shown in Figure 12.

Figure 12: Tsp transpose of the CB-444 data on UniBoard2 via fiber directly to each PN

## Channel band width and time resolution of the Stokes beam data

For SC3 and SC4 the CB beamlets need to be separated into Nchan=4 channels (SR-0.30 [1]). The beamlet bandwidth set by the subband bandwidth Bsub = 781250 Hz or 1 MHz, so the channel bandwidth is Bchan = 195.3125 or 250 kHz. The smaller channel band requirement causes:

- The channel filter is implemented as a polyphase filterbank and consumes substantial resources in the FPGA.

- The resolution of the Stokes power beam data must TStokes ≤ 50 μs (SR-0.31 [1]). This at most about Nint = TStokes*Bchan ≤  9.8 or 12.5 channel samples can be integrated. Therefore set Nint=10 as used in Table 14. Hence given a required time resolution TStokes ≤ 50 μs the data output rate is increases with Nchan.

## Streaming output full Stokes or output only Stokes I data

If only Stokes I data is output then LTAB444_stokes_I_int_band= 6.66 Gbps for SC4, so factor NStokes=4 less. However the disadvantage is then that the transient data buffer with full Stokes data has to be on the Uniboards (see section 6.3).

## Number of bits per sample

The number of bit per sample Wbeamlet= 6 bit, Wtab= 6 bit and Wpower = 8 bit have a direct impact on the Arts BF output load concerning:

- The total output data rate

- Whether the data can be transported via an 1GbE links or has to use 10GbE links

- The integer number of links that are needed per UniBoard

The Arts BF data output rates for SC1, 2, 3 and 4 are listed in Table 14. The LTAB1_band = 450 Mbit for SC1 and LIAB37_stokes_int_band = 2.22 Gbps for SC3 are small enough to be transported via the 1GbE switch on UniBoard. The LCB12_band = 5.4 Gbps and LTAB12_band = 5.4 Gbps for SC2 is too much for the 1GbE on UniBoard, so it requires using at least one 10GbE link. The LTAB444_stokes_int_band= 26.64 Gbps for SC4 requires using at least 3 10GbE link per Uniboard. If only the Stokes I data needs to be transported then LTAB444_stokes_I_int_band= 6.66 Gbps and then 1 10GbE link per UniBoard is enough for SC4.

### Data packing and unpacking

To avoid too much unpacking overhead in the Arts PL is may be necessary to transport the CB voltage data for Wbeamlet = 6 bit and TAB voltage data for Wtab = 6 bit with 6 bit per bytes instead of as packed 6 bit. In that case LTAB1_band = 600 Mbps for SC1 (still fits on a 1GbE link), and LCB12_band = 7.2 Gbps and LTAB12_band = 7.2 Gbps for SC2 (still fits on a 10GbE link). The 6 bit should not be simply sign-extended to 8 bit, because the extra 2 bits are then better used to transport 2 more MSbits of the internal TAB data to reduce the number of clipped samples.

## Duplicate Apertif BF output

Dedicated UniBoards or UniBoard2 for Arts can be connected in parallel to the UniBoards for Apertif X using a second 10G output on the FN of the Apertif BF as shown for 16 UniBoards in Figure 13.

Figure 13: Duplicate Apertif BF output to 16 Uniboards for Arts BF

The pro and con of duplicating the Apertif BF output are:

Pro:

- Dedicated beamlet output for Arts can be tapped of before the Tint integration period transpose that is used for Apertif X (see section 6.4)

- Arts and Apertif X are independent apart from that they share the same Apertif BF

Con:

- Development to provide the extra 10G output in Apertif BF

- Extra set of 384 long distance fiber optics needed. The fibers are already available.

## Pass on Apertif BF output in daisy chain

Dedicated UniBoards or UniBoard2 for Arts series by letting the Apertif X pass on the data as shown for 16 UniBoards in Figure 14. Note that the pass on scheme can be repeated in a daisy chain to add even more boards in series in case more processing is needed or other applications need access to the CB-444 data.

Figure 14: Pass on Apertif BF output via Apertif X to 16 Uniboards for Arts BF

The pro and con of passing on the Apertif BF output are:

Pro:

- Apertif X may pass on the input CB data or Arts can benefit from transpose Tsp that is done on Apertif X.

- Vice versa Arts may pass on the data to Apertif X, which can be used to save resources in either Arts BF or Apertif X.

Con:

- Set of 384 short fiber cables needed

- Development of pass on function in firmware of Apertif X

- Arts can only operate if the Apertif X is also active and relies on Apertif X to always pass on the data.

## Using the same 16 UniBoards of Apertif X also for Arts

Figure 15: Arts BF on 16 UniBoards

Pro:

- Saves the cost of 16 extra UniBoards for Arts

- Fits SC1 and 4 because these are not active at same time as Apertif X.

Con:

- SC2 and SC3 commensal mode and Apertif X probably will not fit together with Apertif X regarding IO, processing and memory.

- Combining SC2 and SC3 with Apertif X in one FPGA firmware design complicates the development.

- Switching between dedicated Apertif X FPGA image and Arts FPGA image requires that they both fit in the flash, because rewriting the flash too often wears the flash (>10000x erase/write is allowed according to datasheet).

## Using 16 dedicated UniBoards for Arts

Pro:

- Arts SC1,2,3,4 can always run in parallel with Apertif X

- If SC4 can run commensal then SC3 is not needed, which saves the development effort for SC3.

Con:

- Cost of 16 extra UniBoards

- Provide the extra 10G output in Apertif BF (section 6.9) or pass on CB data in Apertif X (section 6.10).

## Using more than 16 dedicated UniBoards for Arts processing

Pro:

- With 32 Uniboards and Nband=16 inputs used still 8 10G ports are available for Arts BF output

- With 32 Uniboards there is a factor 2 more DDR3 memory available then with 16 UniBoards

Con:

- Extra cost

- Arts can only operate if the Apertif X is also active.

- Only feasible for multiple of 16 extra UniBoards, so eg. 32

For the original CB-444 data the number of UniBoards needs to be a multiple of 16, whereby the data is then passed on in a daisy chain from Apertif-X to the first set of 16 UniBoards and then to the next set of 16 UniBoard. For the Tsp transposed CB-444 data the number of UniBoards also needs to be a multiple of 16, it is not possible to use eg. only 24 UniBoards for Arts then because each PN physically has only nof_10G = 3 ports and functionally it can only parallelize  the data further over 5 CB (see section 6.5.1). The band (NFN) and the SP (Nsp) need to be kept together, so parallelization can only occur for the 5 CB. However the main restriction is that without a switch it is not possible to distribute the data to other than nof_10G = 3 destinations.

## Using more than 4 dedicated UniBoard2s for Arts processing

To increase the processing and memory capabilities for Arts it is possible to use more than 4 UniBoard2 for Arts. However similar as in section 6.13 the number of Uniboard2 needs to be a multiple of 4 so that they can be connected in a daisy chain to pass on all Nband=16 bands that form the CBBW = 300 MHz.

Adding only one extra UniBoard2 can only add extra processing for ¼ part of the CBBW = 300 MHz, so 75 MHz. One extra Uniboard2 cannot add 25 % more TABs for the entire band, because that would require a transpose Tband for which one UniBoard2 does not have sufficient IO ports.

## Output redistribution inside the Arts BF

### Via 1GbE

On UniBoard and UniBoard2 all PN are connected directly to a 1GbE switch with 4 external ports. The 1GbE network is also used for MAC, but the spare capacity is still close to 100% and can be used for data output offload by the PN.

### Via the UniBoard mesh for 10GbE output

If there need to be less than nof_un=8 10GbE output per UniBoard, then the PN have to use the mesh to pass on their output to another PN. To redistribute not only the Tsp input data but also the Tband output data via the UniBoard mesh complicates the implementation but is possible. The capacity of the mesh is 12 Gbps per BN-FN link. From section 6.5.2 it follows that there is still about 4 Gbps spare capacity per BN-FN link on the mesh. The output redistribution scheme depends on the number of 10GbE outputs that are needed per UniBoard. Table 26 shows the maximum load on per BN-FN link on the mesh for 1, 2, 3 or 4 10GbE output ports per UniBoard. For the lowest data rate per BN-FN link it is necessary have all outputs on one side (eg. at the FN) to use only one output port per node and to distribute the data to all 4 BN for the first hop and to all output FN. Separating the data over multiple streams complicates the implementation. If the output data is not separated then the load per BN-FN link equals the load per PN.

| Number of 10GbE outputs per UniBoard | Maximum load per PN | Output node | Transport scheme for the mesh | Maximum load per BN-FN link |
| --- | --- | --- | --- | --- |
| 1 | 1.25 G | FN0 | BN3:0 send directly to FN0 FN3:1 send to FN0 via BN3:1 | 2 * 1.25G = 2.5 G |
| 1 | 1.25 G | FN0 | BN3:0 send directly to FN0 FN3:1 send to FN0 via BN3:0 | (8-1) / 4 / 1 * 1.25G = 2.1875 G |
| 2 | 2.5 G | FN0, BN0 | BN2:0 send directly to FN0 FN2:0 send directly to BN0 | 2.5 G |
| 2 | 2.5 G | FN1:0 | BN-i sends to FN1:0 FN3:2 send to BN3:0 | (8-2) / 4 / 2 * 2.5G = 1.875 G |
| 3 | 3.75 G | FN2:0 | BN-i sends to FN2:0 FN3 sends to BN3:0 | (8-3) / 4 / 3 * 3.75G = 1.5625 G |
| 4 | 5 G | FN3:0 | BN-i sends to FN-i | 5 G |
| 4 | 5 G | FN3:0 | BN-i sends to FN3:0 | 5 G/ 4 = 1.25 G |

Table 26: Load per BN-FN link for output redistribution via the mesh in Gbps

## Output data reorder

### In time

The data in an Ethernet packet may need to be reordered in the Arts FPGA beam former to ease the processing in the Arts PL GPU cluster. Reordering data takes RAM resources of the FPGA, because the data needs to be stored before it can be read in the required output order.

### Per final destination

The Arts BF will have to reorder the output data such that the packet payloads are already prepared for their final destination. For the option with the Infiniband switch the workstation could unpack the incoming packets and distribute their contents to the final destinations, but it probably does not have time to do this.

## Unidirectional 10GbE links

For UniBoard + OEB and for UniBoard2 all optical 10G ports are used for receiving the Aperif BF data. These ports can still be used for transmitting data to the GPU PL, but only in a unidirectional way. Typically an Ethernet link is full duplex to support e.g. ARP and ping, therefore it needs to be verified that a unidirectional Tx only 10GbE link between UniBoard and a GPU server is possible (both directly or via 1 switch).

### Using UniBoard2 to convert unidirectional 10GbE to full duplex 10GbE

If unidirectional links cannot easily be connected to 10GbE network interfaces then a UniBoard2 can be used to convert an unidirectional link into a full duplex 10GbE link as shown Figure 16. Uniboard2 with HEM has in total 24 + 8 = 32 optical links per PN. For SC4 the full Stokes output has LTAB444_stokes_int = 426.24 Gbps, so then 3 ports per UniBoard are needed as shown in Figure 16.

Figure 16: Using UniBoard2 + HEM to convert Tx only to bidir 10GbE	 (3 ports for full Stokes)

If only the Stokes I is output then LTAB444_stokes_I_int = 106.56 Gbps and then using 1 port per UniBoard and only PN0 on UniBoard2 suffices as in Figure 16 or alternatively using two PN on UniBoard2 without HEM suffices as shown in Figure 17. The data exchange between PN0 and PN1 in Figure 17 uses the ring IO that is available between the PN on UniBoard2 (see Figure 10).

Figure 17: Using two PN on UniBoard2 without HEM to convert Tx only to bidir 10GbE (1 port for Stokes I)

### Using UniBoard2 for processing

If 4 UniBoard2s with HEM can be used for processing instead of 16 UniBoards then the issue of unidirectional links vanishes as shown in Figure 18, because UniBoard2 with HEM has still 8 more 10GbE ports available per PN.

Figure 18: Using UniBoard2 + HEM for processing to avoid use of Tx only 10GbE ports for output

## Transpose Tband

### TAB-1 for SC1 using 1GbE and a dedicated switch

For SC1 the load LTAB1 = 7.2 Gbps (Table 14) from Nband = 16 Uniboards can be collected to a single GPU server by means of a Ethernet switch with at least 16 1GbE ports and 1 10GbE port as shown in Figure 19. The switch also implements the transpose Tband, because it brings together the TAB-1 data for the entire CBBW to a single link.

Figure 19: Arts BF output for SC1 via 1GbE network

Remarks:

- The TAB-1 data for SC1 will be calculated on FN0, the other PN on UniBoard are only needed for the transpose Tsp to get all SP at FN0. Hence the SC1 output could also be done via the 10GbE output of FN0 on the 16 UniBoards. However then a 10GbE switch is needed with 16 + 1 = 17 ports.

- For SC1 probably only 1 workstation suffices, so then it is beneficial that Tband is implemented by the switch as well.

### TAB-444 for SC4 via 10GbE

For SC4 the LTAB444_stokes_int = 406.24 Gbps (Table 14) from Nband = 16 Uniboards or from 16 PN on 4 UniBoard2s. As shown in Figure 16 and Figure 18 this requires a total link capacity of 3 * 16 * 10GbE = 480 bps. If only Stokes I is transported then 1 * 16 * 10 GbE = 160 Gbps is needed. The transpose Tband can be done via:

- 10GbE switches each with 32 ports, see Figure 20

- 1 Uniboard2 + HEM, see Figure 21

- 1 UniBoard2 using two PN, see Figure 22

- An Infiniband switch that interconnects the workstations in the GPU cluster, see Figure 23.

Figure 20: Tband for SC4 via 10GbE switches

Figure 21: Tband for SC4 via one UniBoard2 + HEM

Figure 22: Tband for SC4 via one UniBoard2 without HEM by using two PN

Figure 23: Tband for SC4 via an Infiniband switch that interconnects the GPU workstations

If an Infiniband switch performs Tband and if the workstations in the GPU cluster have a multiple of Nband = 16 10GbE ports, then point-to-point links can be used between Arts BF and Arts PL, see Figure 23.

The advantage of using a UniBoard2 as in Figure 21 is that the PN on UniBoard2 can convert unidirectional Tx only input to full duplex output (section 6.17.1) and the PN can also perform output data reordering (see section 6.16). The advantage of using an Ethernet switch or Infiniband switch is that this requires no FPGA firmware development. An Inifiniband switch costs as much as a few workstations, therefore it may be appropriate to have an Infiniband switch available in the GPU cluster anyway, even if it is not used to do the transpose Tband.

### CB-12 and TAB-12 for SC2

For SC2 the load is LCB12 = 86.4 Gbps or LTAB12 = 86.4 Gbps. The SC2 only uses the central CB and this data is available on FN0. Similar as for SC1 the other PN on UniBoard are only needed for the transpose Tsp to get all SP for the central CB at FN0. With Nband = 16 UniBoards the output load per board is 5.4 Gbps so using 1 10GbE port per FN0 suffices. For the interconnect to the GPU cluster SC2 can use the interconnect of SC4 as shown in section 6.18.2.

### IAB-37 for SC3

For SC3 the LIAB37_stokes_I_int = 8.88 Gbps (Table 14) from Nband = 16 Uniboards. Per UniBoard this requires 1 1GbE link so to collect the data from Nband = 16 UniBoards requires 1 Ethernet switch with 16 1GbE ports and 1 10GbE port similar as for SC1. The IAB-37 data for SC3 will be calculate on all PN, so the on board switch aggregates the data from the FN and BN.

### Transient buffer data readout for SC3 and SC4

After a trigger the transient data only needs to be read for the CB (in case of SC3) or the few TABs (in case of SC4) in which the transient occurred. The load for 1 CB for Ndish = 12 dishes is LBF_CB12=86.4 Gbps, so 10 s is 864 Gbit = 108 Gbyte. This amount is best offloaded via 10GbE interfaces. The load for 1 TAB is LTAB1_stokes_int=0.96 Gbps, so 15 s is 14.4 Gbit = 1.8 GByte. This data for 1 TAB or a few TABs can be read via the 1GbE control interface or offloaded via the 1GbE interface.

# Hardware status

## UniBoard

- The 4 GByte DDR3 is working on UniBoard and on all 4 UniBoards in an Apertif BF subracks (used for Tint storage of Apertif X).

- The prototype CoBI / OEB is not working yet (necessary to support Nsp=24 instead of only 12)

- Order and verify using a 16 GByte DDR3 memory on UniBoard (necessary for 10 sec CB-444 voltage data buffer with SC3)

## UniBoard2

- The UniBoard2 prototype is being tested. The 1GbE and 10GbE interfaces are all working, however the DDR4 memory can be written and read but still has some data bit errors. The first production board of UniBoard2 with Arria10 will arrive December 2015.

- The HEM prototype board is currently in the schematic design layout phase.

- Production samples for the Stratix10 FPGAs are expected in Q3 2016 and engineering samples in Q4 2015. Hence a first production board of UniBoard2 with Stratix10 can be available end 2016.

- Order and verify using a 16 GByte and a 32 GByte DDR4 memory module on UniBoard2 (necessary for 15 sec TAB-444 power data buffer with SC4)

## GPU cluster

- The Dragnet GPU cluster for pulsar and fast radio transient search with LOFAR can serve as reference example.