Detailed Design of the Arts FPGA Beamformer

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Eric Kooistra | ASTRON | 2015 |
| Controle / Checked: |  |  |
| Andre Gunst | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Andre Gunst | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature Andre Gunst | ASTRON |  |
| © ASTRON 2015 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Joeri van Leeuwen (JvL) Andre Gunst (AG) Daniel van der Schuur (DS) Hajee Pepping (HJP) Alwin Zanting (AZ) Leon Hiemstra (LH) | Gert Kruithof (GK) Wim van Capellen (WvC) |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2015-05-13 | E. Kooistra | First draft that describes the CB input. |
| 0.2 | 2015-06-10 | E. Kooistra | Added sections on: - Bsub=1 MHz change request - TAB, IAB data processing relations - CB12, TAB-1/12/444, IAB-37 output - DSP resource usage (multipliers, RAM) |
| 0.3 | 2015-07-06 | E. Kooistra | Defined Muni = Nband = 16. |

Table of contents:

1	Introduction	8

1.1	Scope	8

1.2	Specification	9

1.3	Change request Bsub = 1 MHz	9

2	System overview	10

2.1	Apertif BF subsystem	10

2.2	Apertif X subsystem and Arts subsystem	10

3	Hardware architecture	12

3.1	Apertif BF using Uniboard	12

3.2	Signal path transpose to Arts	12

3.3	Arts using UniBoard	13

4	Data path processing	15

4.1	Channel filterbank	15

4.2	Stokes power vector (IQUV)	15

4.3	Tied array beamformer (TAB)	15

4.3.1	‘Voltage’ beams	15

4.3.2	‘Power’ beams	15

4.4	Incoherent array beamformer (IAB) – ‘power’ beams	15

4.5	Requantization	16

5	Streaming data interfaces	17

5.1	System load overview	17

5.1.1	CB input load from Apertif BF	17

5.1.2	Arts BF output load to Arts PL	17

5.2	CB signal path input	18

5.2.1	Entire system	18

5.2.2	Per UniBoard	19

5.2.3	Distribution on the UniBoard mesh	19

5.2.4	Per PN	20

5.3	Beamlet mapping in the Apertif BF	21

5.4	Processing input and output	21

5.4.1	Channel filterbank	21

5.4.2	Tied array beamformer (TAB)	21

5.4.3	Incoherent array beamformer (IAB)	22

5.5	SC1 TAB-1 output	23

5.6	SC2 TAB-12 output	23

5.7	SC2 CB-12 output	24

5.8	SC3 IAB-37 output	25

5.9	SC4 TAB-444 output	26

6	Transient data capture	27

6.1	External memory on UniBoard	27

6.2	Capture integrated power data	27

6.3	Capture CB voltage data	27

7	Memory-mapped control interface	28

8	Processing	28

9	Storage	28

10	Verification	28

11	Appendix : VHDL component resource usage and IO formats	29

11.1	Available FPGA resources on UniBoard	29

11.1.1	Multipliers	29

11.1.2	Memory	29

11.2	PFB component	29

11.2.1	Multiplier usage	29

11.2.2	Memory usage	30

11.2.3	Input - output	31

11.3	Reorder component	31

11.3.1	Memory usage	32

11.3.2	Input - output	32

11.4	BF unit component	32

11.4.1	Multiplier usage	32

11.4.2	Memory usage	33

11.4.3	Input - output	33

11.5	IBF unit component	33

References:

- “Arts Requirements Specification”, ASTRON-RS-020, J. van Leeuwen

- “Apertif System Requirement Specification”, ASTRON-RS-006, Rev C, W.A. van Capellen

- “Analysis of tied-array beamforming for Arts”, ASTRON-MEM-191, S.J. Wijnholds

- “Impact analysis of change request to use 1 MHz beamlets instead of 0.78125 MHz beamlets in Apertif and Arts”, ASTRON-CR-032, E. Kooistra

- “Detailed Design of the Digital Beamformer System for Apertif”, ASTRON-RP-413, G. Schoonderbeek, A. Gunst, E. Kooistra

- “Apertif Beamformer Firmware Description”, ASTRON-RP-1377, D. vd Schuur

- “Detailed Design of the Correlator System for Apertif”, ASTRON-RP-1400, G. Schoonderbeek, A. Gunst, E. Kooistra, HJ Pepping

- “Representing Streaming Data”, ASTRON-RP-1486, E. Kooistra

- “Apertif Beamformer Output Interface Specification”, ASTRON-SP-061, E. Kooistra

- “Understanding Digital Signal Processing”, 3rd edition, R. Lyons

- “Apertif Filterbank Firmware Specification Part 2”, ASTRON-SP-054, E. Kooistra

- "Selection and Reorder modules", ASTRON-RP-1399, H.J. Pepping

- “Uthernet interface specification”, ASTRON-SP-041, E. Kooistra

- “Data path packet interface specification”, ASTRON-SP-042, E. Kooistra

Terminology:

ADC		Analogue to Digital Conversion

ADU		Analogue to Digital Unit (board with 8 ADC)
AGC		Automatic Gain Control 
Apertif		APERture Tile In Focus

Arts		Apertif Radio Transient System

beam		Group of beamlets that point in the same direction

beamlet		Beam formed subband, a small beam spanning one subband

BF		BeamFormer

BN		Back Node FPGA on UniBoard

bps		Bits per second

BSN		Block Sequence Number (timestamp)
BW		BandWidth

CB		Compound Beam, formed at dish level over the FPA
channel		Unit frequency band within a beamlet
cint		Complex integer

CPU		Central Processing Unit

CR		Change Request
CW		Carrier Wave (single frequency signal)

DP		Data Path (streaming interface)

DSP		Digital Signal Processing

DT		Delay Tracking in Aperif BF
eop		End of Packet (or frame, or block)
FFT		Fast Fourier Transform

FN		Front Node FPGA on UniBoard

FoV		Field of View
FPA		Focal Plane Array (= PAF)
FPGA		Field Programmable Gate Array

FR		Functional Requirement
FRB		Fast Radio Burst

Fsub		Subband filterbank in Apertif BF
Fchan_x		Channel filterbank in Apertif X
Fchan		Channel filterbank in Arts BF for SC3 and SC4

FS		Fringe Stopping (is DT + PT)
GbE		Gigabit Ethernet

GPU		Graphics Processing Unit
HDL		Hardware Description Language

IAB		Incoherent array beam, formed by incoherently combining dishes

IBF		Incoherent BeamFormer

Iab-let		An incoherently beamformed beamlet, spanning one channel
Im		Imaginary

int		Signed integer

IO		Input Output

MAC		Multiply and Accumulate, Medium Access, Monitoring and Control

MM		Memory Mapped (control interface)

node		Processing node (PN), typically one FPGA chip
Nof		Number of

OEB		Optical-Electrical Board (provides UniBoard BN with same optical IO as the FN)

PFB		Poly phase Filter Bank
power beam	Full Stokes power values: I, Q, U, V

PAF		Phased Array Feed (= FPA, better use term FPA)
PL		Pipeline processing
PN		Processing Node (FN or BN), PN0:3 = FN0:3, PN4:7=BN0:3

PPS		Pulse Per Second
PT		Phase Tracking in Aperif BF
Re		Real

RF		Radio Frequency

Rsub		Reorder and select subbands for CBBW=300 MHz in aperitif BF

Rbeam		Reorder and select beamlets for NCB=37 compound beam directions aperitif BF

SC		Science Case
SNR		Signal to Noise Ratio

sop		Start of Packet (or frame, or block)
SP		Signal Path, 1 CB consists of Npol = 2 SP, 1 SP per Apertif BF subrack

SR		Science Requirement

ST		Streaming, statistics
sps		Samples per second

subband	Frequency band, unit output of the filterbank

TAB		Tied array beam, formed by coherently combining dishes
tab-let		A coherently beamformed beamlet, a grating or pencil beam within the CB 
Tant		Transpose to group data from all S = 64 (≥ Nant) antenna elements in the FPA
Tdish		Transpose to group data from all Ndish = 12 dishes
Tpol		Transpose to group data from both Npol = 2 polarizations
Tsp		Transpose to group data from all Nsp = Npol * Ndish signal paths, so combines Tdish and Tpol 
Tband		Transpose to group data from all Nband = 16 bands

Tintegration	Transpose to group data from an integration interval of Nint_x values in time
TFoV		Transpose to group data from all NCB = 37 beams for the full FoV

ToA		Time of Arrival
TT		Terrestrial Time
uint		Unsigned integer

VLBI		Very Large Baseline Interferometry
voltage beam	Dual polarization sample values with phase information:  Xre, Xim, Yre, Yim

WSRT		Westerbork Synthesis Radio Telescope
X		Correlator

Definitions:

Ncomplex	2		Two part of a complex number, the real and imaginary part 
Npol	2		Number of polarizations, X and Y
NStokes	4		Number of power values in the Stokes vector [I, Q, U, V]
Ndish	12		Number of WSRT dishes in Apertif
Nsp	24		Number of signal paths = Ndish * Npol at the output of the Apertif BF
Nant	61		Number of antennas in the frontend FPA of the Apertif BF

S	64		Number of ADC signal paths in the frontend FPA of the Apertif BF (≥ Nant)

SBN	4		Number of ADC signal paths per BN in the frontend FPA of the Apertif BF

fs	800 MHz	Digitizer sample frequency of the ADC at the Apertif BF frontend
Ts	1.25 ns		= 1/ fs, digitizer sample period
f0			Lower edge frequency of a subband, beamlet or channel [1]
RFBW	400 MHz	= fs/2, sampled RF bandwidth 
CBBW	300 MHz	Full bandwidth of the CB and also of the TAB and IAB (SR-0.2)
Bsub	781250 Hz	Subband bandwidth in Apertif BF, = beamlet bandwidth
fclk	200M		Data processing clock rate in the FPGA, = fs / P

P	4		Wideband rate factor of sample clock rate divided by digital processing clock rate

Pcmult	4		Number of real multiplications per complex multiplication

N	1024		FFT size of the FFT in the Apertif BF subband polyphase filter

Nclk	256		= N/P, number of DP clock cycles per subband period
Nsub	512		= N/2, number of subbands that covers RFBW=400MHz
Nsel	384		Number of selected subbands to cover CBBW=300 MHz
Nband	16		= nof_fn_bf, Number of bands in the Apertif BF to process the full CBBW
NFN	24		= Nsel/Nband , number of subbands per band or per FN in the Apertif BF

NFNpol	48		= NFN*Npol, number of subbands per dual polarization band

NCB	37		Required number of compound beams
KCB	40		Implemented average number of beamlets per subband (≥ NCB)

PBF	4		Number of parallel BF units per FN in the Apertif BF

Nblk	240		≤ Nclk, number of valid DP clock cycles per subband period in the Apertif BF

Nbeamlet	960		Number of compound beamlet slots per FN output, maximum PBF * Nclk = 1024,

actual PBF * Nblk = 960, required NCB * NFN = 888

Ninterleave		2	= nof_un/PBF, additional beamlet output interleave factor

Pinterleave		2	Number of interleaved streams that are unfolded in parallel at rate 1/Pinterleave

Mblk	=Nblk/Ninterleave, default block size per subband period in the Apertif X and Arts

Ngr	12		Required number of TAB grating lobe patterns to cover the full CB (SR-0.41)
NVLBI	12		Required number of TABs in the central CB for VLBI, choose = Ngr (SR-0.23)
KTAB	12		Implemented number of TABs per beamlet (≥ Ngr)

NTAB	444		= NCB*KTAB, number of TABs 
NIAB	37		= NCB, number of IABs 
Nlink	384		= NPN, number of physical 10G output links of the Apertif BF, so 1 link per PN
NPN	384		= Nsp * Nband, total number of parallel processing nodes in the Apertif BF
MPN	128		= Nband * nof_un, total number of parallel processing nodes in the Arts

Muni	16		= Nband = nof_fn_bf, total number UniBoards in the Arts BF and in Apertif X

MPFB_units	12	Number of parallel PFB units per PN for Nsp = 24 SP

MBF_units		12	Number of parallel BF units per PN for KTAB = 12 TABs

MIBF_units		1	Number of parallel dual polarization IBF units per PN for the one IAB per CB

Nchan_x	64		Number of channels per beamlet in the Apertif X

Nchan	4		Number of channels per beamlet, for SC3 and SC4
Bchan			= Bsub/Nchan, channel bandwidth within a beamlet, for SC3 and SC4

Nint_x	800000		Number of channel power values that are integrated in the Apertif X
Nint	≈ 10		Number of Stokes channel power values that are integrated in Arts
TStokes	≈ 50 μs		Minimum required sample period for the Stokes power values
fStokes	≈ 20 kHz	= 1/TStokes, minimum required sample frequency for the Stokes power values
nof_uni		4	Number of UniBoards per polarization and dish in the Apertif BF

nof_bn		4	Number of back node FPGAs (BN) per UniBoard

nof_fn		4	Number of front node FPGAs (FN) per UniBoard

nof_un		8	= nof_fn + nof_bn, number of processing node FPGAs per UniBoard

nof_10g	3	Number of 10G links per FPGA node on UniBoard

nof_pn			Number of processing nodes (BN or FN)

nof_bn_fb	16	= nof_uni*nof_bn, number of subband filterbank BN per SP in the Apertif BF

nof_fn_bf	16	= nof_uni*nof_fn, number of beamformer FN per SP in the Apertif BF

byte_w		8	Number of bits in a byte or an octet

word_sz	4	Number of bytes per 32 bit long word
longword_sz	8	Number of bytes per 64 bit long word
Wbeamlet		6	Word width in number of bits of a beamlet voltage sample
Wchan		6	Word width in number of bits of a channel voltage sample
Wtab 		4	Word width in number of bits of a TAB voltage sample
Wpower 		4	Word width in number of bits of a IAB or TAB power sample

# Introduction

## Scope

Arts [1] implements the tied array and VLBI functionality of Apertif [2]. Figure 1 shows the place of Arts within Apertif. Both the Apertif correlator (X) [6] and Arts use the beam data from the Apertif beamformer (BF) [4].

Figure 1: Top level overview of Apertif with Arts included

Within Arts the processing is consists of a FPGA beamformer and a GPU pipeline, as shown in Figure 2. Arts has four science cases (SC) and for all four SC the FPGA beamformer will be implemented on Uniboards.

Figure 2: Arts FPGA beamformer and GPU pipeline

This document specifies the detailed design for the Arts FPGA beamformer (BF) on UniBoard FPGAs. At the input interface the Arts BF receives NCB=37 compound beams from Ndish=12 dishes from the Apertif BF. At the output interface the Arts BF outputs compound beams (CB) for SC2, tied array beams (TAB) for SC1, 2 and 4 or incoherent array beams (IAB) for SC3 to a GPU cluster for further processing.

Note the difference between the Apertif BF and the Arts BF. The Apertif BF forms the compound beams over the focal plane array of each dish. These compound beams are input to Aperitif X and to Arts. The Arts BF uses the compound beams to form IAB or TAB over the array of dishes.

## Specification

This detailed design document is the L3 specification of the FPGA beamformer because it specifies how the FPGA beamformer should be implemented on Uniboards to fulfill all L0 science, L1 system and L2 subsystem requirements that are specified in [1]. This document only specifies the FPGA firmware design and the required FPGA interconnect and IO architecture. The UniBoard hardware and the subrack hardware are assumed to be available. The output to the pipeline processing (PL) is specified. The pipeline processing is not described in this document, because the PL will be implemented on a GPU cluster. Similar the monitoring and control (MAC) interface is specified, but the MAC design itself is not described.

The array notation that is used in this document to describe the data format for the streaming data interfaces is explained in [7].

## Change request Bsub = 1 MHz

There is a pending change request from Arts SC1 and SC2 for the Apertif BF to change Bsub from 781250 Hz to 1 MHz [3]. This will ripple through in all related parameters. In this document the parameter values for Bsub = 781250 Hz are still used as default. Table 1 shows how the parameter values will change for Bsub = 1 MHz.

| Parameter | Bsub = 781250 Hz | Bsub = 1 MHz | Remark |
| --- | --- | --- | --- |
| N | 1024 | 800 | = fs / Bsub, keep fs = 800 MHz |
| Nsub | 512 | 400 | = N / Ncomplex |
| Nsel | 384 | 304 | Nsel * Bsub ≥ CBBW = 300 MHz |
| NFN | 24 | 19 | = Nsel/Nband |
| Nbeamlet | 960 | 760 | = KCB * NFN, keep KCB = 40 |
| Nclk | 256 | 200 | = N / P, keep P = 4 and fclk = fs / P |
| Nblk | 240 | 190 | = Nbeamlet / PBF |
| Bchan | 195.3125 kHz | 250 kHz | = Bsub / Nchan |
| Bchan_x | 12.207 kHz | 15.625 kHz | = Bsub / Nchan_x |

Table 1: Parameter values that depend on Bsub

# System overview

## Apertif BF subsystem

The Apertif BF separates the digitized data from the dish FPA into subbands by means of a filterbank and then it forms beamlets for these subbands. The beamforming (BF) for one single polarization of the FPA cannot be done on a single FPGA node for the full bandwidth, so therefore the subband load has to be distributed across Nband = nof_fn_bf = 16 processing nodes. The beamlet for one subband requires the input from all FPA elements, so therefore there needs to be a transpose Tant that groups the subbands from all S = 64 antennes. A compound beam (CB) is formed by a group of Nsel= 384 beamlets all with the same direction that span CBBW = 300 MHz. Figure 3 shows the filterbank  Fsub, the transpose Tant and the beamformer (BF) that is distributed over Nband nodes. The Tintegration transpose is used for the Apertif correlator (X), for Arts it needs to be bypassed. The MAC takes care of the proper operation, the subband selection and the BF weights.

Figure 3: The Apertif BF subsystem

## Apertif X subsystem and Arts subsystem

In the appendix of [1] various options for the Arts subsystem were investigated. Figure 4 shows the selected option for the Arts subsystem and how it relates to the Apertif X subsystem. The same Tdish and Tpol transpose that are needed for Apertif X to group the data from all Nsp = Npol * Ndish = 24 signal paths can also be used for Arts.

Figure 4: The Apertif X subsystem and the Arts subsystem

SC1 and SC2 use the central CB so they only need to process CB-12, where Ndish = 12. For SC1 only one TAB-1 needs to be made within the CB. For SC2 TAB-12 needs to be made, where the number of TABs for VLBI is NVLBI = 12. For SC2 it must also be possible to pass on the CB-12.

Apertif X, SC3 and SC4 use all CB, so they process CB-444, where Ndish * NCB = 12 * 37 = 444. For SC3 the IAB-37 needs to be made which is one incoherent ‘power’ beam for each CB. For SC4 the TAB-444 needs to be made which is KTAB = 12 ‘voltage beams per CB, so KTAB * NCB = 12 * 37 = 444. Both SC3 and SC4 output integrated (over Nint samples) Stokes (IQUV) ‘power’ beam data to the pipelining (PL).

# Hardware architecture

## Apertif BF using Uniboard

The Apertif BF outputs NCB=37 compound beams with CBBW=300 MHz. The Apertif BF beam forms the FPA input per polarization and per dish. The single dish, single polarization output of the Apertif BF is called a signal path (SP) and to beam form 1 signal path requires a subrack with nof_uni=4 UniBoards. To be able to distribute the processing over nof_fn_bf=16 front nodes (FN) on nof_uni=4 UniBoards the Apertif BF is separated into Nband= nof_fn_bf =16 frequency bands. Figure 5 shows the Apertif BF subrack with 4 UniBoards. The 4 UniBoards connect to the ADU ADC cards and to each other via a backplane. Each FN in the subrack uses one 10GbE port to output its frequency part of the signal path.

Figure 5: One Apertif BF subrack per signal path with nof_uni=4 UniBoards and Nband=16 FN

## Signal path transpose to Arts

In total the Apertif BF has Nsp = Npol * Ndish = 24 signal paths so also 24 subracks, 96 UniBoards and NPN=Nsp*Nband = 384 processing (front) nodes. Hence the total Apertif BF output is carried via Nlink=NPN=384 10GbE links as shown in Figure 6. For both the Apertif X and for Arts the Nsp=24 signal paths from the Apertif BF need to be transposed to gather them together. This transpose Tsp can be implemented by interconnecting the Apertif BF to Nband=Muni = 16 Uniboards as shown in Figure 6. Each of the Nband= Muni =16 UniBoards in Figure 6 processes 1/Nband part of the CBBW band for all Nsp=24 signal paths.

Figure 6: Apertif BF transpose interconnect to Apertif X and Arts

## Arts using UniBoard

Figure 7 shows the UniBoard with the Optical-Electrical Board (OEB). The OEB is needed to be able to use fiber optics IO for the BN. For the Arts application (and also for the Apertif X application) the distinction between FN and BN is not needed, because all nof_un = nof_fn + nof_bn = 8 FPGA have the same function. Therefore the FPGAs on UniBoard are also referred to as processing nodes (PN). Each PN has nof_10G = 3 10G links so in total the UniBoard has nof_un * nof_10G = 24 10G links. This is just enough IO to accept the input from Nsp=24 links. Hence the number of input links determines that all nof_un = 8 FPGAs on UniBoard have to be used for Arts.

Figure 7: One UniBoard to process 1/Nband part of the CBBW for Nsp=24 signal paths

The mesh interconnect on UniBoard consists of gigabit transceivers that connect each BN to each FN and each FN to each BN. On UniBoard the Nsp=24 inputs need to be distributed further via the on board mesh interconnect to gather them together at each PN. Therefore the input needs to be divided into 1/nof_un parts to evenly distribute the processing load over the PN.

In total the Arts FPGA beamformer requires Nband = Muni = 16 UniBoards as shown in Figure 8. The Muni=16 UniBoards are independent, because each processes 1/Nband part of the CBBW. Similar the Apertif X also uses Nband = Muni = 16 independent UniBoards.

Figure 8: Nband = Muni = 16 independent UniBoards for the Arts BF

# Data path processing

## Channel filterbank

The channel filter is indicated by Fchan in Figure 4. For SC3 and SC4 the beamlet bandwidth of Bsub is too wide so therefore an additional channel filter is needed that separates the beamlets into Nchan = 4 channels. Hence Bchan = Bsub/Nchan = 195.3125 kHz or 250 kHz, dependent on whether Bsub = 781250 Hz or 1 MHz respectively.

The channel filterbank in Arts and in the Apertif X reuse the same WPFB component that is also used for the subband filterbank in the Apertif BF. The channel filter is implemented by a poly-phase filterbank (PFB). The PFB has a FIR pre-filter section and an FFT section. The number of taps in the FIR filters determines the steepness of the pass-stop band transition. The stop band attenuation is independent of the number of taps. The subband PFB in the Apertif BF uses Ntaps=16. However for the channel filters in Arts using Ntaps=16 may take too many multiplier resources, therefore set Ntaps=8.

## Stokes power vector (IQUV)

The Stokes power vector [I, Q, U, V] consists of powers that are based on both polarizations X and Y:

Equation 1:

I=x*conjx+y*conjy

Q=x*conjx-y*conj(y)

U=2*Rex*conjy

V=-2*Imx*conjy

The advantage of the (Stokes) power values is that they can be integrated to achieve data reduction.

## Tied array beamformer (TAB)

### ‘Voltage’ beams

The tied array beamformer is indicated by TAB in Figure 4 and operates on ‘voltage’ samples. For SC1 TAB-1 and SC2 TAB-12 the beamformer operates directly on the beamlets samples. For SC4 the TAB-444 operates on the channel samples. The TAB in Arts reuses the same BF component that is also used for the subband beamformer in the Apertif BF that makes beamlets of the compound beams (CB).

### ‘Power’ beams

For SC4 the Stokes powers of the TAB-444 voltage beam data are calculated conform Equation 1 and then integrated over Nint ≈ 10 samples as shown by TAB-444  IQUV ∑ in Figure 4. The integration achieves a data rate reduction of  1/Nint * (Wpower/WTAB).

## Incoherent array beamformer (IAB) – ‘power’ beams

The incoherent array beamformer is indicated by the IAB in Figure 4 and operates on ‘power’ samples. The incoherent beamforming makes that the IAB has the same FoV as the CB and that the IAB does not use weights. Therefore there is one IAB per CB. The input ‘power’ data is calculated conform Equation 1. The integration over Nint ≈ 10 samples can be done before the IAB as shown in Figure 4 by IQUV ∑IAB-37 or after the IAB. The incoherent beamformer (IBF) component is new but merely consists of a adder tree to sum the ‘power’ data from the Ndish = 12 inputs.

## Requantization

Wbeamlet = 6 bit, Wchan = 6 bit, WTAB = 6 bit, Wpower = 8 semi floating point with 1 bit exponent.

# Streaming data interfaces

## System load overview

### CB input load from Apertif BF

Table 2 copied from [1] list loads that can be defined regarding the Apertif BF output interface assuming that the Apertif BF outputs beamlets with Wbeamlet = 6 bits.

| Load | Equation | Value | Description |
| --- | --- | --- | --- |
| LBF_SP1 | = CBBW * Ncomplex * Wbeamlet | 3.6 Gbps | Load for 1 SP |
| LBF_SP1_band | = LBF_SP1 / Nband | 225 Mbps | Load for 1 SP per band (= per BF node) |
| LBF_SP37_band | = NCB * LBF_SP1_band | 8.325 Gbps | Load for NCB = 37 SP per band (= per BF node) |
| LBF_CB1 | = Npol * LBF_SP1 | 7.2 Gbps | Load for 1 CB (= 2 SP, Npol = 2) |
| LBF_CB12 | = Ndish * LBF_CB1 = NPN * LBF_SP1_band | 86.4 Gbps | Total load from Ndish = 12 dishes, for 1 CB |
| LBF_CB444 | = NCB * LBF_CB12 = NPN * LBF_SP37_band | 3.2 Tbps | Total load from Ndish = 12 dishes, for NCB= 37 CB |
| LBF_link1 | = LBF_SP1_band | 225 Mbps | Link load for 1 SP per band (= per BF node) |
| LBF_link37 | = LBF_SP37_band | 8.325 Gbps | Link load for NCB= 37 SP per band (= per BF node) |

Table 2: Load definitions for Apertif BF output interface with Wbeamlet = 6 bit

### Arts BF output load to Arts PL

Table 3 copied from [1] list output loads for SC1, SC2, SC3 and SC4 from the Arts BF to the Arts Pipeline (PL).

| Load | SC | Equation | Value | Description |
| --- | --- | --- | --- | --- |
| LTAB1 | 1 | = CBBW * Npol *Ncomplex * Wtab | 4.8 Gbps | Load for voltage TAB-1 |
| LTAB1_band | 1 | = LTAB1 / Nband | 300 Mbps | Load for voltage TAB-1 per PN0 |
| LTAB12 | 2 | = NVLBI * LTAB1 | 57.6 Gbps | Load for voltage TAB-12 |
| LTAB12_band | 2 | = LTAB12 / Nband | 3.6 Gbps | Load for voltage TAB-12 per PN0 |
| LCB12 | 2 | = LBF_CB12 | 86.4 Gbps | Load for voltage CB-12 |
| LCB12_band | 2 | = LCB12 / Nband | 5.4 Gbps | Load for voltage CB-12 per PN0 |
| LIAB1_stokes | 3 | = CBBW * NStokes * Wpower | 4.8 Gbps | Load for IAB-1 without integration |
| LIAB1_stokes_int | 3 | = LIAB1_stokes / Nint | 0.48 Gbps | Load for IAB-1 with Tstokes ≈ 50 μs |
| LIAB37_stokes | 3 | = NCB * LIAB1_stokes | 177.6 Gbps | Load for IAB-37 without integration |
| LIAB37_stokes_int | 3 | = NCB * LIAB1_stokes_int | 17.76 Gbps | Load for IAB-37 with Tstokes ≈ 50 μs |
| LIAB37_stokes_I_int | 3 | = LIAB37_stokes_int / NStokes | 4.44 Gbps | Load for IAB-37-I with Tstokes ≈ 50 μs |
| LTAB1_stokes_int | 4 | = LIAB1_stokes_int | 0.48 Gbps | Load for TAB-1 with Tstokes ≈ 50 μs |
| LTAB444 | 4 | = NCB * Ngr * LTAB1 | 2.1312 Tbps | Load for voltage TAB-444 |
| LTAB444_stokes | 4 | = LTAB444* (Wtab/Wpower) | 2.1312 Tbps | Load for power TAB-444 |
| LTAB444_stokes_int | 4 | = NCB * Ngr * LTAB1_stokes_int | 213.12 Gbps | Load for TAB-444 with Tstokes ≈ 50 μs |
| LTAB444_stokes_I_int | 4 | = LTAB444_stokes_int / NStokes | 53.28 Gbps | Load for TAB-444-I with Tstokes ≈ 50 μs |

Table 3: Load definitions for Arts BF output interface (with Wbeamlet = 6 bit, Wtab = 4 bit, Wpower = 4 bit)

## CB signal path input

### Entire system

The compound beam data output interface of the Apertif BF is defined by [8]:

Equation 2:	cint6CBdish,   pol,   bandtb

The wiring of the Nlink=384 10GbE links in Figure 6 implements the first part of the Tdish and Tpol transpose to group the SP data from all Nsp=24 signal paths per band at a single UniBoard. The Tdish and Tpol transposed beam data input for Arts BF (and also for Apertif X) is defined by (note the swapped band and dish indices):

Equation 3:	cint6CBband,   pol,   dishtb

The subscript indices indicate parallel links and the array index contains serial data on the link. The subscript band has range 0:Nband-1, subscript pol has range 0:Npol-1, subscript dish has range 0:Ndish-1. In total there are Nband * Npol * Ndish = 16 * 2 * 12 = Nlink = 384 parallel links. The array index t increments at the rate of Bsub. The array index b has range 0:Nbeamlet-1 where Nbeamlet  is the number of compound beamlet slots per FN output of the Apertif BF. Required Nbeamlet  ≥ NCB*NFN = 37 * 24 = 888, where NFN = Nsel / Nband. The actual number of beamlet slots that will be implemented is Nbeamlet = KCB * NFN = 40 * 24 = 960. The order of beamlet directions and beamlet frequencies can be mapped to the beamlet slots in almost any order by the reorder function in the Apertif BF [8].

The order of the subscript indices indicates that band 0 maps on UniBoard 0 and band 15 maps on UniBoard 15. The pol index before the dish index implies that the X-pol inputs are connected via stream [0:11] to FN0:3 and the Y-pol inputs are connected via stream [12:23] to BN0:3 as shown in Figure 7. Based on this the pol and dish indices can be mapped to port and pn indices and to index sp = 0:Nsp-1 = 0:23 according to:

Equation 4:	sp= dish+ pol*Ndish=port+ dish*nof_10G

Equation 5:	pol=sp MOD Npoldish=sp DIV Npol

Equation 6:	port=sp MOD nof_10Gpn=sp DIV nof_10G

With Equation 4 the Equation 3 can be rewritten as:

Equation 7:	cint6CBband,   pn,   porttb

### Per UniBoard

One UniBoard processes one band of all Nsp=24 SP. Call this signal cb_uniboard, so cb_uniboard = CBband where a band is mapped to this UniBoard. For one UniBoard equation Equation 7 then reduces to:

Equation 8:	cint6cb_uniboardpn,   porttb

Internally the Apertif FN beamformer contains PBF=4 parallel BF units that each process Nclk=256 beamlets to achieve Nbeamlet= PBF * Nclk=1024 in total. From these Nclk beamlets only Nblk=240 are actually output [8]. With subscript u = 0:PBF-1 = 0:3 to indicate the parallel BF units and index bu = 0:Nblk-1 = 0:239 to count the number of valid beamlet slots per BF unit, then the relation with the absolute beamlet index b is given by:

Equation 9:	b= u*Nclk+bu

With Equation 9 the Equation 8 can be rewritten as:

Equation 10:	cint6cb_uniboardpn,   port,   utbu

### Distribution on the UniBoard mesh

Next part to complete the Tdish and Tpol transpose is to bring all SP to one processing node (PN) on UniBoard. Therefore the beamlets that are received at each PN need to be split into nof_un=8 parts, whereby one part is kept at this PN and the other nof_un-1 = 7 parts are passed on via the UniBoard mesh to the other 7 PN on the UniBoard. Choose to distribute the beamlets over the nof_un=8 in the order in which they arrive at the PN. The index u already provides range PBF=4 beamlets, therefore define an additional interleave subblock size of Ninterleave = nof_un/PBF = 2 beamlets to be able to distribute the beamlets in order over the nof_un=8 PN. The default block size in the Apertif BF is Nblk=240. The corresponding block size on the UniBoard for the Apertif X and for Arts BF is therefore call Mblk=Nblk/Ninterleave=120. Equation 10 can then be rewritten as:

Equation 11:	cint6cb_uniboardpn,   port,   utbu_ibi

Whereby the relation between index bu and index bi = 0:Ninterleave-1 =0:1 and bu_i = 0:Mblk-1 = 0:119 is given by:

Equation 12:	bu= bu_i+ bi*Ninterleave

Define index dest = 0:nof_un-1 = 0:7 for the destination PN as:

Equation 13:	dest= u+ bi*Ninterleave

With Equation 13 the Equation 11 can be rewritten as:

Equation 14:	cint6cb_uniboardpn,   port,   desttbu_i     @ 1/Pinterleave

Whereby the Ninterleave = 2 beamlets in series are unfolded as Pinterleave = Ninterleave = 2 parallel streams that run at 1/Pinterleave rate of fclk.

Together the indices port and dest cover the entire range of SP, because Nsp = nof_10g * nof_un = 3*8 = 24. The order of the indices port and dest can be swapped at no cost, because all streams are available in parallel within the PN FPGA. It is convenient to swap port and dest, because then the Nsp=24 streams are again in incrementing order. Therefore rewrite Equation 14 as:

Equation 15:	cint6cb_uniboardpn,   dest,   porttbu_i     @ 1/Pinterleave

The beamlets stream for which index pn = dest remains on this PN and the beamlet streams for which pn != dest are received from the corresponding other PN via the UniBoard mesh. This transport operation across the UniBoard mesh implements the last part of the Tdish and Tpol transpose. Starting with Equation 15 this swaps the dest and pn indices and results in:

Equation 16:	cint6cb_uniboarddest,   pn,   porttbu_i     @ 1/Pinterleave

With Equation 4 the Equation 16 can be expressed in terms of the SP index sp = 0:Nsp-1:

Equation 17:	cint6cb_uniboarddest,   sptbu_i     @ 1/Pinterleave

### Per PN

One processing node (PN) on UniBoard processes 1/nof_un = 1/8 part of the beamlets of one band of all Nsp=24 SP. Call this signal cb_node, so cb_node = cb_uniboarddest where dest is this PN. For one PN Equation 17 then reduces to:

Equation 18:	cint6cb_nodesptbu_i     @ 1/Pinterleave

The index bu_i = 0:Mblk-1 = 0:119, so there are Mblk=120 beamlets per SP stream. The 1/Pinterleave = ½ rate  implies that 2 SP can be folded (multiplexed) in per stream. Choose to fold the Npol=2 SP that form a dual polarization CB into one stream. Equation 18 then becomes:

Equation 19:	cint6cb_nodedishtbu_ipol

## Beamlet mapping in the Apertif BF

The beam direction and beam band of the Mblk=120 beamlets in Equation 19 depends on the two programmable reorder stages in the Apertif BF [8] as shown by Rsub and Rbeam in Figure 3. The reorder stage Rsub in the BN filterbank of the Apertif BF maps the subband frequencies to the band of NFN=24 frequencies that are output to one UniBoard in Aperitif X and Arts. The reorder stage Rbeam in the FN beamformer of the Apertif BF replicates these frequency slots to create KCB beamlets per frequency. The order of  the Nbeamlets = KCB * NFN = 960 beamlets in this reorder stage can eg. be set for maximum band per PN or for maximum FoV per PN:

- For maximum band per PN the Mblk beamlets contain KPN = Mblk /NFN = KCB/nof_un= 5 beamlet directions (so 5 out of the KCB = 40 CB) each covering a full band of NFN=24 beamlet frequencies.

- For maximum FoV per PN the Mblk beamlets contain Mblk /KCB = NFN/nof_un = 3 frequencies each covering a full FoV of KCB = 40 beamlet directions (so all CB).

For Arts the reorder stage in the FN beamformer of the Apertif BF needs to be set for full band per PN. Each PN can process KPN = 5 full band beamlet directions. Assume that the central CB is mapped to the first full band beamlet direction in PN0.

## Processing input and output

### Channel filterbank

For SC3 and SC4 the beamlets have to be separated into Nchan = 4 channels. This can be done with a poly-phase filterbank (PFB) that is also used for the subband filterbank in the BN of the Apertif BF. For Arts call the beamlet filterbank output ch_nodesp, then with the PFB function described in appendix 11.2 applied to the beamlet inputs of Equation 19 yields:

Equation 20:	cintch_nodedisht_chchbu_ipol

The range of index ch = 0:Nchan-1 = 0:3, so every 4 beamlet inputs in time yield 4 channel outputs in frequency. The channel rate is fchan = fsub/Nchan, so the channel time index t_ch increments at a rate 1/Nchan compared to the beamlet time index t. Within Arts the channels are kept together, so therefore from a data transport point of view there is no difference in routing groups of Nchan beamlet time samples as in Equation 18 or routing Nchan channel frequency samples as in Equation 20, except for that the data width may differ due to Wchan ≥ Wbeamlet.

The number of parallel data streams that is needed to implement the entire channel filterbank for all Nsp=24 SP is MPFB_units = 12 as given by:

Equation 21:	 MPFB units=Ndish=Nsp/Npol=Nsp/Ninterleave

### Tied array beamformer (TAB)

The BF component that is used in the FN of the Apertif BF is a ‘voltage’ beamformer and can therefore also be reused to beamform the TABs in Arts. The BF weights and adds the Ndish = 12 inputs. For Arts call the TAB output tab_node, then with the BF function described in appendix 11.4 applied to the beamlet inputs of Equation 19 yields:

Equation 22:	cinttab_nodetbu_ipol=BFcb_nodedishtbu_ipol

The complex output samples of TAB are called tab-lets. A tab-let is a tied array beamformed beamlet. For SC1 TAB-1 and SC2 TAB-12 the tab-lets are creating using cb_nodedish as in Equation 19. For SC4 TAB-444 the tab-lets have to be created per channel, so then using ch_nodedish from Equation 20 as input yields.

Equation 23:	cinttab_nodet_chchbu_ipol=BFch_nodedisht_chchbu_ipol

In total Arts needs to make up to KTAB = 12 TAB per beamlet (i.e. per CB). From Equation 22 it follows that within one stream the beamlets cannot be replicated because the interleave factor Pinterleave is already used to fold the Npol = 2 polarizations. Therefore to have KTAB = 12 TABs the stream can simply be to be replicated KTAB times in parallel. Hence per PN each TAB needs one BF unit. Each BF unit takes the same cb_nodedish input streams as described by Equation 22 and Equation 23. In total per PN there are MBF_units = 12 BF units in parallel:

Equation 24:	 MBF units=Ktab=Ktab*Npol/Ninterleave

Call the TAB Stokes power data tab_stokes_node. Applying Equation 1 to Equation 23 then yields:

Equation 25:	cint_2tab_stokes_nodet_chchbu_ist

The range of index st = 0:NStokes/2-1 = 0:1 and represents the 4 integer powers of the Stokes vector that are packed with IQ in parallel and UV in parallel. The two parallel values are represented by the (int_2) type cast. The TAB Stokes power data is integrated over every Nint ≈ 10 samples in time. Call the TAB Stokes power data tab_stokes_int_node. Summing every Nint samples in Equation 26 then yields:

Equation 26:	cint_2tab_stokes_int_nodet_intchbu_ist    @ 1/Nint

where t_int = (t_ch – t_ch_offset) / Nint and t_ch_offset is typically 0.

### Incoherent array beamformer (IAB)

The IAB is defined by the Stokes vector [I, Q, U, V] which consists of real powers that are based on both polarizations X and Y. Call the CB stokes data cb_stokes_node. Applying Equation 1 to the beamlet channel inputs of Equation 20 yields:

Equation 27:	int_2cb_stokes_nodedisht_chchbu_ist

The IAB adds these powers for all Ndish=12 dishes. Call the IAB output iab_stokes_node. The summation by the IBF over the Ndish inputs then yields

Equation 28:	int_2iab_stokes_nodet_chchbu_ist

The range of index st = 0:NStokes/2-1 = 0:1 and represents the 4 integer powers of the Stokes vector that are packed with IQ in parallel and UV in parallel. The two parallel values are represented by the (int_2) type cast. There is only 1 IAB per beamlet (i.e. per CB), so Equation 28 describes the entire IAB per PN. The full Stokes power output samples of IAB are called iab-lets. An iab-let is an incoherent array beamformed beamlet. The iab-let contains Nstokes = 4 real power values that are formed from the dual polarization complex beamlet values, so also Npol * Ncomplex = 4 real values.

One IAB unit processes both Npol = 2 polarizations and there is only 1 IAB per beamlet. Therefore the number of parallel dual polarization data streams that is needed to implement the IAB per CB is:

Equation 29:	 MIBF units=1

## SC1 TAB-1 output

For SC1 only one TAB for only the central CB needs to be output. The TAB-1 for SC1 uses the beamlets of the central CB as input. One PN can process KPN = 5 full band beamlet directions. The central CB can be mapped PN0 and mapped to the first of these beamlets directions of PN0 (section 5.3). For SC1 only one BF unit is needed. In this way the one TAB for the central CB is available via the first NFNpol = NFN * Npol = 24*2 = 48 tab-lets of the stream. Call the range bo = 0:NFN-1. Take range bo from range bu_i in Equation 22. The SC1 TAB-1 tab-lets output per PN0 is then given by:

Equation 30:	cinttab1_node_outputtbopol

In total there are Nband = Muni = 16 PN0 output nodes for SC1, so the total SC1 TAB-1 with range band = 0:Nband-1 becomes:

Equation 31:	cinttab1_outputbandtbo[pol]

The data rate per link conform Equation 30 is LTAB1_band = NFN *Npol *Ncomplex *Wtab *Bsub = 24 *2 *2 *4 *781250 = 300 Mbps. This LTAB1_band = 300 Mbps can be offloaded via the 1 GbE control interface of the UniBoards as shown in Figure 9. The total output load for SC1 conform Equation 31 is LTAB1 =Nband * LTAB1_band = 4.8 Gbps, which agrees with Table 3. Figure 9 shows in green that only PN0=FN0 is used for the TAB-1 processing of SC1, the other PN are only needed to pass on the data from the other SP to PN0 as indicated in yellow.

Figure 9: SC1 TAB1 output

## SC2 TAB-12 output

The TAB-12 for SC2 uses the beamlets of the central CB as input. Like for SC1 (section 5.5) the central CB can be mapped PN0 and mapped to the first NFNpol = NFN * Npol = 24*2 = 48 beamlets of PN0 (section 5.3). To have KTAB = 12 TABs the stream can simply be to be replicated KTAB times in parallel, similar as needed for SC4, rather than trying to make use of the Nblk – NFNpol = 240 – 48 = 192 beamlet slots that are also available. Define index range tab = 0:KTAB-1. Take range bo from range bu_i in Equation 22. The SC2 TAB-12 tab-lets output per PN0 is then given by:

Equation 32:	cinttab12_node_outputtabtbopol

In total there are Nband = Muni = 16 PN0 output nodes for SC2, so the total SC2 TAB-12 output with range band = 0:Nband-1 becomes:

Equation 33:	cinttab12_outputband,tabtbopol

The data rate per link conform Equation 32 is LTAB12_band = KTAB *NFN *Npol *Ncomplex *Wtab *Bsub = 12 *24 *2 *2 *4 *781250 = 3.6 Gbps. This LTAB12_band = 3.6 Gbps can be offloaded via one 10 GbE output port of the PN0 on the UniBoards as shown in Figure 10. The total output load for SC2 conform Equation 33 is LTAB12 =Nband * LTAB12_band = 57.6 Gbps, which agrees with Table 3. Figure 10 shows in green that only PN0=FN0 is used for the TAB-12 processing of SC2, the other PN are only needed to pass on the data from the other SP to PN0 as indicated in yellow.

Figure 10: SC2 TAB12 output

## SC2 CB-12 output

For SC2 it is also required to be able to pass on and output the original central CB from the individual dishes. Like for SC1 (section 5.5) the central CB can be mapped PN0 and mapped to the first NFNpol = NFN * Npol = 24*2 = 48  beamlets of PN0 (section 5.3). Take range bo = 0:NFN-1 from range bu_i in Equation 19. The SC2 CB-12 beamlets output per PN0 is then given by:

Equation 34:	cint6cb12_node_outputdishtbopol

In total there are Nband = Muni = 16 PN0 output nodes for SC2, so the total SC2 CB-12 with range band = 0:Nband-1 becomes:

Equation 35:	cintcb12_outputband,dishtbo[pol]

The data rate per link conform Equation 34 is LCB12_band = Ndish *NFN *Npo l*Ncomplex *Wbeamlet *Bsub = 12 *24 *2 *2 *6 * 781250 = 5.4 Gbps. This LCB12_band = 5.4 Gbps can be offloaded via one 10 GbE output port of the PN0 on the UniBoards as shown in Figure 11. The total output load for SC2 conform Equation 35 is LCB12 =Nband * LCB12_band = 86.4 Gbps, which agrees with Table 3. Figure 11 shows in that only PN0=FN0 is used for the CB-12 output of SC2. PN0 is drawn in yellow, because the CB-12 output does not require processing. The other PN are only needed to pass on the data from the other SP to PN0 as indicated in yellow.

Figure 11: SC2 CB12 output

## SC3 IAB-37 output

CB-444UIQV Nint IAB-37  output ????

Figure 12 shows in green that all PN are used for the IAB-37 processing of SC3.

Figure 12: SC3 IAB37 output

## SC4 TAB-444 output

The TAB-444 for SC4 uses the channels of all CB as input. There are NCB = 37 CB and for each CB there are KTAB = 12 TABs so NTAB = KTAB * KCB ≥ KTAB * NCB = 444.

TAB-444UIQV  Nint  output ????

Figure 13 shows in green that all PN are used for the TAB-444 processing of SC4.

Figure 13: SC4 TAB444 output

# Transient data capture

## External memory on UniBoard

Each PN on UniBoard has two DDR3 memories of maximum 16 GiByte each. In total the Arts BF has MPN = 128 PN so 4 TiByte.

## Capture integrated power data

For SC3 and SC4 the integrated Stokes power values need to be captured for 15 s (SR-0.35). The integrated power samples are output to the PL so therefore these transient capture buffers are implemented in the PL. for SC4 the load is given by LIAB_stokes_int = 17.75 Gbps and for SC4 the load is given by LTAB444_stokes_int = 213.12 Gbps.

## Capture CB voltage data

For SC3 the input ‘voltage’ samples of the CB-444 need to be captured for Tcapture = 10 s (SR 0.40). The ‘voltage’ samples need to be stored on the UniBoards, because they are not output to the PL. From Table 2 it follows that for Wbeamlet = 6 bit the total CB input load is LBF_CB444 = 3.2 Tbps = 400 GByte/s. Hence if the Muni = 16 UniBoards of the Arts BF are equipped with 16 GiByte DDR3 memories then it has just sufficient storage to capture 10 s of CB-444 input ‘voltage’ beamlet data.

When the PL detects a transient then only the CB in which the transient occurred needs to be read. This amounts to reading Tcapture * LBF_CB444 / NCB / MPN = 10 * 3.2 T / 37 / 128 = 6.7 Gbit per PN.

# Memory-mapped control interface

Apertif BF control (DT, Rsub, Rbeam, BF weights).

Apertif X control (none, except for adding status meta data to Apertif X output).

Arts BF:

- Default board control (as with unb1_minimal reference design)

- Default IO test control (as with unb1_test reference design)

- TAB control (BF weights)

- IAB control (none)

- CB-444 transient data buffer trigger and read out

Control rate:

- Only at start of measurement (e.g. reorder/selections)

- Occasionally (e.g. for each noise source calibration, read out transient capture buffers)

- Regularly during measurement (e.g. DT and BF coefficients)

# Processing

- Block diagram (IO terminals, BSN aligners + DSP for SC1, 2, 3, 4)

- New Stokes IQUV powers + intergation component

- Reuse PFB for channel filterbank

- Reuse BF for TAB

- New IBF component for IAB (= common_adder_tree)

- One image for all SC or one image revision per SC ????

# Storage

- 4, 8 or 16 G DDR3 modules

- Store on Arts Uniboards (not on Apertif BF UniBoards)

- Store voltage values or intergrated power values (Wpower = ????)

- Buffer size (seconds) ????

- Trigger control and read out control ????

# Verification

- Unb1_test for IO

- Where to put BG and DB for Arts test  ????

# Appendix : VHDL component resource usage and IO formats

## Available FPGA resources on UniBoard

### Multipliers

The Stratix IV FPGA on UniBoard has 1288 (18x18)-bit real multipliers. These multipliers are grouped per four to fit one complex multiplication. The factor Pcmult = 4 is used to define that 1 complex MAC requires 4 real MACs. For a complex multiplication all 4 real multipliers can be used, because a complex 18x18 bit multiply yields a complex 36x36 bit product that fits in the 72 bit output. Four independent real multiplications cannot fit, because they would need 144 bit output, whereas only 72 bit output is available per group of four (18x18)-bit real multipliers. Therefore a group of four (18x18) bit real multipliers can fit:

- 1 complex (18x18) bit multiply

- 2 independent real (18x18) bit multiplies

- 4 independent real (9x9) bit multiplies

- 2 complex (9x9) bit multiplies ???

### Memory

The Stratix IV FPGA on UniBoard has 1235 M9K RAM blocks. One M9K RAM block is 1024 x 9 bit large, so size is about 1 Kbyte or exactly SM9K = 9 kbit. The M9K RAM blocks can be used with various data widths (WM9K) and address depths (DM9K), whereby SM9K = DM9K * WM9K. The maximum data width to access a M9K block is WM9K=36 bit and the depth is then at least DM9K=256 words, so 36x256. Other combinations are 18x512 and 9x1024. Translating number of bits into number of M9K is difficult to predict in advance because it is implementation dependent:

- The limited number of possible block RAM dimensions can cause that not all bits or words can be used.

- Parallel instances (streams) that have the same control  can share the same RAM, but this may depend on whether the synthesis tool recognizes this optimization

Therefore in practice it is necessary to do a trial synthesis to get a more reliable estimate of the RAM.

## PFB component

### Multiplier usage

The PFB consists of two FIR filters with real coefficients and an NFFT points (complex) FFT. The number of (18x18)-bit multiplier elements per PFB instance is given by the sum of Equation 36 and Equation 37:

Equation 36:	 FIRnof mult=Ntaps*Pcmult*1Ffit

Equation 37:	 FFTnof mult=log2NFFT*Pcmult

These Equation 36 and Equation 37 are partly empirical. The data rate per signal path is fdata and the processing clock rate is fclk. When fclk < fdata then the wideband factor fdata / fclk = P and the PFB becomes a wideband PFB (WPFB), as is the case for the subband filter in the Apertif BF. When fclk ≥ fdata and then fclk / fdata = Ninterleave is the number of multiplexed signal paths that can be processed by a single PFB component, as is the case for the channel filter in Arts. The number of PFB instances that is needed to implement the entire PFB for all SP is given by:

Equation 38:	 MPFB units=Nsp*fdatafclk=Nsp/Ninterleave

The number of multipliers in the FIR part is equal to the number of taps. The FIR coefficients are real and the input per each FIR filter is also real, however this does not reduce the number of multipliers, because the fitter cannot make use of this to fit them in the FPGA when the operands are wider than 9 bit. Therefore in Equation 36 Ffit =2 when the multiplier operands are ≤ 9 bit and Ffit=1 when the operands are ≤ 18 bit.

In theory the FFT would require 0.5*log2(NFFT) multipliers, but the radix-2 FFT implementation achieves only 50% utilization. The last stage of an FFT reduces to multiplication by ±1 which would yield log2(NFFT)-1, however the FFT implementation does not yet make use of that.

Table 4 shows the multiplier usage of the channel filterbank with Nchan=4 in the Arts BF. For comparison the resource usage of the similar channel filterbank with Nchan_x=64 for the Aperif X are shown. The numbers for the Apertif X channel filterbank agree with fitter reports from FPGA synthesis. The number of multipliers can be reduced by using them with 9x9 bit operands (data and FIR coefficients) instead of 18x18bit operand as shown by respectively Ffit = 2 and Ffit = 1.

| Application | Ntaps | Ffit | NFFT | FIRnof_mult | FFTnof_mult | PFBnof_mult |
| --- | --- | --- | --- | --- | --- | --- |
| Arts BF | 8 | 1 | 4 | 24/2 *8 *4/1 = 384 | 24/2 *log2(4)*4 = 96 | 480 |
| Arts BF | 8 | 2 | 4 | 24/2 *8 *4/2 = 192 | 24/2 *log2(4)*4 = 96 | 288 |
| Apertif X | 8 | 1 | 64 | 24/2 *8 *4/1 = 384 | 24/2 *log2(64)*4 = 288 | 672 |
| Apertif X | 8 | 2 | 64 | 24/2 *8 *4/2 = 192 | 24/2 *log2(64)*4 = 288 | 480 |

Table 4: Multiplier usage for the channel filterbank (PFB) with MPFB_units=24/2, Pcmult=4

### Memory usage

The FIR part of the PFB has complex input but real coefficients. The number of memory bits that the implementation of the FIR part for one instance of the PFB needs is given by:

Equation 39:	 FIRnof memory bits=Ntaps*NFFT*Wcoeff+Ninterleave*Wdata*Ncomplex

The FFT part of the PFB needs memory to store one block of data for the FFT operation and a dual-page memory in case the output of the FFT needs to be reordered. Therefore the number of memory bits that the implementation of the FFT part for one instance of the PFB needs is given by:

Equation 40:	 FFTnof memory bits=NFFT*Wtwiddle+Ninterleave*1+Npage*Wdata*Ncomplex

The memory consists of RAM blocks that best fit either 9 bit data or 18 bit data. Therefore assume in Equation 39 en Equation 40 that Wcoeff and Wdata are 9 bit or 18 bit to be able to express then in number of M9K blocks. The data at different taps are processed in parallel and can share the same RAM blocks, but the maximum data width per RAM block is 36 bit, therefore define width WM9K= 36, 18 or 9 bit and corresponding size SM9K = 256, 512 or 1024 words. The coefficients and the data cannot share the same RAM blocks. With these assumptions Equation 39 and Equation 40 can be expressed in M9K RAM block units:

????

Equation 41:	 FIRnof M9K coeff=NFFTSM9K*Ntaps*WcoeffWM9K

Equation 42:	 FIRnof M9K data=Ninterleave*NFFTSM9K*Ntaps*Ncomplex*WdataWM9K

Equation 43:	 FFTnof M9K twiddle=NFFT*Wtwiddle*Ncomplex

Equation 44:	 FFTnof M9K data=NFFT*Ninterleave*Wdata*Ncomplex

Equation 45:	 FFTnof M9K reorder=Npage*NFFT*NinterleaveSM9K*Wdata*NcomplexWM9K

| Application | MPFB_units, Ninterleave, Npage | Ntaps | NFFT | Wcoeff, Wdata | FIR | FFT | PFB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apertif BF | 2, 1, 2 | 16 | 1024 | 18, 18 | 2*(32+64)=192 |  |  |
| Apertif X | 12, 2, 0 | 8 | 4 | 18, 18 |  |  |  |
| Arts BF |  | 8 | 64 | 18, 18 |  |  |  |

Table 5: Memory usage for the channel filterbank (PFB) with Ncomplex=2

????

### Input - output

In general the PFB with input s and output b can be described by:

Equation 46:	bt_ff=PFBst

The range of the frequency index f=0:N-1 and N is the number of frequencies. The actual order of index f may be scrambled due to the FFT in the PFB. For every N inputs in time there are N outputs in frequency, so output time index t_f increments when input time index t has incremented by N.

The PFB component can also operate on multiplexed input signals in series. This can be represented by index ser=0:Nser-1 in:

Equation 47:	bt_ffser=PFBstser

The PFB can also operate on input signals that have a sample rate fs that is a wideband factor P larger than the processing clock rate fclk. Typically Nser=1 when P>1. Therefore the WPFB can be represented by index p=0:P-1 in:

Equation 48:	bpt_ff=WPFBspt

The PFB has a processing gain that requires that the output samples have log2(N)/2 more bits than the input samples [5].

## Reorder component

The reorder component can:

- transpose multiplexed streams into parallel streams

- change the order of the input data

- can omit some input data

- replicate inputs

Replication of inputs can be used to eg. facilitate forming multiple beams per input.

### Memory usage

### Input - output

If the input s with Npar parallel streams (index par) and Nser multiplexed serial streams (index ser) and with Nf frequency samples (index f) per time block (index t) is:

Equation 49:	spartfser

A serial to parallel transpose with index Nsp = par * Nser + ser can be used to yield:

Equation 50:	ssptf    @ 1/Nser

A replication of KCB beams (index k) per frequency can be used to yield:

Equation 51:	ssptfk

The total block size must fit within a block processing interval, so Nf * KCB ≤ Nclk clock cycles.

## BF unit component

The BF unit component consists of a reorder function (section 11.3) and a MAC function. The MAC weights each input SP and then adds them.

### Multiplier usage

The (18x18) multiplier usage of the MAC in the BF unit with S parallel inputs is:

Equation 52:	 BFnof mult=S*Pcmult*fdatafclk*1Ffit

For TAB-12 and TAB-444 each PN needs to calculate KTAB = 12 TAB. The factor fclk/fdata = Ninterleave = 2 in Equation 52, so MBF_units = KTAB * Npol / Ninterleave = 12 BF units are needed in parallel.

Table 6 shows the multiplier usage of the ‘voltage’ beamformer in the Arts BF. For comparison the resource usage of the similar BF for the Aperif BF are shown. The numbers for the Apertif BF agree with fitter reports from FPGA synthesis. The number of multipliers can be reduced by using them with 9x9 bit operands (data and FIR coefficients) instead of 18x18bit operand as shown by respectively Ffit = 2 and Ffit = 1.

| Application | MBF_units | S | Npol | Ndish | Ninterleave | Ffit | BFnof_mult |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apertif BF | 4 | 64 | 1 | - | 1 | 1 | 4*64*4/1/1=1024 |
| Arts BF | 12 | - | 2 | 12 | 2 | 1 | 12*24*4/2/1 = 576 |
| Arts BF | 12 | - | 2 | 12 | 2 | 2 | 12*24*4/2/2 = 288 |

Table 6: Multiplier usage for the ‘voltage’ beamformer (BF) with Pcmult=4

### Memory usage

The BF Mac does not use RAM. If the BF power statistics are needed then these use a few M9K RAM blocks per BF unit.

### Input - output

With Equation 51 the BF component with input s and output b can be described by:

Equation 53:	btfk=BFssptfk

Per time block the BF needs Nsp * N * KCB weights. Typically the same weights can be used for multiple time blocks. The weights are updated via the MM interface.

## IBF unit component