PPS Handler Module Description

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Eric Kooistra | ASTRON |  |
| Controle / Checked: |  |  |
| Gijs Schoonderbeek | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Andre Gunst | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature Andre Gunst | ASTRON |  |
| © ASTRON 2013 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Andre Gunst (AG, ASTRON) Gijs Schoonderbeek (GS, ASTRON) Eric Kooistra (EK, ASTRON) Daniel van der Schuur (DS, ASTRON) Harm-Jan Pepping (HJP, ASTRON) |  |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2013-01-29 | Eric Kooistra | Draft. |

Table of contents:

1	Introduction	4

2	Software interface	4

3	Hardware interface	5

4	Design	6

5	Implementation	7

6	Verification	7

Terminology:

DDIO		Double Data rate IO

DDR		Double Data Rate

DP		Data Path

HDL		Hardware Description Language

IO		Input Output

IOE		IO Element

LSBit		Least Significant bit

MM		Memory Mapped

MSBit		Most Significant bit

PPS		Pulse Per Second

PPSH		PPS Handler

Sl		Std_logic (1 bit)

Slv		Std_logic_vector (bit vector)

ST		Streaming

TB		Test Bench

UNB		UniBoard

Wi		Word Index

References:

- UNB = https://svn.astron.nl/UniBoard_FP7/UniBoard/trunk/

# Introduction

This document describes the PPS Handler firmware module that contains VHDL components for capturing an external periodic time pulse into the system clock domain of the FPGA. Typically the periodic pulse is a pulse per second (PPS) that is locked to the system clock.

The PPS Handler (PPSH) module is located at $UNB/Firmware/modules/ppsh [1].

# Software interface

The PPSH can be controlled and monitored via the memory-mapped (MM) interface. The register map is shown Table 1 and the register fields are described in Table 2.

31             24 23             16 15              8 7               0  wi

|-----------------|-----------------|-----------------|-----------------|

|toggle[31], stable[30]   xxx                        capture_cnt = [n:0]|  0

|-----------------------------------------------------------------------|

|edge[31],                xxx                       expected_cnt = [n:0]|  1

|-----------------------------------------------------------------------|

Table 1: PPSH MM register map

| Register field | Width | R/W | Description |
| --- | --- | --- | --- |
| Toggle | 1 | R | Toggles on each PPS rising edge. |
| Stable | 1 | R | When '1' then the PPS capture_cnt was equal to expected_cnt for every PPS interval since the last read access to wi = 0. Else at least one PPS interval had a different capture_cnt value. |
| Edge | 1 | W | When ‘0’ then clock in PPS with the rising edge of the system clock, else use the falling edge of the system clock. |
| Capture_cnt | 30 | R | Detected number of system clock cycles between two PPS. |
| Expected_cnt | 31 | W | Expected number of system clock cycles between two PPS. |

Table 2: PPSH MM register fields

The actual width ot the capture_cnt and expected_cnt is ceil_log2(g_st_clk_freq), so sufficient to fit the largest supported system clock frequency value in Hz.

The Software/python/peripherals/pi_ppsh.py Python peripheral script provides methods for accessing the PPSH registers and the util_pphs.py provides usage examples.

# Hardware interface

The mms_ppsh interface parameters and ports are given in respectively Table 3 and Table 4.

| Generic | Type | Description |
| --- | --- | --- |
| g_cross_clock_domain | boolean | Use false when mm_clk and st_clk are the same, else use true to cross the clock domain |
| g_st_clk_freq | natural | System clock st_clk frequency in Hz |

Table 3: mms_ppsh parameters

| Signal | IO | Type | Description |
| --- | --- | --- | --- |
| MM clock domain |  |  |  |
| mm_rst | IN | std_logic | Reset in mm_clk domain |
| mm_clk | IN | std_logic | MM clock |
| reg_mosi | IN | t_mem_mosi | MM interface for ppsh |
| reg_miso | OUT | t_mem_miso |  |
| pin_pps | OUT | slv | PIO support for backwards compatibility with pin_pps on ctrl_unb_common. The pin_pps did not support stable yet and is defined as: toggle & '0' & RESIZE_UVEC(capture_cnt, 30) |
| ST clock domain |  |  |  |
| pps_ext | IN | std_logic | External periodic sync pulse with unknown but constant rising-edge phase to st_clk. The pulse width must be ≥ 1 st_clk cycle. The falling edge is not used for the timing. |
| st_rst | IN | std_logic | Reset in system clock st_clk domain |
| st_clk | IN | std_logic | System clock domain clock |
| pps_sys | OUT | sl | The pps_ext pulse detected in the in st_clk domain The pps_sys pulse width is 1 st_clk cycle. |

Table 4: mms_ppsh IO

# Design

Figure 1 shows the block diagram of the PPSH that is used inside mms_ppsh.

Figure 1: Block diagram of ppsh.vhd

The pps_ext signal is assumed to be locked to the system clock clk and arrives with unknown but stable phase offset to the system clock. The PPSH module can be set to capture the PPS pulse using the rising edge or the falling edge of the system clock via the capture_edge control input. This allows choosing the maximum time offset (i.e. eye-opening) between the rising edge of the PPS and the edge of the clock.

The common_interval_monitor in the PPSH module counts the number of system clock pulses that occur between every two external pulses

When the external pulse period and phase relation with the system clock are stable, then the number of clock pulses per period remains fixed. The common_stable_monitor in the PPSH module reports stable PPS for as long as the capture_cnt equals the expected_cnt. And not stable if one or more capture_cnt were not expected. The stable monitor interval restarts after each pps_stable_ack pulse.

# Implementation

The ppsh uses a common_ddio_in component to clock in the pps_ext via a double data rate (DDR) component that is located near the IO pin in the IO Element (IOE). The DDR component also eases the selection of which edge of the clk is used via capture_edge.

The input delay element in IOE shown in Figure 1 is not instantiated in the ppsh VHDL. Typically using either the rising edge or falling edge of the system clock provides sufficient margin for reliable capture of the external PPS, so therefore the input delays are set to 0.  The input delay can be set via an input delay constraint for the PPS pin in the synthesis script. For example  as done in $UNB\designs\unb_common\src\tcl\COMMON_NODE_general_pins.tcl:

set_instance_assignment -name D2_DELAY 0 -to PPS

set_instance_assignment -name D3_DELAY 0 -to PPS

set_instance_assignment -name D1_DELAY 0 -to PPS

# Verification

The tb_ppsh.vhd and tb_mms_ppsh.vhd are self-check test benches that verify the correct behavior of the PPSH module.