UniBoard transceiver module

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Daniel van der Schuur | ASTRON | 10 march 2011 |
| Controle / Checked: |  |  |
| Eric Kooistra | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Andre Gunst | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature Andre Gunst | ASTRON |  |
| © ASTRON 2011 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Andre Gunst Eric Kooistra Jonathan Hargreaves (JIVE) Salvatore Pirruccio (JIVE) | Gijs Schoonderbeek Sjouke Zwier Harro Verkouter (JIVE) |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2011-01-31 | Daniel van der Schuur | Draft |
| 0.2 | 2011-03-03 | Daniel van der Schuur | Added sections on the ST interface |

Table of contents:

1	Introduction	5

1.1	Purpose	5

1.2	Module overview	5

2	Software interface	6

2.1	Operation	6

2.2	MM registers	6

3	Hardware interface	8

3.1	Clock domains	8

3.2	Parameters	9

3.3	MM interface	9

3.4	ST interfaces	10

3.5	PHY interface	10

4	Application	11

4.1	SOPC design	11

4.1.1	Streaming diag modules	12

4.2	Software	12

4.3	Synthesis	12

4.4	Running the design on hardware	12

5	Design	13

6	Implementation	14

6.1	Reset state machine	14

6.2	Alignment of received data	14

6.2.1	Word alignment	15

6.2.2	Byte ordering	16

6.3	Instantiated ALTGX instances	17

7	Verification	18

7.1	Simulation	18

7.2	Target hardware	18

8	Appendix – list of files	19

8.1	tr_nonbonded	19

8.2	unb_tr_nonbonded	19

9	Appendix:  DIAG modules for ST interface	20

9.1	mms_diag_tx_seq_12	20

9.2	mms_diag_rx_seq_12	21

Terminology:

BN		Back Node

BN_BI		Back node – Backplane Interface

CMU		Clock Multiplier Unit

FIFO		First In First Out

FN		Front Node

FN_BN		Front Node – Back Node

FPGA		Field Programmable Gate Array

FSM		Finite State Machine

Nof		Number of

PCS		Physical Coding Sublayer

PHY		Physical layer

PMA		Physical Media Attachment

RX		Receive

SI_FN		Serial Interface – Front Node

SOPC		System On a Programmable Chip (Altera)

TX		Transmit

XGB		10 Gigabit Breakout board

References:

- ‘Design considerations for UniBoard’s Stratix IV Transceivers’, ASTRON-RP-386, Daniel van der Schuur

- ‘Quartus II Handbook’, quartusii_handbook.pdf, www.altera.com

- ‘Altera Stratix IV Device Handbook’, July 2010, , www.altera.com

- ‘UniBoard Board Design’, ASTRON RP-316, Gijs Schoonderbeek, Sjouke Zwier

- www.altera.com, “Avalon Interface Specifications”, mnl_avalon_spec.pdf

- https://svn.astron.nl/UniBoard_FP7/UniBoard/trunk, the UniBoard FP7 SVN repository ($UNB)

- “Specification for module interfaces using VHDL records”, ASTRON-RP-380, Eric Kooistra

- “Common Library Memory and Register Component Descriptions”, ASTRON-RP-415, Eric Kooistra

- “UNB_Common Module Description”, ASTRON-RP-426, Eric Kooistra

- “UniBoard transceiver module: tr_nonbonded”, ASTRON-RP-405, Daniel van der Schuur

# Introduction

## Purpose

The UniBoard uses high speed Stratix IV transceivers to interface between FPGAs using the mesh (front node - back node or FN_BN) interconnect, and to interface to hardware via the back node to backplane interface (BN_BI). Details and background information can be obtained by reading [1] and [4].

This document describes a VHDL transceiver module, mms_tr_nonbonded, that can be used on the UniBoard to implement up to twelve full-speed transceivers in non-bonded mode per FPGA side, providing a theoretical throughput of 81,6 Gbps (12 x 8.5Gbps x [8/10]) per FPGA.

The mms_tr_nonbonded module is a generic design meant to be software controlled, and will be discussed in chapters 1, 2 and 3.

Chapter 4 discusses a wrapper entity, avs_tr_nonbonded, which is used to instantiate the mms_tr_nonbonded module in an SOPC design.

The mms_tr_nonbonded module itself instantiates an entity called tr_nonbonded. This entity contains all the transceiver functionality – mms_tr_nonbonded just adds an MM and ST interface to it. The tr_nonbonded entity is described in more detail in chapters 5 and 6.

## Module overview

The interfaces of the mms_tr_nonbonded module are shown in Figure 1. This module uses the ALTGX (Altera Gigibit Transceiver) IP block (instantiated in entity phy_gx), but this IP could theoretically be replaced by transceiver IP of any vendor. The PHY interface consists of the serial TX and RX connections to and from the FPGA pins. The number of transceiver instances (and therefore the number of transceiver links) is configurable.

Figure 1: mms_tr_nonbonded module interfaces

The 32-bit receiver data can be (under)sampled to verify that data is being received (counter data is easily distinguished). For this purpose, the memory mapped (MM) RX sample registers have been added. Via the MM interface one can also control the diagnostic modules inside the mms_tr_nonbonded module, as it is possible to instantiate the mms_tr_nonbonded module with onboard transmitter data sequencers (tx_seq) and verifiers to monitor the incoming data (rx_mon) at the receiver ends.

# Software interface

## Operation

The MM registers are used to control the diagnostics modules connected to the TX and RX sides of the transceivers. Using these registers, the user can enable or disable the transmission of a test data sequence (pseudo random or counter data) by enabling the diag TX module, and let the diagnostics module on the RX side verify its integrity. In addition, twelve registers are provided that can be used to sample the incoming 32-bit data of each transceiver, which is useful to ascertain that (counter) data is being received.

## MM registers

Available registers of the tr_nonbonded module are listed in Table 1. All registers are 32 bits wide, however only the rx_dataout registers use the full 32 bits. The number of used bits of the remaining registers corresponds directly to the number of instantiated transceivers (bit 0 corresponds to transceiver 0, etc). This implies a maximum of 32 transceivers per tr_nonbonded module.

| Name | Address (words) | Size (words) | Read/ Write | Description |
| --- | --- | --- | --- | --- |
| rx_enable | 0 | 1 | RW | Enables RX diag module. |
| rx_mode | 1 | 1 | RW | Sets mode of RX diag module. |
| tx_enable | 2 | 1 | RW | Enables TX diag module (overrides ST data) |
| tx_mode | 3 | 1 | RW | Sets mode of TX diag module. |
| diag_res | 4 | 1 | R | Diagnostics results of all RX diag modules. |
| diag_val | 5 | 1 | R | Shows whether diagnostics result is valid or not. |
| rx_dataout_0 | 6 | 1 | R | Parallel RX data out of transceiver 0. |
| rx_dataout_1 | 7 | 1 | R | Parallel RX data out of transceiver 1. |
| rx_dataout_2 | 8 | 1 | R | Parallel RX data out of transceiver 2. |
| rx_dataout_3 | 9 | 1 | R | Parallel RX data out of transceiver 3. |
| rx_dataout_4 | 10 | 1 | R | Parallel RX data out of transceiver 4. |
| rx_dataout_5 | 11 | 1 | R | Parallel RX data out of transceiver 5. |
| rx_dataout_6 | 12 | 1 | R | Parallel RX data out of transceiver 6. |
| rx_dataout_7 | 13 | 1 | R | Parallel RX data out of transceiver 7. |
| rx_dataout_8 | 14 | 1 | R | Parallel RX data out of transceiver 8. |
| rx_dataout_9 | 15 | 1 | R | Parallel RX data out of transceiver 9. |
| rx_dataout_10 | 16 | 1 | R | Parallel RX data out of transceiver 10. |
| rx_dataout_11 | 17 | 1 | R | Parallel RX data out of transceiver 11. |

Table 1: tr_nonbonded module registers

The register bits that control the diagnostics settings are listed in Table 2 and Table 3.

| Bits | Field name | Description |
| --- | --- | --- |
| [32..g_nof_gx] | unused | unused |
| [g_nof_gx-1 .. 0] | DIAG_EN | Enables diag module of transceiver number [bit] when ‘1’, disables when ‘0’. Bit 0 corresponds to transceiver 0. The TX diag modules must already be outputting data before one enables the corresponding RX diag modules, otherwise the RX monitor will not receive valid data and the diag result will indicate errors. As long as the TX diag module is enabled, its output data will override any data that is presented on the ST data input. |

Table 2: rx_enable and tx_enable registers

| Bits | Field name | Description |
| --- | --- | --- |
| [32..g_nof_gx] | unused | unused |
| [g_nof_gx-1 .. 0] | DIAG_MODE | Selects diag mode of transceiver number [bit]. ‘1’ sets it to counter mode, ‘0’ sets it to pseudo random mode. Bit 0 corresponds to transceiver 0. Naturally, the TX diag mode must be the same as the diag mode setting on the RX side to get an OK diag result. Nonetheless, it is possible to force errors by setting different diag modes on the TX and RX side. |

Table 3: rx_mode and tx_mode register bits

To read the diagnostics results at the receiver ends, the register bits in Table 4 and Table 5 are used.

| Bits | Field name | Description |
| --- | --- | --- |
| [32..g_nof_gx] | unused | unused |
| [g_nof_gx-1 .. 0] | DIAG_RES | Diag result for each individual diag RX module. ‘0’ indicates a correct diag result, ‘1’ indicates errors in the received data stream. Bit 0 corresponds to transceiver 0. As long as no bit errors occur, diag_res remains ‘0’. If one or more bit errors occur, diag_res becomes and remains ‘1’. A new verification interval can be started by disabling and re-enabling the diag_en bit(s) in the rx_enable register of the corresponding transceiver(s). |

Table 4: diag_res register bits

| Bits | Field name | Description |
| --- | --- | --- |
| [32..g_nof_gx] | unused | unused |
| [g_nof_gx-1 .. 0] | DIAG_VAL | Diag valid indicator bits for each individual diag RX module. A ‘1’ indicates that the corresponding diag result is valid, else ‘0’. Bit 0 corresponds to transceiver 0. The diag_val bit remains ‘0’ until two clock cycles after the RX module is enabled. This is because the module needs two subsequent counter or PRSG values: the first received value is the seed from which the next (expected) received value is calculated. This calculated value is compared to the data word that is received after the seed value. As long as the calculated values match the received values, diag_val remains ‘1’ indicating a valid diag_res. |

Table 5: diag_val register bits

# Hardware interface

## Clock domains

Figure 2 shows an overview of the mms_tr_nonbonded module, in which one can distinguish four user provided clocks and two derived clocks. These clocks and their descriptions are listed below in Table 6.

Figure 2: mms_tr_nonbonded clock domains

| Name | Frequency (MHz) | Description |
| --- | --- | --- |
| tr_clk | 156.25 | Used to generate transmitter clock, also provides receiver reference clock. |
| tx_clk | 156.25 | Transmitter clock, one for each instantiated transceiver, derived from tr_clk. Used to clock in parallel transmitter data. There might be a phase difference between the individual tx_clk signals and /or the tr_clk, depending on settings of the Altera IP [3]. |
| rx_clk | 156.25 | Receiver clock, one for each instantiated transceiver, derived from the clock that is recovered from the received serial stream. Used to clock out parallel receiver data. |
| mm_clk | 125 | MM clock for the memory-mapped bus shared with e.g. a NIOS II processor |
| st_clk | 200 | Clock for the streaming interfaces. |
| cal_reconf_clk | 40 | Used to clock the ALTGX calibration block and the ALTGX_RECONFIG transceiver reconfiguration block. The frequency must lie between 37.5 and 50MHz [3]. |

Table 6: avs_tr_nonbonded external and internal clocks

## Parameters

Available parameters when instantiating the tr_nonbonded or mms_tr_nonbonded module are listed in Table 7. In the avs_tr_nonbonded wrapper most parameters are fixed and therefore not available for the user to modify. If this is the case it is mentioned in the description.

| Generic | Type | Description |
| --- | --- | --- |
| g_nof_gx | NATURAL | The number of instantiated transceivers. Limited to 32 as result of control register width, also limited by the FPGA device used. Defaults to 12, the maximum number of full-speed transceivers on one side of a UniBoard FPGA. |
| g_mbps | NATURAL | Transceiver data rate in Mbps. Supported: 3125, 5000, 6250 |
| g_diag | BOOLEAN | When TRUE, diagnostics modules are instantiated within the tr_nonbonded module – one for each transmitter, one for each receiver. Fixed TRUE in avs_tr_nonbonded. |
| g_tx | BOOLEAN | Transmitters are instantiated when TRUE. The number of instances equals g_nof_gx. Fixed TRUE in avs_tr_nonbonded. |
| g_rx | BOOLEAN | Receivers are instantiated when TRUE. The number of instances equals g_nof_gx. Fixed TRUE in avs_tr_nonbonded. |
| g_fifos | BOOLEAN | Instantiate FIFOs between user clock domain (or ST clock domain) and TX/RX clock domains. Fixed FALSE in avs_tr_nonbonded. |
| g_fifo_depth | NATURAL | Default: 16 words. |
| g_fifo_tx_almost_full_nof_words | NATURAL | Number of words at which the FIFO is considered to be almost full. When almost full, a signal is asserted to provide flow control. Fixed (10 words) in tr_nonbonded.vhd. |
| g_ser_loopback | BOOLEAN | When true, the ALTGX instances will have their serial TX looped back to their own RX. FALSE by default. |

Table 7: tr_nonbonded parameters

These generics are meant to be passed from the top-level entity, which in most cases is an SOPC system containing an avs_tr_nonbonded instance. However, these generics are maintained downward the hierarchy, with tr_nonbonded.vhd being the lowest level file that still accepts all of these parameters.

## MM interface

Table 8 lists the defined interface signals for the Memory Mapped registers.

| Signal | Type | Description |
| --- | --- | --- |
| rddata[31..0] | MISO | Read data word |
| address[6..0] | MOSI | Byte address range for the 18 words listed in table 1. |
| wrdata[31..0] | MOSI | Write data word |
| wr | MOSI | Write enable |
| rd | MOSI | Read enable |

Table 8: MM interface signals

## ST interfaces

The streaming interfaces are the interfaces on which the user data is fed into the transceiver module for transmission. For the user data to be actually passed on to the transmitters, the internal diag modules on the transmitter sides must be disabled – the internally generated diagnostics data overrides any user data present on the transmitter ST interfaces. Naturally, this does not apply to the receiver ends. Any received data is forwarded onto the receiver ST interfaces.

The mms_tr_nonbonded module (Figure 3) provides a streaming interface to each transmitter, and from each receiver. In the receiver ST interfaces the ready signal is not connected, as one cannot stop data from being received by the receivers.

Figure 3: mms_tr_nonbonded ST interfaces per transceiver instance

The transmitter ST interfaces however do provide a ready signal for flow control from the sink (transmitter) to the (user data) source. The whole is summarized in the tables below.

| Signal | Type | Description |
| --- | --- | --- |
| ready | SISO | Used for flow control |
| data[31..0] | SOSI | Data word |
| valid | SOSI | Data valid signal |

Table 9: ST interfaces: Transmitters

| Signal | Type | Description |
| --- | --- | --- |
| data[31..0] | SOSI | Data word |
| valid | SOSI | Data valid signal |

Table 10: ST interfaces: Receivers

## PHY interface

By default, the mms_tr_nonbonded module has 12 serial TX and RX connections that must be connected to the FPGA SERDES pins.

# Application

## SOPC design

An AVS wrapper that instantiates the mms_tr_nonbonded module enables the use of the mms_tr_nonbonded module in an SOPC design. An example of this is shown in Figure 4, displaying an SOPC system used in design unb_tr_nonbonded.

Figure 4: SOPC system unb_tr_nonbonded, containing an avs_tr_nonbonded instance

The unb_tr_nonbonded design can be run on one or more nodes on the UniBoard. The design uses an SOPC system in which an instance of avs_tr_nonbonded is used. This avs_tr_nonbonded instance provides 12 transceiver links, resulting in 12 streaming transmitter interfaces and 12 streaming receiver interfaces that can be connected in the SOPC Builder GUI. If one would like to use an avs_tr_nonbonded instance on its own (using the internal diag modules), without connecting other components to its ST interfaces, one has to connect the receiver ST sources to the transmitter ST sinks as demonstrated in Figure 5. The AVS wrapper provides a dummy ready signal (always ‘1’) for the receiver ST interface, solely for compatibility with other ST interfaces – including its own transmitter ST interface.

Figure 5: avs_tr_nonbonded with unused ST interfaces

### Streaming diag modules

In the unb_tr_nonbonded design, the 12 transmitter ST interfaces of avs_tr_nonbonded connect to an avs_diag_tx_seq_12 instance, which is an Avalon-wrapped bundle of twelve diag TX modules – the same as the ones that exist inside the avs_tr_nonbonded module itself for diagnostics purposes, only these 12 diag TX modules have a streaming interface and are used to mimic user data. Similarly, the 12 receiver ST interfaces originating from avs_tr_nonbonded connect to an avs_diag_rx_seq_12 instance that verifies the ‘user data’ coming in on its 12 ST interfaces. As mentioned, the TX and RX diag modules these avs_diag wrappers instantiate are identical to the ones used in the mms_tr_nonbonded module, and therefore the avs_diag_tx_12 and avs_diag_rx_12 components provide MM registers with mostly the same functionality as the ones in mms_tr_nonbonded. See Appendix 9 for details.

## Software

As the mms_tr_nonbonded module is meant to be connected to a memory mapped bus, its registers can be controlled with simple IORD and IOWR statements. A basic example of software that issues a series of these commands to configure and enable the diagnostics mode of the module can be found in Appendix 8.2.

## Synthesis

To synthesize the unb_tr_nonbonded design using Quartus II, one needs to open the QPF file and perform a full compile. It is recommended to have compiled the software application in advance, because Quartus II will integrate the software application’s HEX-file into the programming file during the assembly stage.

## Running the design on hardware

To test the transceiver links of e.g. a front node, one has to download this design onto that front node and all back nodes. Each back node will generate data and transmit this data sequence onto its 12 transceiver lines, of which 3 connect to the active front node. The front node will receive 12 (3 streams times 4 back nodes) data streams and verifies their integrity. This setup verifies the 12 receivers of the front node, but only 3 transmitters from each back node. To verify 12 receivers and 12 transmitters per node, the unb_tr_nonbonded design must be downloaded onto all eight nodes of the UniBoard.

# Design

Figure 6 shows the main components used in the tr_nonbonded module, and offers a more elaborate view of the different clock domains that exist. To facilitate using the module in a design, these clock domains are abbreviated in the VHDL signal prefixes. This not only avoids unintended clock domain crossings, but is also necessary because of the optional FIFOs: when instantiating tr_nonbonded with FIFOs, the parallel input and output data is in the streaming clock domain and the rxc and txc prefixed signals are not used. Without FIFOs, one must discriminate between data in the transmitter clock (txc) domain and data in the receiver clock (rxc) domain, leaving the ssc prefixed signals unused. This is the case in mms_tr_nonbonded and avs_tr_nonbonded.

Figure 6: Main components of tr_nonbonded

# Implementation

This chapter gives an overview of the VHDL architecture and hierarchy of the tr_nonbonded module and its components.

Figure 7: Hierarchy of tr_nonbonded

As illustrated in Figure 7, the mentioned phy_tx and phy_rx of Figure 6 are actually contained within the ALTGX IP block by Altera. This IP block is instantiated in entity phy_gx, along with the ALTGX_RECONF IP block – a reconfiguration block that can be used to reconfigure certain properties of the ALTGX IP during operation. Within tr_nonbonded, this feature is not used. It is however mandatory to connect the reconfiguration block to the ALTGX.

## Reset state machine

The reset block contains an FSM that reads transceiver status signal originating from the ALTGX IP and controls various reset signals for the digital and analog parts of the transceivers. After performing initialization of the transceivers, it waits for a relatively long period (approx. 1 second) before initiating alignment. The reason for this long delay is that the transmitters of which data is received must already be sending the alignment patterns. These transmitters could be on the same board, but they could also be on another board connected via a backplane or cables.

## Alignment of received data

Upon serialization of parallel data, the word boundary is lost. To restore this word boundary, an alignment block is used on the receiver side. Besides that, depending on when the system comes out of reset, the byte order may not match the original byte order. This problem is addressed by the byte ordering block, available as part of the Altera ALTGX IP block. This paragraph describes the alignment procedure and these two necessary blocks.

Figure 8: Transmitter data path within ALTGX

To align the data on the receiver side, the transmitter must send a specific alignment pattern. This is performed by the tx_align entity. This entity is responsible for the selection of two types of data that can be fed into the transmitter:

- Data from ST interface or TX diag output

- The alignment pattern

During initialization after power up, the tx_align entity continuously outputs the alignment pattern to the parallel input of the transmitters. The enable input of tx_align and the duration of the assertion of this signal are controlled by the reset state machine. When enabled, the input user data is not forwarded to the transmitter. Instead, the alignment pattern is sent. This pattern (c_alignment_pattern) is set in the tr_nonbonded.pkg file. The actual alignment to this pattern on the receiver side is performed by the Word Alignment Block, part of the Altera ALTGX IP block. The same pattern is also used by the byte ordering block to restore the byte order. Word alignment and byte ordering blocks are available as options in the ALTGX Megafunction.

### Word alignment

When a 32-bit width is used for the fabric-transceiver interface, as illustrated in Figure 8, the 32-bit data is byte serialized into half the width at twice the frequency. The resulting 16 bits are fed into the 8b/10b encoder that encodes the MSB and LSB into 10-bits each and outputs these 20 bits to the serializer. On the receiver side, the word aligner is located before the 8b/10b decoder and thus looks for word alignment patterns that are 8b/10b encoded. This word alignment pattern, 10 (single width) or 20 (double width) bits wide, is entered as a parameter in the corresponding ALTGX Megafunction field (see Figure 9).

Figure 9: Word alignment

The 20-bit default alignment pattern for the tr_nonbonded module is:

0011110100 0011111010 = 8b/10b encoded, LS Bits right

K28.0            K.28.5          = Comma symbol followed by control word

0xBC             0x1C            = 8b/10b decoded

This corresponds to 0xBC1C after 8b/10b decoding at the receiver end and before 8b/10b encoding at the transmitter end. The comma symbol (K.28.5 or 0xBC) differs from the second byte so their places cannot be swapped during alignment. The second byte (0x1C) is also used in the byte ordering block described in the next paragraph.

The alignment pattern is input in the Megafunction in reverse, the way it is sent after serialization:

0101111100 0010111100 (0x5F0BC) = 8b/10b encoded, LS Bits left (reversed)

### Byte ordering

As described in the previous paragraph, the 0x1C byte is used for byte ordering. The byte ordering block (available as option in the ALTGX Megafunction, see Figure 10) uses the synchronization status output of the word aligner to start the byte ordering – this is necessary because the byte deserializer at the receiver could output the bytes in the wrong order. When it finds byte ordering pattern 0x1C at the LSB position, it considers the data to be aligned. Otherwise, it inserts a padding pattern (set to 0x00) to effectively shift the data onto the LSB position.

Figure 10: Byte ordering options

## Instantiated ALTGX instances

The user does not need to use the Quartus MegaWizard to generate the IP files (ALTGX, ALTGX_RECONFIG). Generated ALTGX VHD files, including the necessary byte ordering settings etcetera, are available on SVN [6], also see Appendix 8.1.

# Verification

Verification of the tr_nonbonded, mms_tr_nonbonded or avs_tr_nonbonded is done by configuring and enabling the built-in diagnostics modules and reading the diagnostics result on the RX side. The result can be read directly in a ModelSim wave window, and/or can be read out via the MM bus in case of mms_tr_nonbonded. In either case it is recommended to set the diag mode to counter to ease debugging if necessary.

Alternatively, one can decide to provide user data via the ST interface and verify it using the built-in RX monitors or a verification module connected to the ST interface(s) of the receivers.

## Simulation

A test bench tb_mms_tr_nonbonded is used to configure, enable and read out the TX and RX diagnostics modules of mms_tr_nonbonded via its MM interface. To do so, it uses the proc_mem_mm_bus_wr and proc_mem_mm_bus_rd functions from tb_common_mem(pkg). In tb_mms_tr_nonbonded, an mms_tr_nonbonded instance is used with its serial TX outputs looped back to its own serial RX inputs. After about 30 us, the test bench should report whether the read back diagnostics result was zero (test passed) or not by printing a message in the transcript window of ModelSim. In addition, the test bench instantiates an mms_diag_tx_seq and an mms_diag_rx_seq module. These diag modules are used to mimic user data, and are used to verify the streaming data path. Like the testing procedure that controls the internal diag modules, also these ST diag modules are controlled and read out using their MM buses.

## Target hardware

Testing on the target hardware (UniBoard) works in the same manner. However, the registers must now be written to and read from by a NIOS II processor. This can be done with the fn_base and bn_base designs, but for faster compile times and better overview an additional design unb_tr_nonbonded has been created. The SOPC in this design only contains the most basic components and can be verified with its software unb_tr_nonbonded.c – which uses simple IORD and IOWR functions. See appendix 8.2.

# Appendix – list of files

## tr_nonbonded

The tr_nonbonded directory contains the source code for tr_nonbonded, mms_tr_nonbonded, avs_tr_nonbonded and the test bench tb_mms_tr_nonbonded. It is located at:

$UNB/Firmware/modules/tr_nonbonded/src/vhdl

The pre-generated ALTGX and ALTGX_RECONFIG files are available from:

$UNB/Firmware/modules/tr_nonbonded/build/synth/quartus

## unb_tr_nonbonded

A stripped-down design meant for fast testing of the avs_tr_nonbonded module with the NIOS II. The files for synthesis and simulation are in the following directory:

$UNB/Firmware/designs/unb_tr_nonbonded

In addition, a C main program is located at:

$UNB/Firmware/software/apps/unb_tr_nonbonded

# Appendix:  DIAG modules for ST interface

To verify streaming data, MMS wrappers were created around the existing LOFAR diagnostics modules available from the diag library. These wrappers instantiate a number g_nof_diag_inst of diag instances, and offer a streaming data interface for each instance, plus one memory mapped interface for all instances for control and reading back status and results.

## mms_diag_tx_seq_12

Available registers of the mms_diag_tx_seq_12 module are listed in Table 11. All registers are 32 bits wide. The number of used bits in the registers corresponds directly to the number of instantiated diag modules (bit 0 corresponds to diag module 0, etc).

| Name | Address (words) | Size (words) | Read/ Write | Description |
| --- | --- | --- | --- | --- |
| diag_en | 0 | 1 | RW | Enables TX diag module. |
| diag_mode | 1 | 1 | RW | Sets mode of TX diag module. |

Table 11: mms_diag_tx_seq_12 registers

The register bits that control the diagnostics settings are listed in Table 12 and Table 13.

| Bits | Field name | Description |
| --- | --- | --- |
| [32..g_nof_diag_inst] | unused | unused |
| [g_nof_diag_inst-1 .. 0] | DIAG_EN | Enables diag module of transceiver number [bit] when ‘1’, disables when ‘0’. Bit 0 corresponds to transceiver 0. The TX diag modules must already be outputting data before one enables the corresponding RX diag modules, otherwise the RX monitor will not receive valid data and the diag result will indicate errors. As long as the TX diag module is enabled, its output data will override any data that is presented on the ST data input. |

Table 12: diag_enable registers

| Bits | Field name | Description |
| --- | --- | --- |
| [32..g_nof_diag_inst] | unused | unused |
| [g_nof_diag_inst-1 .. 0] | DIAG_MODE | Selects diag mode of diag instance [bit]. ‘1’ sets it to counter mode, ‘0’ sets it to pseudo random mode. Bit 0 corresponds to diag instance 0. Naturally, the TX diag mode must be the same as the diag mode setting on the RX side to get an OK diag result. Nonetheless, it is possible to force errors by setting different diag modes on the TX and RX side. |

Table 13: diag_mode register bits

## mms_diag_rx_seq_12

| Name | Address (words) | Size (words) | Read/ Write | Description |
| --- | --- | --- | --- | --- |
| diag_enable | 0 | 1 | RW | Enables RX diag module. |
| diag_mode | 1 | 1 | RW | Sets mode of RX diag module. |
| diag_res | 2 | 1 | R | Diagnostics results of all RX diag modules. |
| diag_val | 3 | 1 | R | Shows whether diagnostics result is valid or not. |
| rx_dataout_0 | 4 | 1 | R | Parallel RX data out of diag instance 0. |
| rx_dataout_1 | 5 | 1 | R | Parallel RX data out of diag instance 1. |
| rx_dataout_2 | 6 | 1 | R | Parallel RX data out of diag instance 2. |
| rx_dataout_3 | 7 | 1 | R | Parallel RX data out of diag instance 3. |
| rx_dataout_4 | 8 | 1 | R | Parallel RX data out of diag instance 4. |
| rx_dataout_5 | 9 | 1 | R | Parallel RX data out of diag instance 5. |
| rx_dataout_6 | 10 | 1 | R | Parallel RX data out of diag instance 6. |
| rx_dataout_7 | 11 | 1 | R | Parallel RX data out of diag instance 7. |
| rx_dataout_8 | 12 | 1 | R | Parallel RX data out of diag instance 8. |
| rx_dataout_9 | 13 | 1 | R | Parallel RX data out of diag instance 9. |
| rx_dataout_10 | 14 | 1 | R | Parallel RX data out of diag instance 10. |
| rx_dataout_11 | 15 | 1 | R | Parallel RX data out of diag instance 11. |

Table 14: mms_diag_tx_seq_12 registers

The register bits that control the diagnostics settings are listed in Table 15 and Table 16.

| Bits | Field name | Description |
| --- | --- | --- |
| [32..g_nof_diag_inst] | unused | unused |
| [g_nof_diag_inst-1 .. 0] | DIAG_EN | Enables diag module of diag instance [bit] when ‘1’, disables when ‘0’. Bit 0 corresponds to diag instance 0. |

Table 15: diag_enable register

| Bits | Field name | Description |
| --- | --- | --- |
| [32.. g_nof_diag_inst] | unused | unused |
| [g_nof_diag_inst-1 .. 0] | DIAG_MODE | Selects diag mode of diag instance [bit]. ‘1’ sets it to counter mode, ‘0’ sets it to pseudo random mode. Bit 0 corresponds to diag instance 0. Naturally, the TX diag mode must be the same as the diag mode setting on the RX side to get an OK diag result. Nonetheless, it is possible to force errors by setting different diag modes on the TX and RX side. |

Table 16: diag_mode register bits

To read the diagnostics results, the register bits in Table 17 and Table 18 are used.

| Bits | Field name | Description |
| --- | --- | --- |
| [32.. g_nof_diag_inst] | unused | unused |
| [g_nof_diag_inst -1 .. 0] | DIAG_RES | Diag result for each individual diag RX module. ‘0’ indicates a correct diag result, ‘1’ indicates errors in the received data stream. Bit 0 corresponds to diag instance 0. As long as no bit errors occur, diag_res remains ‘0’. If one or more bit errors occur, diag_res becomes and remains ‘1’. A new verification interval can be started by disabling and re-enabling the diag_en bit(s) in the tx_enable and rx_enable registers of the corresponding diag instance. |

Table 17: diag_res register bits

| Bits | Field name | Description |
| --- | --- | --- |
| [32.. g_nof_diag_inst] | unused | unused |
| [g_nof_diag_inst -1 .. 0] | DIAG_VAL | Diag valid indicator bits for each individual diag RX module. A ‘1’ indicates that the corresponding diag result is valid, else ‘0’. Bit 0 corresponds to diag instance 0. The diag_val bit remains ‘0’ until two clock cycles after the RX module is enabled. This is because the module needs two subsequent counter or PRSG values: the first received value is the seed from which the next (expected) received value is calculated. This calculated value is compared to the data word that is received after the seed value. As long as the calculated values match the received values, diag_val remains ‘1’ indicating a valid diag_res. |

Table 18: diag_val register bits