I2C SMBus Module Description

| Verified: |  |  |  |
| --- | --- | --- | --- |
| Name | Signature | Date | Rev.nr. |
| Andre Gunst |  |  |  |

| Accepted: |  |  |
| --- | --- | --- |
| Work Package Manager | System Engineering Manager | Program Manager |
| Andre Gunst |  |  |

© ASTRON 2009 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner.

Distribution list:

| Group: UniBoard | For Information: |
| --- | --- |
| ASTRON |  |

Document history:

| Revision | Date | Author | Modification |
| --- | --- | --- | --- |
| 0.1 | 2009-09-04 | Kooistra | Creation from LOFAR RSP design documents |
| 0.2 | 2009-09-04 | Kooistra | Added I2C slave module from LOFAR RCU Minor edits. |

Table of contents:

1	Introduction	4

2	I2C SMBus master module design	5

3	I2C SMBus master module interface	7

4	I2C slave module	10

5	Appendix	11

5.1	Firmware source code	11

5.2	I2C access example	12

5.3	Known issues	13

5.3.1	Clk rate and bit rate	13

5.3.2	Bus arbitration	13

5.3.3	Ringing on SCL and SDA edges	13

5.3.4	Comma logic	13

5.3.5	Sometimes access to LOFAR TDS fails	13

References:

- “RSP Firmware Design Description””, LOFAR-ASTRON-SDD-018, Rev. 3.1, Oct 2007

- “RSP-MAC Interface Description”, LOFAR-ASTRON-ICD-002, Rev. 6.2, Sep 2008

- http://www.opencores.org, “I2C-Master Core Specification”, Richard Herveille, rev 0.9, Jul 2003

- http://www.freemodelfoundry.com, HDL open source code

- http://www.smbus.org, System Management Bus Specification, Rev. 2.0

- http://www.nxp.com, Philips I2C standard, I2C slave IO expander chips

- http://www.maxim-ic.com, I2C slave sensors chips

- http://www.microchip.com, microcontrollers with I2C interface

Terminology:

ADDR		Address

CMD		Command

CNT		Count

CTRL		Control

HDL		Hardware Description Language (typically VHDL or Verilog)

I2C		Inter-IC	Interface

IC		Integrated Circuit

ID		Identifier

I/O		Input/Output

NOP		No Operation

OP		Operand

SCL		I2C Clock line

SDA		I2C Data line

SENS		Sensors module

SMBUS	System Management Bus

T,V		Temperature, Voltage sensor

VHDL		Very High Speed IC HDL

# Introduction

The I2C SMBus module was originally designed for the LOFAR Remote Processing Board (RSP) firmware, and has also been reused for the EMBRACE Center Board (ECB) firmware. In LOFAR the I2C is used to access various devices:

- IO expanders [6] on the Timing Distribution clock board (TDS)

- Temperature and voltage sensors [7] on RSP, TDS and the Subrack Power Unit (SPU)

- The LOFAR Receiver Unit (RCU) with a proprietory I2C slave implementation in VHDL

- PIC microcontroller [8] for the LOFAR High Band Antenna (HBA) client modem on the RCU

For the RadioNet FP7 UniBoard development the I2C master SMBus module and RCU I2C slave are made available too and described in this design document. This document combines the I2C information described in the LOFAR RSP design and interface documents [1], [2].

The I2C SMBus master module consists of:

- I2C master core obtained from Open Cores [3] that drives the I2C bus [6]

- SMBus protocol processor that follows the protocols defined in [5]

- Memory access control interface

The RCU I2C slave consists of one block.

Furthermore there are several behavioural models of I2C slave devices and an example test bench.

# I2C SMBus master module design

The I2C controller consists of an I2C master core [3] that provides a byte level interface to the physical I2C interface and an SMBus [5] protocol processor that allows access based on a set of opcodes. A sequence of opcodes is called a protocol and defines for example an I2C write byte access. The SMBus protocol processor can run a protocol and report its result. The control block provides a memory access interface to the protocol list and protocol result registers. The blocks are shown in Figure 1 and the VHDL source files that correspond to these blocks in are marked in bold in appendix 5.1.

I2C

master core

SMBUS

CTRL

Figure 1: I2C module block diagram

Opcodes

An I2C access is defined by a sequence of basic operations called opcodes. Table 1 lists the I2C access opcodes that are sufficient to define any standard I2C accesses.

| OPCODE | Description |
| --- | --- |
| OP_LD_ADR | Load address register |
| OP_LD_CNT | Load count register |
| OP_WR_CNT | I2C write count register |
| OP_WR_ADR_WR | I2C start and write address for write |
| OP_WR_ADR_RD | I2C start and write address for read |
| OP_WR_DAT | I2C write byte of data |
| OP_WR_BLOCK | I2C write block of count data bytes |
| OP_RD_ACK | I2C read byte data and acknowledge |
| OP_RD_NACK | I2C read byte data and do not acknowledge |
| OP_RD_BLOCK | I2C read block of count data bytes |
| OP_STOP | I2C stop |

Table 1: Opcodes for an I2C access

The opcodes work like instructions for a dedicated microprocessor. Given such a microprocessor it is possible to add some more opcodes for other functions. This is useful to control the flow of I2C accesses. Table 2 lists the control opcodes.

| OPCODE | Description |
| --- | --- |
| OP_IDLE | Idle |
| OP_END | Signal end of list of protocols |
| OP_LD_TIMEOUT | Load timeout value |
| OP_WAIT | Wait for timeout time units |

Table 2: Opcodes for flow control

Protocols

The network layer of the SMBus standard [5] specifies the protocols for several common I2C accesses. For example the write quick protocol can be implemented by applying opcodes: OP_LD_ADR, OP_WR_ADR_WR, OP_STOP. In addition some custom protocols are supported. Each protocol returns a status and the read data (in case of an I2C read access). For the complete list of protocols that is supported by the SMBus protocol processor see section 3.

Appendix 5.1 lists the VHDL source code for the I2C module and some more useful files for test benches and I2C slave models.

# I2C SMBus master module interface

The protocol list registers contain the protocols that need to be executed by the SMBus protocol processor in order to do the appropriate I2C accesses on an I2C bus. The I2C access results can be read back from the protocol result register.

| BIT O/S | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | PROTOCOL_0[7:0] |  |  |  |  |  |  |  |
| 1 | PROTOCOL_1[7:0] |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |

#### Table 3: Protocol list register

| BIT O/S | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | RESULT_0[7:0] |  |  |  |  |  |  |  |
| 1 | RESULT _1[7:0] |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |

Table 4: Protocol results register

Protocol request message

To execute a protocol the dedicated protocol processor needs to get a request message containing the protocol ID followed by the appropriate parameters, see Table 5.

| Protocol name | Message fields (bytes) |  |
| --- | --- | --- |
|  | ID | Parameters |
| PROTOCOL_WRITE_QUICK | 02 | ADDR |
| PROTOCOL_READ_QUICK | 03 | ADDR |
| PROTOCOL_SEND_BYTE | 04 | ADDR, DATA |
| PROTOCOL_RECEIVE_BYTE | 05 | ADDR |
| PROTOCOL_WRITE_BYTE | 06 | ADDR, CMD, DATA |
| PROTOCOL_READ_BYTE | 07 | ADDR, CMD |
| PROTOCOL_WRITE_WORD | 08 | ADDR, CMD, DATA, DATA |
| PROTOCOL_READ_WORD | 09 | ADDR, CMD |
| PROTOCOL_WRITE_BLOCK | 0A | ADDR, CMD, CNT, DATA[1..CNT] |
| PROTOCOL_READ_BLOCK | 0B | ADDR, CMD, CNT |
| PROTOCOL_PROCESS_CALL | 0C | ADDR, CMD, DATA, DATA, ADDR, CMD |
| PROTOCOL_C_WRITE_BLOCK_NO_CNT | 0D | ADDR, CMD, CNT, DATA[1..CNT] |
| PROTOCOL_C_READ_BLOCK_NO_CNT | 0E | ADDR, CMD, CNT |
| PROTOCOL_C_SEND_BLOCK | 0F | ADDR, CNT, DATA[1..CNT] |
| PROTOCOL_C_RECEIVE_BLOCK | 10 | ADDR, CNT |
| PROTOCOL_C_NOP | 11 | - |
| PROTOCOL_C_WAIT | 12 | TIMEOUT[0..3] |
| PROTOCOL_C_END | 13 | - |

Table 5: Protocol request message format.

Where:

- ID		= Protocol ID byte.

- ADDR		= Slave I2C address byte.

- CMD		= Command byte.

- CNT		= Count of number of data bytes to write or to read.

- DATA		= Data byte.

- TIMEOUT	= Timeout value, 4 bytes, LSByte first. Only values < 228 are supported.

Protocols 0x02 to 0x0C follow the SMBus standard [5]. The SMBus protocols are not sufficient to access all kinds of I2C slaves. Furthermore it is useful to define some protocols for protocol flow control. Therefore some custom protocols are added. For example to access the LOFAR RCU the protocols PROTOCOL_C_SEND_BLOCK and PROTOCOL_C_RECEIVE_BLOCK need to be used.

Brief description of the protocols: The protocols for flow control do not access the I2C interface, the others do. The SEND/RECEIVE protocols define a plain data access while the WRITE/READ protocols first write a command byte and then perform the data access. For the READ protocols first having to write the command byte causes that the required I2C restart is performed to subsequently receive the actual data. The WRITE/READ_BLOCK_NO_CNT protocols are similar to the SMBus WRITE/READ_BLOCK protocols except that the count value is not written to the I2C interface. For CNT=1 the WRITE/READ_BLOCK_NO_CNT is equivalent to SMBus WRITE/READ_BYTE and for CNT=2 the WRITE/READ_BLOCK_NO_CNT is equivalent to SMBus WRITE/READ_WORD. The SEND/RECEIVE_BLOCK protocols are similar to the SMBus SEND/RECEIVE_BYTE/WORD protocols, but allow an arbitrary number of bytes given by count. The SEND/RECEIVE_BLOCK protocols are also similar to the WRITE/READ_BLOCK_NO_CNT protocols, except that they do not write a command. The PROTOCOL_C_WAIT protocol allows postponing subsequent protocol handling. The timeout value is given by the 4 timeout bytes (LSByte first) and counts system clock cycles. An unknown protocol is detected and executed as PROTOCOL_C_END.

Protocol response message

The protocol processor returns a protocol result byte for each protocol. In case the protocol concerned an I2C read access, then the result byte is preceded by the read data bytes. The number of read data bytes depends on the protocol type. If the read access failed somehow, then the read data bytes are undefined.

The result byte == 0 when the protocol executed OK and <> 0 otherwise. The result byte defaults to OK except for an I2C write. For an I2C write the result becomes not OK in case a written byte was not acknowledged by the slave, this may occur for the write of the slave address or of the data.

Sequence of protocol messages

The protocol processor can play one or more protocols from a list of request messages. The corresponding response messages are returned in a response list. The PROTOCOL_C_END protocol is useful to signal that the end of a list was reached, so that all protocols in the list have been executed.

Note that by means of the PROTOCOL_C_WAIT timer it is possible to delay (parts of) the I2C access execution of the protocol list. This can be useful to fine tune the synchronization between access to the TTD on ECB and the BF chips on the HEX boards. The time unit of the PROTOCOL_C_WAIT timer is one period of the system clock.

Appendix 5.1 gives an example on how to access an I2C slave using the module interface.

# I2C slave module

The I2C slave module is used on the LOFAR RCU and consists of one block. The corresponding VHDL files are marked in bold in appendix 5.1. Although designed for accessing a 3 byte register on the RCU this I2C slave module can easily be adapted to other applications.

The RCU I2C slave can be run directly using the SCL clock or by means of a higher rate clock that samples the SCL and SDA line. On the LOFAR RCU the I2C slave reset input is connected to one of the control register pins, hence an I2C access setting this pin will reset the slave.

# Appendix

## Firmware source code

List of VHDL and project files:

i2c/build/sim/modelsim/i2c.mpf			Modelsim project file to compile i2c_lib

i2c/build/synth/quartus				Quartus II project directory

i2c/doc/I2C_SMBus_module.doc		This document

i2c/doc/I2C_specs.doc				[3]

i2c/doc/i2c_specs.pdf				[3]

i2c/src/vhdl/i2c_bit.vhd			[3], master I2C core

i2c/src/vhdl/i2c_byte.vhd			[3], master I2C core

i2c/src/vhdl/i2c_list_ctrl.vhd			Register interface control for the I2C SMBus module

i2c/src/vhdl/i2c_list_ctrl(rtl).vhd

i2c/src/vhdl/i2c_smbus(pkg).vhd

i2c/src/vhdl/i2c_smbus.vhd			The I2C SMBus module

i2c/src/vhdl/i2c_smbus(rtl).vhd

i2c/src/vhdl/i2c_slave.vhd			The I2C as implemented on the LOFAR RCU

i2c/src/vhdl/i2c_slave(rtl).vhd

i2c/tb/vhdl/dev_pca9555.vhd

i2c/tb/vhdl/dev_pca9555(beh).vhd		Behavioral model of PCA9555 [6] IO expander I2C slave

i2c/src/vhdl/dev_max1617(pkg).vhd

i2c /tb/vhdl/dev_max1618.vhd

i2c /tb/vhdl/dev_max1618(beh).vhd		Behavioral model of MAX1618 [7] T-sensor I2C slave

i2c /tb/vhdl/dev_max6652(pkg).vhd

i2c /tb/vhdl/dev_max6652.vhd

i2c /tb/vhdl/dev_max6652(beh).vhd		Behavioral model of MAX6652 [7] T,V-sensor I2C slave

i2c/tb/vhdl/i2c_slv_device.vhd

i2c/tb/vhdl/i2c_slv_device(beh).vhd		RTL model of I2C slave based on LOFAR RCU I2C slave

i2c/tb/vhdl/tb_i2cslave.vhd			Test bench for LOFAR RCU I2C slave

fmf/build/sim/modelsim/fmf.mpf			Modelsim project file to compile fmf_lib

fmf/src/vhdl/i2c.vhd				[4], models an I2C master or an I2C slave

fmf/src/vhdl/i2c_drive_ms_pkg.vhd

sens/build/sim/modelsim/sens.mpf		Modelsim project file to compile sens_lib

sens/build/synth/quartus			Quartus II project directory

sens/src/vhdl/sens(str).vhd

sens/src/vhdl/sens.vhd				Sensor temp and voltage read out as with LOFAR RSP

sens/src/vhdl/sens_ctrl(rtl).vhd

sens/src/vhdl/sens_ctrl.vhd

sens/tb/vhdl/tb_sens.vhd			Test bench with I2C module usage example

The test bench tb_sens.vhd simulates the I2C master and some I2C sensor slaves. The Modelsim file libraries.ini must contain the paths to the i2c_lib, fmf_lib and sens_lib. The Modelsim project files <module_name>.mpf create a work directory for each library.

## I2C access example

This example shows how to configure and read the MAX6652 [7] I2C sensor. To ensure that the I2C access results are fresh it is useful to first overwrite the protocol results register with some arbitrary data. To access the I2C sensor write the sequence from Table 6 to the protocol list register.

| Octet | Value | Description |
| --- | --- | --- |
| 0 | 0x06 | PROTOCOL_WRITE_BYTE |
| 1 | 0x14 | ADDR = I2C address of MAX6652 connected to ground |
| 2 | 0x40 | CMD = Access MAX6652 config register |
| 3 | 0x11 | DATA = Enable config_start and config_line_freq_sel |
| 4 | 0x07 | PROTOCOL_READ_BYTE |
| 5 | 0x14 | ADDR |
| 6 | 0x20 | CMD = Read voltage measured at the MAX6652 2v5 input |
| 7 | 0x07 | PROTOCOL_READ_BYTE |
| 8 | 0x14 | ADDR |
| 9 | 0x22 | CMD = Read voltage measured at the MAX6652 3v3 input |
| 10 | 0x07 | PROTOCOL_READ_BYTE |
| 11 | 0x14 | ADDR |
| 12 | 0x21 | CMD = Read voltage measured at the MAX6652 12v input |
| 13 | 0x07 | PROTOCOL_READ_BYTE |
| 14 | 0x14 | ADDR |
| 15 | 0x23 | CMD = Read voltage measured at the MAX6652 vcc input |
| 16 | 0x07 | PROTOCOL_READ_BYTE |
| 17 | 0x14 | ADDR |
| 18 | 0x27 | CMD = Read the measured temperature |
| 19 | 0x13 | PROTOCOL_C_END |

Table 6  Protocol list example

The protocol list gets executed once the protocol list has been written. A few ms later the result will be available in the protocol result register. The actual latency depends on the I2C bit rate, which is set e.g. to 50 kbps. The expected protocol result for the protocol list of Table 6 is shown in Table 7.

| Octet | Value | Description |
| --- | --- | --- |
| 0 | 0 | 0 = PROTOCOL_WRITE_BYTE went OK |
| 1 | # | Value of 2v5 |
| 2 | 0 | 0 = PROTOCOL_READ_BYTE done |
| 3 | # | Value of 3v3 |
| 4 | 0 | 0 = PROTOCOL_READ_BYTE done |
| 5 | # | Value of 12v |
| 6 | 0 | 0 = PROTOCOL_READ_BYTE done |
| 7 | # | Value of vcc |
| 8 | 0 | 0 = PROTOCOL_READ_BYTE done |
| 9 | # | Value of temperature |
| 10 | 0 | 0 = PROTOCOL_READ_BYTE done |
| 11 | 0 | 0 = PROTOCOL_C_END done |

Table 7  Expected protocol result example

Octet 0 in Table 7 will read 1 if the I2C slave did not give an ACK (pull up), same for octets 2, 4, 6, 8 and 10. Similar octets 1, 3, 5, 7 and 9 will read 0xFF (pull up) when the I2C slave is not connected.

## Known issues

The I2C master module on LOFAR RSP and EMBRACE ECB and the I2C slave on the LOFAR RCUs are working reliably in practise. However there have been several issues.

### Clk rate and bit rate

LOFAR bug 575. Both the master and slave sample the SDA at the rising edge of SCL. The SDA setup time is (1+clk_cnt)*clk-period and the hold time is 4*(1+clk_cnt)*clk-period, where clk-period = 5 ns and clk_cnt = 399 for 200 MHz to get SCL clock rate of 1/(5*(1+clk_cnt)*clk-period) = 100 kbps. The setup and hold times are OK, so the I2C master is robust. The master also allows clock stretching by the slave (provided parameter clk_cnt >= 2).

### Bus arbitration

LOFAR bug 575. The bus arbitration logic in i2c_bit.vhd has been commented. This quick fix targets the problem of ocillating SDA line (looks like start every 4 I2C clk en cycles). Probably the issue is more severe if the difference between clk rate and bit rate is high, causing the SDA line to be sampled to soon by the I2C master, thereby mistaking its own activity for a collision. During an access SDA is sometimes not driven by neither the master nor the slave, causing the SDA to go briefly high while SCL = '0'.

### Ringing on SCL and SDA edges

LOFAR bug 1111. On LOFAR RSP the SCL showed undershoot for larger FPGA drive strengths. Once it was observed that an SDA ack from the RCU slave went high some 450 ns after the rising edge of SCL. The slave should release SDA after the falling edge. Hence apparently the RCU slave logic had recognized a falling edge in during the rising edge of SDA. There is indeed some ringing on the (slow) rising edge of SDA.. Sometimes the slave gives its ack one cylce too early, this can be seen e.g. when the last write bit is a 1, while on the line it is a 0 (due to the ack). This can be explained by an extra false SCL edge. The RCU slave release SDA on the falling edge of SCL. If timing is awkward, then SDA going high due to SCL going low could perhaps still be recognized as an I2C stop by the slave. Similar for I2C start. Using a stronger pull up 2k7 on the RCU instead of 10k gave a big improvement. In addition the RCU firmware has been adapted. The SCL and SDA are now first clocked in and filtered by the system clock. This avoids the potential false edge detections and it gives some more hold time of SDA after SCL falling edge.

### Comma logic

LOFAR bug 995. The LOFAR HBA client I2C handler on the RCU is implemented in software. It requires that an idle comma is inserted between octets. A comma of 1 msec is sufficient. Probably also after start a comma should be inserted, since I2C start (and restart) can also cause a slave interrupt. The I2C master already supports clock stretching, but this is not sufficient for the HBA client I2C slave, because it only can stretch the clk low for a read bytes access. Therefore comma logic was implemented in i2c_smbus(rtl).vhd.

### Sometimes access to LOFAR TDS fails

LOFAR bug 1242. Programming the LOFAR TDS PLL requires several 100 I2C accesses to the IO expander on the TDS clock board, because it uses bit-banging to program the PLL via SPI. About once every 5 runs the test fails, the protocol result buffer then shows that an I2C write access did not get an ACK. This is still an open issue, but with low priority.