Memo (note):

# Title:

Station base-band shift to decrease the effect of aliasing after the first filter bank at system level.

## Summary:

As a solution to the aliasing effect that appears in the subband separation and propagates in the rest of the system, it is proposed to shift the baseband of the width of the aliased band (~30 kHz) for different stations. If the shift is accurate (an integer number of kHz say N) it is possible to keep the signal in regularly spaced channels and correlate them. Because the base-band is shifted, the aliased part of the sub-band is correlated with a non aliased part and the effects of aliasing after cross correlation are reduced.

If multiple shifts are used say K=4 for the 100 stations, the contribution of the aliased bands in the cross correlation products is reduced by 16=4^2 and will affect only 1/16 of them.

## Drawback:

A direct effect of this is that the edges of the base-band are reduced by K*N kHz. This is a marginal loss because anyway the edges of the base-band are not sharp and had to be discarded.

Another effect that can be more damaging is that there are no alias free zones anymore for extreme spectral lines observations.  Of course, the station base-band shift should be an operating mode and not the only operating mode.

## To be checked:

- The baseband can be shifted by an integer number of kHz and this mode can be changed on demand.

- The contibutions of the RFI aliased into other subband do not significantly raise the noise floor with the baseband shift mode.

- There can be a procedure to detect the contribution of the aliased bands in the baselines and get a detection mechanism in case of a strong RFI.

- ….