Problematic choice of the filter:

- Critically sample polyphase filter :

Advantages:

- Already implemented on Matlab, C++, FPGA

- 20% less demanding on chirp than the oversampled version

Drawbacks:

- No synthesis filter available yet: Pulsar needs (filter_requirement(final).ppt)

- Aliasing due to downsampling: -20 to -40 db depending on the filter window used.      These aliasing have an occupation of 40-30 percent of the band (see filterbank2.ppt), it means that we hae to throw the corresponding channel in case of interferer. No possibility to play wit th LO settings to get rid of the RFI after correlation because no LO are used.

- Oversampled polyphase filter:

Advantages:

- More flexible: we can adapt the subband width depending on the type of filter used. It gives alternative to relax the number of coefficient.

- Already implemented on Matlab, C++.

- Synthesis filter available (Matlab)

Drawbacks:

- FPGA implementation will last at least a month (Wessel estimates)

- 20% more demanding in term of computation power than the critically sampled version

Conclusion: The oversampled filter bank gives a better solution in term of signal quality compare to the critically sampled filter +30% with an increase of the cost of about 20%. This solution is more flexible due to the oversampling as a parameter for the filter.