Lofar station digital filtering:

The development of the digital technology allows now performing a major part of the signal processing digitally. In fact, a Lofar station is composed by 92 RCUs (Receiver Unit: Antenna, mixing and low pas filtering, Low amplifier, ADC), which give out 13 bits digital signals sampled at 64Mhz. A Lofar station is based on beamforming using phased shifter. The particularity of this phase rotation technique is that the delay applied will be frequency dependant. Therefore a first stage of filtering will split the signal into subbands in order to apply the phase delay corresponding to the particular frequency. In fact, the beamforming for a station will be based on an approximation, which will be completely dependant of the the frequency resolution obtained after filtering. After applying the good rotation the signal coming from each antenna will be added together. The output signal will be then filtered again to reach about 1Khz resolution, what we will call channels.

The filtering stage is essential in order to realize beamforming in maximizing the signal to noise ratio and minimizing unwanted effects like aliasing, distortions… Therefore a group has been created, which has got one goal, gives out of studies, experiments and implementations, the requirements and possible implementations for the filtering involved in a Lofar station. There are two important aspects for this goal, the first one is to meet the signal quality required in order to achieve astronomical observations and furthermore give alternatives for non-distorted, realizable, and low cost implementation.

The signal quality required by the astronomers should derive some requirements related to the type of studies and modes that Lofar is supposed to handle (X rays, Pulsars…). But other stakeholders will have to give their point of view in taking into account the calibration procedure or the possible RIF environment, which will had constraint on the filtering solution.

Two different perspectives are interesting to take into account: the top-down study, which will end up with the requirements for the signal quality needed by the stakeholders; and, the bottom-up point of view, which consists on having implemented platforms respecting the requirements as much as possible (involving quantization, non-linearity…). Therefore, the filtering group has been split into two groups a core and a knowledge support group.

Working task force:

The core is made of 5 engineers who work mostly on implementation purposes but also achieve different studies in order to evaluate the signal quality for different filtering conditions. The work has been then divided into 3 levels.

1 - Study level where different implementation solutions will be tested under Matlab.

2 – the design will then be implemented in VHDL and the design simulated in order to match with Matlab result

3 – Test bench: As soon as the design is implemented in VHDL we will place it on chirp and evaluate first the quality of the implementation and the signal quality on the real platform. The FPGA used will have also Theta connection available to study the behavior of the filtering solution and a real environment.

Out of this statement, we can derive a schema, which will summarize the investigation flow:

Test  bench

Or Theta

implementation

Design Filter: Matlab & VHDL

Initial requirements

Implementation evaluation

Evaluate filter

Filter investigation Flow

The knowledge support group:

Specialists in different fields, signal processing, and quantization compose the knowledge group, RFI... they are gathered to advise and to support the work force in commenting the different designs proposed, analyze different results and propose alternatives.

Status:

This activity started 3 weeks ago in defining the first targets: create the test bench and compare the different simulations for a simple example one FIR filter:

We already have compared the VHDL implementation with the Matlab one. The filter response is respected, but It turns out that the result are different for data < 10^-6 due certainly some rounding effects and truncation.

Some quantization studies are still under development, but the first results are already interesting:

The filter response will change according to the number of bits used to code the coefficients. The linearity of the phase will be conserved due to the remaining symmetry of the coefficients after quantization.

Test bench:

The test bench is under development but the missing connection is Ethernet link 100Mb/s due to some driver development.

ADC : 80Ms/s

Control     Pc

GPIB interface

Ethernet link 100Mb/s

Data Wrapper

Data Unwrapper

FPGA

ADC

Synthetiser

Knowledge group:

Some studies are conducted in order to evaluate the limitations of the current implementation of the polyphase filter and the different solutions valuable to increase the signal quality.