APERTIF Filter Bank Firmware Specification

Part 2

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Eric Kooistra | ASTRON |  |
| Controle / Checked: |  |  |
| Andre Gunst | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Andre Gunst | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature | ASTRON |  |
| © ASTRON 2012 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Andre Gunst Harm-Jan Pepping Daniel van der Schuur Raj Rajan Thilak | Gijs Schoonderbeek |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2012-03-8 | Eric Kooistra | First draft. |

Table of contents:

1	Introduction	6

1.1	Purpose	6

1.2	Background	6

1.3	Planning	6

2	Functional specification	7

2.1	General	7

2.2	Using standard logic vectors or sosi records	7

2.3	Complex FFT	7

2.3.1	Algorithm	7

2.3.2	rTwoSDF PFT entity	9

2.3.3	rTwoSDF PFT architecture	10

2.4	Complex FFT for two real inputs	12

2.4.1	Reorder function	13

2.4.2	Separate function	13

2.4.3	PFT entity and interface	13

2.4.4	Biplex PFT	14

2.5	Wideband complex FFT	14

2.5.1	Topological approach	14

2.5.2	Parallel FFT entity and interface	17

2.5.3	The WFFT entity and interface	17

2.6	Wideband complex FFT for two real inputs	18

2.7	PFS	18

2.7.1	PFS entity and interface	20

2.8	Wideband PFS	20

2.8.1	WPFS entity and interface	21

2.9	PFB for two real inputs	21

2.10	Wideband PFB for two real inputs	22

2.10.1	WPFB entity and interface	22

3	Implementation specification	23

3.1	Firmware	23

3.2	Verification requirements	24

3.2.1	Verification approach for WPFS	25

3.2.2	Verification approach for WFFT	26

3.2.3	Verification approach for WPFB	26

3.3	Validation requirements	26

4	Deliverables	27

4.1	Firmware	27

4.1.1	Not to do (yet)	27

4.2	Documentation	27

Terminology:

ADC		Analogue to Digital Convertor

APERTIF	Aperture Tile In Focus

DFT		Discrete Fourier Transform

DIF		Decimate In Frequency

DIT		Decimate In Time

DUT		Device Under Test

Eop		End of packet

FFT		Fast Fourier Transform

FIR		Finite Impulse Response

FPGA		Field Programmable Gate Array

HDL		Hardware Description Language

HW		Hardware

IO		Input Output

IP		Intellectual Property

LOFAR		Low Frequency Array

LS		Least Significant (part, bit, byte)

MM		Memory-Mapped

MS		Most Significant (part, bit, byte)

PFB		Poly-phase Filter Bank

PFS		Pre Filter Structure for the PFT in a PFB

PFT		Pipelined FFT

R2SDF		Radix-2 Single Delay Feedback (pipelined FFT architecture)

RTL		Register Transfer Level

SISO		Source In Sink Out

Slice		FFT block size

SL		Standard Logic (VHDL)

SLV		Standard Logic Vector (VHDL)

SNR		Signal to Noise Ratio

Sop		Start of packet

SOSI		Source Out Sink In

ST		Streaming

SW		Software

Subband	Frequency bin of the PFB

UNB		Path to UniBoard Firmware directory [12]

WFFT		Wideband FFT

WPFS		Wideband PFS

WPFB		Wideband PFB

References:

- “APERTIF Filter bank Firmware Specification”, feb 2011, ASTRON-RP-474, E. Kooistra

- “Detailed Design of the Digital Beamformer System for APERTIF”, ASTRON-RP-413, G. Schoonderbeek, A. Gunst, E. Kooistra

- “A R2SDF architecture based generic FFT for firmware implementation”, ASTRON-RP-755, dec 2011, R.T. Rajan

- “A wideband FFT design for firmware implementation”, ASTRON-RP-880, dec 2011, R.T. Rajan

- “Specification for module interfaces using VHDL records”, ASTRON-RP380, E. Kooistra

- "RSP Firmware Design Description", LOFAR-ASTRON-SDD-018, Wessel Lubberhuizen, the relevant chapter is also available as LOFAR_pfb.pdf in $UNB/Firmware/modules/LOFAR/pft2/doc

- “Brief description of the LOFAR station signal processing”, LOFAR-ASTRON-MEM-238, 2007, E. Kooistra

- "Understanding Digital Signal Processing", R. Lyons

- “Theory and Application of Digital Signal Processing”, L.R. Rabiner, B. Gold

- “Polyphase filter bank quantization analysis”, 2004, LOFAR-ASTRON-MEM-109, J. Stemerdink

- “Quantization Error Analysis of Digital Signal Processing Blocks”, 2004, LOFAR-ASTRON-MEM-129, J. Stemerdink

- https://svn.astron.nl/UniBoard_FP7/UniBoard/trunk, the UniBoard FP7 SVN repository ($UNB)

- $UNB/Firmware/modules/LOFAR/pfs, contains the PFS

- $UNB/Firmware/modules/LOFAR/pft2, contains the PFT and the PFB, doc/lofar_pfb_readme.txt

- $UNB/Firmware/modules/dsp/rTwoSDF, contains rTwoSDF PFT, /doc/r2sdf_readme.txt

- https://casper.berkeley.edu, CASPER community

- “UniBoard Pulsar Project Definition”, April 2011, University of Manchester, Aziz AhmedSaid, e.a.

- “A Scalable Correlator Architecture Based on Modular FPGA Hardware and Data Packetization”, Feb 4, 2008, Aaron Parsons e.a.

- "A New Approach to Radio Astronomy Signal Processing: Packet Switched, FPGA-based, Upgradeable, Modular Hardware and Reusable, Platform-Independent Signal Processing Libraries", 200509URSI.pdf, Aaron Parsons e.a.

- “The Symmetic Group in Data Permutation, with Applications to a High-Bandwidth Streaming FFT Architecture”, Aaron Parsons

- www.rfel.com

# Introduction

## Purpose

This document specifies the Wideband Poly-phase Filter Bank (WPFB) firmware module that is required for APERTIF [2]. This document is called ‘part 2’ because it supersedes the previous specification document [1]. In addition this document also provides extra information to facilitate the development of the WPFB.

## Background

The result of the previous specification [1] was the R2SDF pipelined FFT VHDL firmware implementation described in [3], plus a design note on how to extend the FFT to a wideband implementation for higher sample rates [4]. The R2SDF pipelined FFT will be reused in the wideband filterbank.

The wideband filterbank is a poly-phase filterbank (PFB). The term wideband is used to indicate that the data sample clock rate is a factor P higher than the digital processing clock rate. The wideband factor P ≥ 1 and typically P is a power of 2. The LOFAR PFB module [6] was ported to UniBoard and is available at [13, 14], but only supports P =1. For APERTIF P = 4 is needed to be able to process 800 MSps using a digital clock frequency of 200 MHz [2]. The wideband filterbank consists of:

- A wideband Finite Impulse Response (FIR) pre-filter structure (WPFS)

- A wideband pipelined complex Fast Fourier Transform (WPFT) including a separate function to be able to use a complex FFT for two real inputs

## Planning

The complex pipelined FFT is the R2SDF pipelined FFT available at [15] and forms the building block for the wideband filterbank. Hence the WPFB will not reuse the LOFAR PFT. The next development steps are:

- Parallel FFT

- Wideband complex FFT

- Wideband complex FFT for two real inputs

- Wideband PFS

- Wideband PFB for two real inputs

The SOSI sync, sop, and eop signals are not essential for the operation of the WPFB. Therefore these IO signals should be added to the WPFB as a final step after the functional development of the WPFB and its internal components has been done.

# Functional specification

## General

The Wideband Poly-phase Filter Bank is referred to as WPFB.

- The WPFB firmware should be implemented in a generic and modular way.

- The WPFB implementation must support P power of 2 and gracefully include P=1.

- The WPFB performance only needs to be verified for the settings of the APERTIF beamformer [2] application case, so for 512 subbands (so an N=1024 points FFT), 8-bit real input samples, 800 MSps and 200 MHz digital processing clock (so P=4).

- The WPFB must be capable of handling arbitrary data invalid gaps in the input data stream. Being able to cope with data invalid helps to ensure a proper implementation of the WPFB.

## Using standard logic vectors or sosi records

For the wideband signals the data of P samples arrives in parallel. The choice is to:

- Use P separate standard logic vector (slv) signals in parallel

- Concatenate the P signals into a single slv

- Use a sufficiently wide general slv array like e.g. t_rtwo_slv_arr in rtwo_pkg.vhd (subtype of t_slv_32_arr in $UNB - common_pkg.vhd) to contain the P signals

- Use a single sosi record (from $UNB – dp_stream_pkg.vhd [5]) for all P data samples by concatenating them in the slv data field or in the ‘re’ and ‘im’ fields

- Use an array of P sosi records (from $UNB – dp_stream_pkg.vhd [5])

Option 1 is suitable for P=1 but it discarded for P>1, because the entity IO then fixed so not flexible for supporting different values for P. Option 2 and 4 are discarded because concatenating data signals into a single slv is awkward to read as hexadecimal numbers when the data width in not an integer number of nibbles. Hence this leaves using option 3 with an array of P sufficiently wide slv or option 5 with an array of sosi records. Both are fine. The disadvantage of using sosi records is that many fields are unused and in case of large arrays the relevant fields get more difficult to view in the Wave Window. Therefore option 3 is deemed most suitable to use for the WPFB and its components.

In addition it option 5 can still be supported by defining wrapper entities that map the IO to the sosi records. This may typically be useful for the higher level blocks like the WPFS, WFFT and the WPFB.

## Complex FFT

### Algorithm

The FFT consists of butterfly operations. Figure 1 shows the optimized structure of the FFT butterfly for decimation-in-frequency as explained in section 4.6 in [8] (left) and an instance diagram (right).

Figure 1: Optimized form of butterfly structure for decimation-in-frequency

The term WN=e-j2π/N and is called the twiddle factor. Note that:

- The choice between using decimation-in-frequency (DIF) or decimation-in-time (DIT) is arbitrary. The rTwoSDF [3] uses DIF.

- Using DIF or DIT has nothing to do with whether the FFT output will be bit-reversed or not, see section 4.5 in [8]. The bit reversing is due to the crossing signal flows that are needed to let each input sample be weighted directly or indirectly by the twiddle factors (carrier waves).

Figure 2 shows the signal flow diagram for a parallel N=16 point FFT using DIF and with-bit reversed outputs. The twiddle factors in Figure 2 are represented by the exponent k of W16 and follow from section 13.16 in [8].

Figure 2: Signal flow diagram for a parallel N=16-point decimation-in-frequency FFT

### rTwoSDF PFT entity

Figure 3 show a block diagram of the Pipelined complex FFT (PFT) that is implemented in rTwoSDF.vhd [3].

Figure 3: Block diagram of the rTwoSDF implementation in rTwoSDF.vhd

The stage numbers in Figure 3 and rTwoSDF.vhd correspond to the stage numbers in Figure 2. The rTwoSDF PFT has two modes:

- g_use_reorder = false yields bit reversed order output

- g_use_reorder = true yields normal order output

The tb_rTwoSDF.vhd uses g_use_reorder =true to verify the PFT using a complex noise input signal with a know result.

### rTwoSDF PFT architecture

The pipelined aspect of the PFT appears from the fact that each stage processes its N inputs sequentially in time. Hence each stage only needs the hardware resources for a single butterfy operation of Figure 1. It is important to distinguish the algorithm pipelining of z-1 sample delays from the digital pipelining for RTL clock delays. In Figure 2 the algorithm pipelining progresses vertically from top to bottom. The RTL pipelining that is necessary to achieve timing closure for higher processing clock rates can be placed horizontally between the stages. The rTwoSDFStage has several internal pipeline settings for z-1 delays and for RTL delays that are kept in a record in rTwoSDFPkg.vhd. The rTwoSDF simulates OK, also when all RTL pipelining delays are set to 0 and also when in_val is only active at arbitrary clock cycles. Figure 4 shows the block diagram of rTwoSDFStage.

Figure 4: Block diagram of rTwoSDFStage.vhd

The feedback delay lines (FIFOs) to store the data for the z-1 delays are located inside rTwoBFStage as shown in Figure 5. Note the AND-gate to make out_val. After the stage delay the feedback valid output goes high and remains high and acts as an enable for in_val to out_val. This is important for controlling the local counter in each rTwoSDFStage that controls the BF select and the twiddle selection (see Figure 4).

Figure 5: Block diagram of rTwoBFStage with the feedback delay lines

The rTwoBF in Figure 5 implements:

c = a + b when sel='1' else a

d = a - b when sel='1' else b

There is the option via t_rtwo_sdf_stage_pipeline to move some of the feedback z-1 delays into rTwoBF to ease timing closure for synthesis, but typically this is not useful.

Dependent on the in_sel input the rTwoWMul in Figure 4 multiplies the rTwoBFStage output with the twiddle factor from rTwoWeight or it passes the rTwoBFStage output on.

The rTwoWMul also performs truncation and resizing on the product. For safe scaling the output of each FFT stage must have 1 bit extra than the input [10, 11]. The twiddle factors are normalized 16 bit numbers, therefore the 16 - 1 = 15 LSbits of the product get truncated. The truncate function keeps the MS part. The remaining product vector is then resized to the output width. The resize function extends the sign bit or keeps LS part. Note that the resize function differs from the resize function in the IEEE numeric_std VHDL package, because that resize function always preserves the sign bit. All rTwoSDFStage stages use the same data width as the final rTwoSDF g_out_dat_w output width. By choosing g_out_dat_w = g_in_dat_w + ½(1+log2(N)) the contribution of the FFT implementation to the quatization noise is about equal to the input quantization noise [11]. Therefore the tb_rTwoSDF.vhd uses 6 bits extra for the output when g_nof_points =  N = 1024.

For more information on the implementation of the rTwoSDF PFT see [3] and the r2sdf_readme.txt in [15].

## Complex FFT for two real inputs

Figure 6 shows the Pipelined FFT (PFT) for a time series complex input, including the option to use two real inputs.

Figure 6: Data flow diagram for the PFT

The PFT has 3 modes:

- bit reversed order output

- normal order output

- normal order output separated for 2 real inputs

Figure 7 shows the interface signals for the PFT at the input (x = xre + jxim) and after the ‘transform’ with ‘reorder’ (X = Xre + jXim) and after ‘separate’ (a = xre, b = xim, A = Are + jAim and B = Bre + jBim).

Figure 7: Interface signals for the PFT

The separated outputs A and B are interleaved as shown in Figure 7 with their real and imaginary parts available in parallel at the same time. A block of N real input samples for a and for b results in a block of N/2 complex frequency samples for A and for B. These complex frequency samples for A and for B are called subbands [7].

### Reorder function

The reorder function is already available in rTwoOrder.vhd. This rTwoOrder uses a dual page memory whereby N samples are written in one page while the previous N samples are read from the other page. Reference [20] explains that at the expense of some more addressing logic it is possible to use a single page memory to achieve the reordering. However for our purposes the existing rTwoOrder.vhd suffices so no improvements should be made for that yet.

### Separate function

The separate function is available in the LOFAR PFT pft_separate.vhd, but it is better to redesign it for the APERTIF PFT and call it rtwo_separate.vhd. The separate function follows from section 13.5 in [8] and is defined by:

2 Are(m) = Xre(N-m) + Xre(m)

2 Bim(m) = Xre(N-m) - Xre(m)

2 Aim(m) = Xim(m) - Xim(N-m)

2 Bre(m) = Xim(m) + Xim(N-m)

For m = 0 to N/2-1, whereby X(N)=X(0), which is automatically achieved by using modulo N addressing. The FFT output A is conjugate symmetric so only the first N/2 values need to be calculated, similar for B.

### PFT entity and interface

The PFT entity is called rtwo_pft.vhd. Conform Figure 6 rtwo_pft.vhd instantiates rTwoSDF.vhd,vhd from section 2.3 that implements the ‘transform’ and ‘reorder’ and it instantiates rtwo_separate.vhd from section 2.4.2 that implements the ‘separate’. The interface for rtwo_pft is defined in Table 1.

| Signal | Type | I/O | Width | Description |
| --- | --- | --- | --- | --- |
| clk | SL | IN | - | Data path clock |
| rst | SL | IN | - | Data path resetReal input |
| in_re | SLV | IN | in_dat_w | Real input xre or a |
| in_im | SLV | IN | in_dat_w | Imaginary input xim or b |
| in_valid | SL | IN | - | Input data valid strobe |
| in_sop | SL | IN | - | Input start of data block |
| in_eop | SL | IN | - | Input end of data block |
| out_re | SLV | OUT | out_dat_w | Real output Xre or Are,Bre |
| out_im | SLV | OUT | out_dat_w | Imaginary output Xim or Aim,Bim |
| out_valid | SL | OUT | - | Output data valid strobe |
| out_sop | SL | OUT | - | Output start of data block |
| out_eop | SL | OUT | - | Output end of data block |

Table 1: Interface signals for rtwo_pft

The sop and eop mark the start and end of the FFT block (also called FFT slice) as shown in Figure 7. The rtwo_pft should merely delay them appropriately. The other sosi fields are not used and the PFT does not support backpressure so there are no siso signals. The rtwo_pft should use a single generic record t_rtwo_fft as defined in Table 2. For the rTwoSDF pipeline settings the default c_rtwo_sdf_stage_pipeline settings from rTwoSDFPkg can be used (section 2.3).

| Generic field | Type | Default | Description |
| --- | --- | --- | --- |
| use_reorder | Boolean | true | When false output in bit reversed order, else output in normal order |
| use_separate | Boolean | true | When false output for complex input, else separates the output for two real inputs |
| wb_factor | Natural | 1 | Wideband factor P is not used in rtwo_pft so effectively P = 1 |
| twiddle_offset | Natural | 0 | Twiddle offset for PFT sections in a WFFT, default 0 for P = 1. |
| nof_points | Natural | 1024 | FFT size N |
| in_dat_w | Natural | 8 | Input data width |
| out_dat_w | Natural | 13 | Output data width |

Table 2: Generic record type t_rtwo_fft for PFT and WFFT

The record type and default value c_rtwo_pft should be kept in the package rtwo_pkg.vhd. If necessary this package can also include definitions for the other components within the rtwo_pft.

### Biplex PFT

Examining Figure 2 it becomes clear that the multipliers in the rTwoSDF are only used 50% of the time. A biplex PFT gets this to 100% usage by applying a second input stream to the PFT in such a way that this stream can use the multiplier the other 50% of the time [18, 19, 20]. However for our purposes the existing rTwoSDF.vhd suffices so no ‘biplex’ improvements should be made for that yet.

## Wideband complex FFT

### Topological approach

Figure 8 shows a principle block diagram for the Wideband FFT that was derived from figure 3 of [1] that was reproduced from reference [21].

Figure 8: Principle block diagram of the Wideband FFT based on [21] reproduced in [1]

In principle an N-point WFFT consists of P M-point PFT sections followed by a P-point parallel DFT, whereby N=P*M. Note that the parallel DFT becomes an parallel FFT for P powers of 2. Between the PFT sections and the parallel DFT the outputs of the PFT sections get rotated dependent on the section number. The CASPER community [16] have a WFFT and WPFB, but there is no reference that describes these in detail [18, 19], although figure 3 in reference [20] provides some more insight. The WFFT could be seen as the problem of performing a large FFT using small FFTs, this problem is analyzed section 13.47 in [8] and in [17]. In [4] Raj Rajan Thilak derives a structure for a WFFT and provides a MATLAB simulation for it. However this structure does not seem to solve the parallel DFT as an FFT. Most these references take a mathematical approach. An alternative is to take a topological approach based on the data flow graph of the parallel FFT in Figure 2. Figure 9 shows the same as Figure 2 but now using the more compact butterfly representation from Figure 1 with bfN(k) indicated by k.

Figure 9: Butterfly block diagram for a parallel N=16-point decimation-in-frequency FFT

Similar as in figure 3 in reference [20] the samples in Figure 9 with the same color appear serially, while samples of different color arrive in parallel. For P=4 stage 1 and 2 = log2(P) need to be processed in parallel while the other stages can still be pipeline processed. The topological approach is now to redraw Figure 9 as shown in Figure 10.

Figure 10: Butterfly block diagram for an N=16-point wideband FFT

Clearly the wideband FFT consists of P M-point PFT sections and P P-point FFT sections. The PFT sections need an additional twiddle_offset parameter to be able to adjust the twiddles. This twiddle_offset parameter is defined in t_rtwo_fft in Table 2. It avoids the need for the complex multipliers section in Figure 8 and so the WFFT of Figure 10 results in the block diagram of Figure 11.

Figure 11: Block diagram for an N=MP-point wideband FFT

The parallel P-point FFT sections can be constructed by simply instantiating the appropriate number of butterfly sections from rTwoSDF in parallel with fixed twiddle factors per instance.

### Parallel FFT entity and interface

The parallel FFT entity is called rtwo_fft.vhd. The parallel FFT must support the same functionality as the PFT, so N-points, arbitrary control of in_val, and able to operate for RTL latencies ≥ 0. Table 3 defines the interface signals for the rtwo_fft.vhd.

| Signal | Type | I/O | Width | Description |
| --- | --- | --- | --- | --- |
| clk | SL | IN | - | Data path clock |
| rst | SL | IN | - | Data path reset |
| in_re_arr[0:N-1] | t_rtwo_slv_arr | IN | in_dat_w | Real inputs for index t is [0, 1, 2, …, N-1] |
| in_im_arr[0:N-1] | t_rtwo_slv_arr | IN | in_dat_w | Imaginary inputs for index t is [0, 1, 2, …, N-1] |
| in_valid | SL | IN | - | Input data valid strobe, same for all in array |
| out_re_arr[0:N-1] | t_rtwo_slv_arr | OUT | out_dat_w | Real output Xre, index f is [0, 1, 2,…, N-1] or Separate output Are,Bre for two real inputs, index f is: [0,0, 1,1, 2,2, …, (N-1)/2,(N-1)/2] |
| out_im_arr[0:N-1] | t_rtwo_slv_arr | OUT | out_dat_w | Imaginary output Xim, index f is [0, 1, 2,…, N-1] or Separate output Aim,Bim for two real inputs, index f is: [0,0, 1,1, 2,2, …, (N-1)/2,(N-1)/2] |
| out_valid | SL | OUT | - | Output data valid strobe, same for all in array |

Table 3: Interface signals for rtwo_fft (parallel FFT)

The rtwo_wfft.vhd should use the same single generic t_rtwo_fft as defined in Table 2, and ignore the wb_factor and twiddle_offset fields. The rtwo_fft.vhd must also support use_reorder and use_separate fields. Note that for the parallel FFT the ‘reorder’ operation becomes a rewiring function with t_rtwo_slv_arr in and out.

### The WFFT entity and interface

The Wideband FFT entity is called rtwo_wfft.vhd. The wideband factor P > 1 implies that P data samples arrive in parallel per clock cycle. Table 4 defines the interface signals for the rtwo_wfft.vhd.

| Signal | Type | I/O | Width | Description |
| --- | --- | --- | --- | --- |
| clk | SL | IN | - | Data path clock |
| rst | SL | IN | - | Data path reset |
| in_re_arr[0] in_re_arr[1] … in_re_arr[P-1] | t_rtwo_slv_arr | IN | in_dat_w | Real input, index t is p + [0, P, 2P, …, (M-1)P]: xre or a for p = 0 xre or a for p = 1 … xre or a for p = P-1 |
| in_im_arr[0] in_im_arr[1] … in_im_arr[P-1] | t_rtwo_slv_arr | IN | in_dat_w | Imaginary input, index t is p + [0, P, 2P, …, (M-1)P]: xim or b for p = 0 xim or b for p = 1 … xim or b for p = P-1 |
| in_valid | SL | IN | - | Input data valid strobe, same for all in array |
| out_re_arr[0] out_re_arr[1] … out_re_arr[P-1] | t_rtwo_slv_arr | OUT | out_dat_w | Real output, index f is p + [0, P, 2P,…, (M-1)P]: Xre for p = 0 Xre for p = 1 … Xre for p = P-1 Separate output for two real inputs, index f is [p,p] + [0,0, P,P, 2P,2P, …, (M-1)P/2,(M-1)P/2]: Are,Bre for [p,p] = [0,0] Are,Bre for [p,p] = [1,1] … Are,Bre for [p,p] = [P-1,P-1] |
| out_im_arr[0] out_im_arr[1] … out_im_arr[P-1] | t_rtwo_slv_arr | OUT | out_dat_w | Imaginary output, index f is p + [0, P, 2P,…, (M-1)P]: Xim for p = 0 Xim for p = 1 … Xim for p = P-1 Separate output for two real inputs, index f is [p,p] + [0,0, P,P, 2P,2P, …, (M-1)P/2,(M-1)P/2]: Aim,Bim for [p,p] = [0,0] Aim,Bim for [p,p] = [1,1] … Aim,Bim for [p,p] = [P-1,P-1] |
| out_valid | SL | OUT | - | Output data valid strobe, same for all in array |

Table 4: Interface signals for rtwo_wfft (wideband FFT)

The rtwo_wfft.vhd should use the same single generic t_rtwo_fft as defined in Table 2, but now with default P = 4 for the wb_factor field.

## Wideband complex FFT for two real inputs

The WFFT for two real inputs reuses the ‘reorder’ and ‘separate’ functions from section 2.3.

## PFS

The pre-filter structure (PFS) from LOFAR can be re-used. The pfs.vhd from LOFAR was ported to $UNB and is available at [13]. The LOFAR PFS implements P=1 and is described in [6]. Figure 12 shows the block diagram of the LOFAR PFS. Figure 13 shows the details of the FIR filter implemented by pfs_filter.vhd that is used in the LOFAR PFS.

Figure 12: Block diagram of the LOFAR PFS implemented in pfs.vhd

Figure 13: Block diagram of pfs_filter.vhd

### PFS entity and interface

Table 5 and Table 6 show the interface signals and generic parameters of the LOFAR pfs.vhd.

| Signal | Type | I/O | Width | Description |
| --- | --- | --- | --- | --- |
| clk | SL | IN | - | Data path clock |
| rst | SL | IN | - | Data path reset |
| dp_in.x | SLV | IN | g_dp_in_w | Input x |
| dp_in.y | SLV | IN | g_dp_in_w | Input y |
| dp_in.val | SL | IN | - | Input data valid strobe |
| dp_out.x | SLV | OUT | g_dp_out_w | Output x input a for N-point complex PFT with two real inputs |
| dp_out.y | SLV | OUT | g_dp_out_w | Output y input b for N-point complex PFT with two real inputs |
| dp_out.val | SL | OUT | - | Output data valid strobe |

Table 5: Interface signals for LOFAR pfs.vhd

| Generic | Type | Default | Description |
| --- | --- | --- | --- |
| g_nof_bands | Natural | 512 | Number of subbands is N/2 |
| g_nof_taps | Natural | 16 | Number of FIR taps per subband |
| g_dp_in_w | Natural | 8 | Input data width |
| g_dp_out_w | Natural | 12 | Output data width (bit growth is ceil_log2(g_nof_taps) |
| g_coef_dat_w | Natural | 16 | FIR coefficients width |
| g_coef_init_file | String | - | Path to and name of coefficients RAM initialization file |

Table 6: Generics for LOFAR pfs.vhd

## Wideband PFS

The Wideband PFS may be constructed as sketched in Figure 14 using the pfs.vhd as building block. The pfs[1], pfs[2], and pfs[3] sections could use the FIR coefficients stored in the pfs[0] section to save RAM, because all P = 4 sections need the same FIR coefficients at the same time.

Figure 14: Sketch of the block diagram of the Wideband PFS for 16 taps and P=4

### WPFS entity and interface

The WPFS entity is called rtwo_wpfs.vhd. Table 7 defines the interface signals for the rtwo_wpfs.vhd.

| Signal | Type | I/O | Width | Description |
| --- | --- | --- | --- | --- |
| clk | SL | IN | - | Data path clock |
| rst | SL | IN | - | Data path reset |
| in_dat_arr[0] in_dat_arr[1] … in_dat_arr[P-1] | t_rtwo_slv_arr | IN | in_dat_w | Real input, index t is p + [0, P, 2P, …, (M-1)P]: x for p = 0 x for p = 1 … x for p = P-1 |
| in_valid | SL | IN | - | Input data valid strobe, same for all in array |
| out_dat_arr[0] out_dat_arr[1] … out_dat_arr[P-1] | t_rtwo_slv_arr | OUT | out_dat_w | Real output, index t is p + [0, P, 2P, …, (M-1)P]: y for p = 0 y for p = 1 … y for p = P-1 |
| out_valid | SL | OUT | - | Output data valid strobe, same for all in array |

Table 7: Interface signals for rtwo_wpfs

| Generic | Type | Default | Description |
| --- | --- | --- | --- |
| wb_factor | Natural | 1 | Wideband factor P |
| nof_bands | Natural | 512 | Number of subbands is N/2 |
| nof_taps | Natural | 16 | Number of FIR taps per subband |
| in_dat_w | Natural | 8 | Input data width |
| out_dat_w | Natural | 12 | Output data width |
| coef_dat_w | Natural | 16 | FIR coefficients width |
| coef_init_file | String | - | Path to and name of coefficients RAM initialization file |

Table 8: Generic record type t_rtwo_pfs for WPFS

## PFB for two real inputs

The PFB for two real inputs combines the LOFAR PFS from section 2.7 with the complex PFT for two real input from section 2.4 as shown in Figure 15 and in more detail in figure 1 in [1].

Figure 15: Block diagram of the PFB

Note that each subband has 1 complex sample per FFT slice, hence the downsample factor per subband is N, so the downsampling occurs at the Nyquist rate. This is called a critically sampled PFB and is the default for using an FFT with no input block overlap.

It may be useful to try this configuration in a test bench to prepare for the WPFB. However the WPFB also contains the PFB, because the WPFB must also suit P=1.

## Wideband PFB for two real inputs

The WPFB has the similar structure as the PFB in Figure 15 but now using the Wideband variants for P≥1 of the WPFS and the WFFT.

### WPFB entity and interface

The WPFB entity is called rtwo_wpfb.vhd. Table 9 defines the interface signals for the rtwo_wpfb.vhd. The rtwo_wpfb uses the generic records from t_rtwo_fft from Table 2 and t_rtwo_pfs from Table 8.

| Signal | Type | I/O | Width | Description |
| --- | --- | --- | --- | --- |
| clk | SL | IN | - | Data path clock |
| rst | SL | IN | - | Data path reset |
| in_dat_a_arr[0] in_dat_a_arr[1] … in_dat_a_arr[P-1] | t_rtwo_slv_arr | IN | in_dat_w | Real input a, see Table 7 |
| in_dat_b_arr[0] in_dat_b_arr[1] … in_dat_b_arr[P-1] | t_rtwo_slv_arr | IN | in_dat_w | Real input b, see Table 7 |
| in_valid | SL | IN | - | Input data valid strobe, same for all |
| out_re_arr[0] out_re_arr[1] … out_re_arr[P-1] | t_rtwo_slv_arr | OUT | out_dat_w | Real output Are,Bre, see Table 4 |
| out_im_arr[0] out_im_arr[1] … out_im_arr[P-1] | t_rtwo_slv_arr | OUT | out_dat_w | Imaginary output Aim,Bim, see Table 4 |
| out_valid | SL | OUT | - | Output data valid strobe, same for all |

Table 9: Interface signals for rtwo_wpfb

# Implementation specification

## Firmware

Requirements:

- The WPFB source code can be kept in dsp/rTwoSDF and all new VHDL files have prefix ‘rtwo_’ and use lower case names. Table 10 provides a preliminary list of the expected files.

- The FPGA specific resources like multipliers and RAM must be instantiated via the VHDL wrapper components in common_lib.

- The WPFB should be synthesized for the Stratix IV PFGA (type EP4SGX230KF40C2) using the Quartus II synthesis tool.

- The WPFB should be able to clock at >250 MHz. The timing bottleneck must be clear and not trivial to improve.

| File | Description |
| --- | --- |
| rTwoSDF.vhd | The already existing Radix-2 PFT component as described in [3, 15] |
| rtwo_separate.vhd | Separate function for two real inputs (section 2.4.2) |
| rtwo_pft.vhd | PFT is the pipelined FFT (section 2.4) |
| rtwo_fft | Parallel FFT |
| rtwo_wfft.vhd | Wideband FFT (section 2.5) |
| rtwo_wpfs.vhd | Wideband PFS (section 2.8) |
| rtwo_wpfb.vhd | Wideband PFB (section 2.10) |
| tb_rTwoSDF.vhd | Test bench for rTwoSDF |
| tb_rtwo_pft.vhd | Test bench for rtwo_pft (see tb_rTwoSDF and LOFAR tb_pft2.vhd) |
| Tb_rtwo_fft.vhd | Test bench for rtwo_fft (same stimuli and results as with tb_rtwo_pft) |
| tb_rtwo_wfft.vhd | Test bench for rtwo_wfft (same stimuli and results as with tb_rtwo_pft) |
| tb_rtwo_wpfs.vhd | Test bench for rtwo_wpfs (same stimuli and results as with LOFAR tb_pfs.vhd) |
| tb_rtwo_wpfb.vhd | Test bench for rtwo_wpfb.vhd (see LOFAR tb_pfb2.vhd) |
| r2sdf.qip | Quartus IP file in synth/quartus with all rTwoSDF source files |
| rtwo.qip | Quartus IP file in synth/quartus including r2sdf.qip and all other rtwo_* source files |
| rtwo_top.vhd | Wrapper entity in synth/quartus_top/ to obtain and maintain the synthesis results for the different components in the WPFB, similar as the LOFAR pf_top.vhd. |

Table 10: Preliminary list of the expected WPFB VHDL source and test bench files

## Verification requirements

Requirements:

- The Wideband PFS must be verified by checking that an impulse input signal yields the FIR filter coefficients.

- The Wideband FFT output must be compared to the (ideal) floating point FFT for a full scale, uniform noise input to show that the implementation loss (due to quantization and internal rounding) is conform what can be expected.

- The Wideband PFB must be verified using a sinus signal or a stream of impulses with a period that does not integer divide the FFT size

- All VHDL test benches must self check that the DUT VHDL works OK. If necessary use a golden reference result file for verification.

- All VHDL test benches must support using ‘run –all’ using some tb_end signal to stop the clock.

- The resource usage of the WPFB must be reported in a table (see for example pf_top.vhd in the LOFAR pft2 module).

Table 10 provides a list of existing testbenches for the rTwoSDF and for the LOFAR PPB2 that serve as examples for verifying the WPFB.

| File | Description |
| --- | --- |
| tb_rTwoSDF.vhd | Testbench for rTwoSDF. The testbench can simulate: a) complex uniform noise input from a file generated by MATLAB testFFT_input.m b) impulse input from a manually created file Stimuli b) are useful for visual interpretation of the FFT output, because an impulse at the real input and zero at the imaginary input will result in DC and zero if the pulse occurs at the first sample or in sinus and cosinus wave if the impulse occurs at a later sample. However because the imaginary input is zero this does not cover all internals of the PFT implementation. Therefore stimuli a) are needed to fully verify the PFT. The rTwoSDF output can be verified in two ways: 1) The MATLAB testFFT_output.m can calculate the floating point FFT and compare it with the rTwoSDF implementation output file result. The testFFT_output.m als calculates the SNR value. 2) The rTwoSDF implementation output file is also kept in SVN as golden reference result to allow verification using a file diff command like e.g. WinMerge. This then avoids the need to run MATLAB to verify. The testbench asserts an error when the output does not match the expected output that is read from the golden reference file. The output is also written to a default output file to support offline analysis. |
| tb_pfs.vhd | Testbench for the LOFAR pfs.vhd. It verifies the impulse response of the PFS. |
| tb_pft.vhd | Testbench for the LFOAR pft.vhd. It can verify the output of the PFT for bit-reversed, normal complex or separated outputs. The input real and imaginary signal are selected by uncommenting the signal file name. The output gets automatically verified by checking that the output sample values do not differ too much from the golden reference values that were calculated using MATLAB. The input signal files and the golden reference result files are stored in the tb/data/ directory. The fft_responses_sketch.pdf [5] provides a sketch of the input signals and the expected FFT output signals. |
| tb_pfb2.vhd | Test bench for the polyphase filterbank PFS+PFT2 as used in LOFAR. After the PFB2 impulse response time of 16 FFT slices the 16*1024 taps PFS has flushed undefined internal status, so after that the PFB output subbands reflect the input. The test bench is self checking for: . a = "cosin_1.sig" has frequency 1 * fs/N so will appear in bin 1 . b = "cosin_39.sig" has frequency 39 * fs/N so will appear in bin 39 |
| top.m | The top.m at Lofar/pft2/src/matlab provides a MATLAB simulation of the PFB2. It shows the response of the PFB2 for a sinus input. |

Table 11: List of existing test benches

### Verification approach for WPFS

A FIR filter like the PFS can be verified by checking that the impulse response yields the FIR coefficients [13]. This applies to both the PFS and the WPFS.

### Verification approach for WFFT

For an FFT some fixed noise signal with golden result output verifies all internal states of the FFT as in tb_rTwoSDF.vhd. The noise signal FFT can also be calculated in floating point e.g. using MATLAB to determine the algorithm output as with testFFT_output.m [3, 15]. The difference between the implementation golden result output and the floating-point algorithm output must be sufficiently small [1, 14] and is called the implementation loss. To verify the scaling in the FFT (i.e. no overflow) it is also necessary to use an impulse at t0 and a sinus wave [14]. Using a shifted impulse response e.g. at t1 is also interesting, because that results in a sinus output. The stimuli and expected results for the WFFT are the same as for the PFT, but in a different data format.

### Verification approach for WPFB

The WPFB can be verified as in the LOFAR tb_pfb2.vhd selecting a waveform from a file. However it seems more appropriate to verify the WPFB using a sinus generated with the diag_wideband_wg. Two cases are interesting. First use a sinus that has a period that divides the FFT size N. Second use a sinus with a frequency that falls somewhere between two frequency bins. The WPFB has an impulse response of nof_taps * N samples, so after that the output will become stable.

## Validation requirements

There are no validation requirements other then that the WPFB component must synthesize OK. Hence no reference design needs to be made to validate the WPFB on hardware.

# Deliverables

## Firmware

All source code must be available in the UniBoard SVN repository [12]. The code must compile, simulate and synthesize correctly.

### Not to do (yet)

The following features are not required for the Apertif WPFB, so therefore they are not required and should not be done yet.

- Saving RAM for the reorder function (see section 2.4.1).

- Improving the multiplier usage by using a biplex FFT (see section 2.4.4).

- Investigating rounding instead of truncation in rTwoSDF (see section 2.3.3).

- Scramble the rounding crosstalk that occurs between two real inputs using the (un)switch feature from the LOFAR PFT [6]

- The PFS FIR coefficients for the two real inputs of the PFB are the same, so they could be shared to save memory.

- PFS with multiple ports that can potentially share the FIR coefficients to save memory

- FFT with multiple ports that can potentially share the twiddle factors to save memory

- Support WPFS and WPFB for wideband factors P that are not a power of 2.

- An oversampled or overlap PFB using a downsample factor that is less than the FFT size

## Documentation

The APERTIF WPFB must be documented according to the ASTRON report module document template.