Data Path Packet Module Description

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Eric Kooistra | ASTRON |  |
| Controle / Checked: |  |  |
| Andre Gunst | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Andre Gunst | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature Andre Gunst | ASTRON |  |
| © ASTRON 2011 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Andre Gunst (AG, ASTRON) Eric Kooistra (EK, ASTRON) Daniel van der Schuur (DS, ASTRON) Harm-Jan Pepping (HJP, ASTRON) | Gijs Schoonderbeek (GS, ASTRON) Jonathan Hargreaves (JH, JIVE) Salvatore Pirrucci (SP, JIVE) |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2011-11-19 | Eric Kooistra | Draft. |
| 0.2 | 2012-06-29 | Eric Kooistra | Redefined SOSI frame sync. Added g_use_sosi_err for dp_packet_dec. Added dp_packet_enc_channel_lo component. Added dp_packet_dec_channel_lo component. |

Table of contents:

1	Introduction	4

1.1	Purpose	4

1.2	Scope	4

2	Interface, design and implementation of the components	5

2.1	dp_packet_enc – Encode DP SOSI to DP packet	5

2.2	dp_packet_dec – Decode DP packet to DP SOSI	7

2.3	dp_packet_enc_channel_lo - Encode channel low bits into the CHAN field	9

2.4	dp_packet_dec_channel_lo - Decode channel low bits from the CHAN field	10

3	Verification of the components	11

3.1	DP packet level test bench	11

3.2	PHY level test bench	12

Terminology:

BSN		Block Sequence Number

CHAN		Channel

CRC		Cyclic Redundancy Check

DP		Data Path

eop		end of packet

ERR		Error

ETH		Ethernet

LSBit		Least Significant bit

MSBit		Most Significant bit

PHY		Physical interface

RL		Ready Latency

SFD		Start of Frame Delimiter

SISO		Source in Sink Out

sop		start of packet

SOSI		Source Out Sink In

TLEN		Type/Length

UTH		Uthernet

References:

- “Data Path Packet Interface Specification”, ASTRON-SP-042, E. Kooistra

- “Specification for module interfaces using VHDL records”, ASTRON-RP-380, E. Kooistra

- “DP Streaming Module Description”, ASTRON-RP-382, E. Kooistra

- “Data Path Interface Description”, ASTRON-RP-394, E. Kooistra

- “Timing in the Streaming interface”, ASTRON-RP-481, E. Kooistra

- “Uthernet Interface Specification”, ASTRON-SP-041, E. Kooistra

- “Uthernet (UTH) Module Description”, ASTRON-RP-871, E. Kooistra

# Introduction

## Purpose

This document describes the DP packet encoding and decoding part of the data path (DP) module [3]. The DP contains components to encode the SOSI signals into a DP packet and to decode a DP packet into the SOSI signals. The DP module uses the streaming SOSI and SISO records that were defined in [2]. The DP packet has been specified in [1]. The SISO backpressure ready signal is not (yet) supported at DP packet level.

## Scope

The DP packet encoding and decoding components make the similar DP packetizing components that were defined in [4] obsolete.

# Interface, design and implementation of the components

## dp_packet_enc – Encode DP SOSI to DP packet

### Interface

The dp_packet_enc encodes the SOSI signals into a DP packet as shown below in respectively Figure 1 and Figure 2. The DP packet structure with the CHAN, BSN, DATA and ERR fields has been specified in [1].

Figure 1: dp_packet_enc snk_in

Figure 2: dp_packet_enc src_out

The dp_packet_enc interface parameters and ports are given in respectively Table 1 and Table 2.

| Generic | Type | Description |
| --- | --- | --- |
| g_data_w | natural | >= 1, the data width of the packet |
| g_channel_lo | natural | >= 0, the snk_in.channel[g_channel_lo-1:0] bits are not encoded into the CHAN field, but passed on to src_out.channel instead. This allows using these LSbit channel fields to define different groups of DP packet streams. |

Table 1: dp_packet_enc parameters

| Signal | IO | Type | Description |
| --- | --- | --- | --- |
| rst | IN | std_logic | Reset |
| clk | IN | std_logic | Clock |
| snk_out | OUT | t_dp_siso | SISO: ready |
| snk_in | IN | t_dp_sosi | SOSI: sync, bsn, data, valid, sop, eop, channel, empty, err |
| src_in | IN | t_dp_siso | SISO: ready |
| src_out | OUT | t_dp_sosi | SOSI: data, valid, sop, eop, channel_lo |

Table 2: dp_packet_enc

### Design

The CHAN, BSN and ERR field have a specified fixed width of respectively 16 bit, 48 bit and 16 bit. The packet data width is set by g_data_w and can have any value >= 1. Therefore the number of packet data words that is needed to encode the CHAN, BSN and ERR field depends on g_data_w. For example for g_data_w = 16 the CHAN takes 1 data word, BSN takes 3 data words and the ERR field takes 1 data word [1]. If the number of data words in the input data block marked by the snk_in.valid, snk_in.sop and snk_in.eop is N then the total src_out output packet will have N+5 data words.

The snk_in.channel field is read at the snk_in.sop. The LSBits snk_in.channel[g_channel_lo-1:0] are passed on directly to src_out.channel. The rest of the MSBits of snk_in.channel are transported via the CHAN field.

For framed data the snk_in.sync is active during the entire frame (so for all active snk_in.valid from snk_in.sop to snk_in.eop) and applies to the next frame [5]. The dp_packet_enc reads the snk_in.sync at the snk_in.sop and transports the snk_in.sync as MSBit of the BSN field. The snk_in.bsn is read at the snk_in.sop and transported via the BSN field. If the BSN field just fits in an integer number of packet data words then the input sync will be transported instead of the MSbit 47 of the snk_in.bsn value.

The snk_in.data is transported via the DATA field. The snk_in.data is copied to src_out.data as it is, so any resizing to g_data_w is left to do by the downstream blocks.

The snk_in.err field is read at the snk_in.eop and transported via the ERR field.

The snk_in.empty is valid at the snk_in.eop but ignored, because it is assumed to be implicitly known per channel. Hence the recipient of the packet will know it based on the CHAN field.

In case the CHAN, BSN and/or ERR field do not fit in an integer number of packet data words (of width g_data_w) then the unused MSbits of the MSword are set to 0.

### Implementation

Figure 3 shows the RTL block diagram of the dp_packet_enc implementation. The dp_packet_enc uses dp_hold_input to maintain the input RL of 1 and the output RL of 1 [3].

Figure 3: dp_packet_enc implementation

## dp_packet_dec – Decode DP packet to DP SOSI

### Interface

The dp_packet_dec decodes the SOSI signals from a DP packet as shown below in respectively Figure 4 and Figure 5.

Figure 4: dp_packet_dec snk_in

Figure 5: dp_packet_dec src_out

The dp_packet_enc interface parameters and ports are given in respectively Table 1 and Table 2.

| Generic | Type | Description |
| --- | --- | --- |
| g_data_w | natural | >= 1, the data width of the packet |
| g_channel_lo | natural | >= 0, the snk_in.channel[g_channel_lo-1:0] bits are not encoded into the CHAN field, but passed on to src_out.channel instead. This allows using these LSbit channel number to define different groups of DP packet streams. |
| g_use_sosi_err | boolean | When the DP packet comes from a PHY link then it may have gotten corrupted contents so then it is useful to set g_use_sosi_err to TRUE to get the error status from the PHY link instead of from the received ERR field. |
| g_use_this_siso | boolean | Default use TRUE for best throughput performance. When FALSE then dp_packet_dec does not need to control snk_out and it is ready when the downstream sink is ready, this may ease achieving timing closure. When TRUE then in addition dp_packet_dec can also be ready when it is receiving inter frame gaps or the header to increase the throughput. |

Table 3: dp_packet_dec parameters

| Signal | IO | Type | Description |
| --- | --- | --- | --- |
| rst | IN | std_logic | Reset |
| clk | IN | std_logic | Clock |
| snk_out | OUT | t_dp_siso | SISO: ready |
| snk_in | IN | t_dp_sosi | SOSI: data, valid, sop, eop, channel_lo |
| src_in | IN | t_dp_siso | SISO: ready |
| src_out | OUT | t_dp_sosi | SOSI: sync, bsn, data, valid, sop, eop, channel, empty, err |

Table 4: dp_packet_dec ports

### Design

The DP packet overhead words are stripped.

The src_out.sync is derived from the BSN MSbit and output before the src_out.sop. The src_out.bsn is derived from the BSN field and is valid at the src_out.sop.

The src_out.channel is derived from the CHAN field and is valid at the src_out.sop.

The first src_out.data word is the first DATA field word and is also valid at the src_out.sop. The last src_out.data word is the last DATA field word and is valid at the src_out.eop. The dp_packet_dec can handle DP packets with only one DATA field word, so then the src_out.valid, src_out.sop and src_out.eop

are all active in the same clock cycle.

The src_out.empty is set to 0. The assumption is that the downstream recipient of the src_out will know the actual empty value based on the src_out.channel identifier.

The src_out.err is derived from the ERR field if g_use_sosi_err = false and is valid at the src_out.eop. In case the DP packet was transported over some PHY link with cyclic redundancy check (CRC) protection, then a CRC error at the receiver may be reported into the src_out.err signal instead by using g_use_sosi_err = true.

### Implementation

Figure 6 shows the RTL block diagram of the dp_packet_dec implementation. The dp_packet_dec uses dp_hold_input to maintain the input RL of 1 and the output RL of 1 [3].

The dp_packet_dec uses dp_shiftreg to strip the packet ERR field words from the src_out stream. Therefore the dp_shiftreg contains c_error_len + 1 number of data words, where c_error_len is the number of packet data words used for the ERR field. When the blk_sosi.eop is there the process p_src_out strips these words by setting the src_out.eop at the last payload data word that is just still in the shift register. Together with that src_out.eop it also sets the src_out.err based on the ERR field value that was aggregated by process p_src_err.

Figure 6: dp_packet_dec implementation

## dp_packet_enc_channel_lo - Encode channel low bits into the CHAN field

### Interface

The dp_packet_enc_channel_lo encodes the sosi.channel low bits into the high part of the CHAN field of a DP packet. The snk_in.data must be DP packet data. The [g_channel_lo-1:0] bits of the sosi.channel field get placed into the [high-1:high-g_channel_lo] bits of the CHAN field. The [high] bit of the CHAN field is not used because it is reserved for future use. The dp_packet_enc_channel_lo interface parameters and ports are given in respectively Table 5 and Table 6.

| Generic | Type | Description |
| --- | --- | --- |
| g_data_w | natural | >= 1, the data width of the packet |
| g_channel_lo | natural | Number of channel bits. Pre-conditions: - g_channel_lo+1 <= g_data_w - g_channel_lo+1 <= c_dp_packet_channel_w = 16 - g_channel_lo+1 <= c_dp_packet_channel_w - currently used CHAN width |

Table 5: dp_packet_enc_channel_lo parameters

| Signal | IO | Type | Description |
| --- | --- | --- | --- |
| snk_out | OUT | t_dp_siso | SISO: ready |
| snk_in | IN | t_dp_sosi | SOSI: data, valid, sop, eop, channel_lo |
| src_in | IN | t_dp_siso | SISO: ready |
| src_out | OUT | t_dp_sosi | SOSI: data, valid, sop, eop |

Table 6: dp_packet_enc_channel_lo ports

### Design

The DP packet CHAN field occurs at the sop. By placing the channel_lo bits in the high part of the CHAN field this dp_packet_enc_channel_lo can combinatorially modifiy the CHAN data at the sop. For small g_data_w the CHAN field can cover multiple data words, therefore using the low part of the CHAN field would imply shifting in the g_channel_lo bits and would require more logic including registers to modify the entire CHAN field. The snk_in channel is passed on unmodified to the src_out.channel, because DP packets typically do not use the channel field, so the g_channel_lo bits are the only valid bits in the channel field and therefore they may as well remain, rather then being shifted out.

### Implementation

The dp_packet_enc_channel_lo is a combinatorial component.

## dp_packet_dec_channel_lo - Decode channel low bits from the CHAN field

### Interface

The dp_packet_dec_channel_lo decodes the sosi.channel low bits from the high part of the CHAN field of a DP packet. The snk_in.data must be DP packet data. The [high-1:high-g_channel_lo] bits of the CHAN field get set to 0 and placed into the [g_channel_lo-1:0] bits of the sosi.channel field. The dp_packet_dec_channel_lo interface parameters and ports are given in respectively Table 7 and Table 8.

| Generic | Type | Description |
| --- | --- | --- |
| g_data_w | natural | >= 1, the data width of the packet |
| g_channel_lo | natural | Number of channel bits. |

Table 7: dp_packet_dec_channel_lo parameters

| Signal | IO | Type | Description |
| --- | --- | --- | --- |
| rst | IN | std_logic | Reset |
| clk | IN | std_logic | Clock |
| snk_out | OUT | t_dp_siso | SISO: ready |
| snk_in | IN | t_dp_sosi | SOSI: data, valid, sop, eop |
| src_in | IN | t_dp_siso | SISO: ready |
| src_out | OUT | t_dp_sosi | SOSI: data, valid, sop, eop, channel_lo |

Table 8: dp_packet_dec_channel_lo ports

### Design

The sosi.channel field must be valid during the entire frame, so not only at the sop. Therefore it is necessary to use channel_lo_hold and to have a clock and reset.

### Implementation

The dp_packet_dec_channel_lo decodes combinatorialy.

# Verification of the components

## DP packet level test bench

Figure 7 shows the tb_dp_packet testbench that verifies dp_packet_enc and dp_packet_dec together. The two main stimuli for a streaming interface component are:

- upstream source enable

- downstream sink ready.

These stimuli can active, random or pulsed. The tb_dp_packet uses the procedures described in [3] to generate blocks of data for the dp_packet_enc sink and to verify that these blocks of data arrive properly at the dp_packet_dec source. The proc_dp_verify_data() is used to verify the sosi.data, but also the sosi..channel, sosi.bsn and the sosi.err. The expected_rx_* signals in Figure 7 are used to verify that the test as run at all. When the stimuli have finished then signal tb_end stops the testbench clock to automatically stop the simulation.

Figure 7: tb_dp_packet testbench for dp_packet_enc and dp_packet_dec

The tb_tb_dp_packet multi-testbench instantiates testbench tb_dp_packet several times with different generic settings to perform regression tests on the dp_packet_enc and dp_packet_dec.

The dp_packet_enc_channel_lo.vhd and dp_packet_dec_channel_lo.vhd are verified in tb_dp_distribute.vhd.

## PHY level test bench

Figure 8 shows the tb_uth_dp_packet testbench that verifies the transport of DP packets over an Uthernet [6] PHY link. Note that the scheme of Figure 8 models a practical use case for communicating multiple SOSI streams over a single PHY link.

Figure 8: tb_uth_dp_packet testbench for dp_packet and UTH

The tb_uth_dp_packet testbench uses the uth_tx and uth_rx components from [7]. The p_stimuli process generates c_nof_tlen different types of packets. For each packet type the p_stimuli process generates c_nof_input parallel input streams. All input streams are identified by the channel number. The low part of the channel number reflects the different packet types and gets mapped on different TLEN type field values of the Uthernet protocol. The high part of the channel number gets mapped on the DP packet CHAN field. The distinction is set by the DP packet encoding and decoding parameter g_channel_lo = c_nof_tlen_w. The testbench uses dp_mux and dp_demux to perform the multiplexing and de-multiplexing to and from a single stream.

All p_stimuli(I)(J) get the same input enable stimulus and all out_siso_2arr(I)(J) get the same sink ready stimulus. These flow control stimuli can be active, random or pulsed dependent on g_in_en and g_out_ready. By means of setting c_loopback_mux = TRUE or c_loopback_packet = TRUE the same test bench can also run looping back already at this interfaces as shown in Figure 8. Default both loopback constants are FALSE. Using c_loopback_mux = TRUE and always active flow control stimuli via g_in_en and g_out_ready reveals that the muliplexers and demultiplexers can pass on the framed data without gaps.

The tb_tb_uth_dp_packet multi-testbench instantiates testbench tb_uth_dp_packet several times with different g_data_w and with different flow control.