DIAG Module Description

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Eric Kooistra Harm Jan Pepping | ASTRON | 24 February 2012 |
| Controle / Checked: |  |  |
| Andre Gunst | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Andre Gunst | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature Andre Gunst | ASTRON |  |
| © ASTRON 2011 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Andre Gunst Eric Kooistra Daniel van der Schuur | Gijs Schoonderbeek Sjouke Zwier Harro Verkouter (JIVE) Jonathan Hargreaves (JIVE) Salvatore Pirruccio (JIVE) |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2012-03-07 | Harm Jan Pepping | Creation |

Table of contents:

1	Introduction	5

1.1	Purpose	5

2	DIAG components	6

2.1	mms_diag_block_gen component	6

2.1.1	mms_diag_block_gen parameters	6

2.1.2	mms_diag_block_gen interface signals	6

2.1.3	mms_diag_block_gen software interface	7

2.1.4	mms_diag_block_gen mode of operation	8

2.1.5	mms_diag_block_gen design	9

Terminology:

DIAG		Diagnostics (VHDL module)

MM		Memory Mapped

ST 		Streaming

UNB		https://svn.astron.nl/Uniboard_FP7/Uniboard/trunk/, the Uniboard SVN repository

References:

- ‘DP Streaming Module Description’, ASTRON-RP-382, Eric Kooistra

- ‘Detailed Design of the Digital Beamformer System for Apertif’, ASTRON-RP-413, G. Schoonderbeek, A. Gunst, E. Kooistra

- Digital Beamformer Module for APERTIF, ASTRON-RP-517, H.J. Pepping

- $UNB/Firmware/modules/common/

- $UNB/Firmware/modules/Lofar/st/

# Introduction

## Purpose

The DIAG module contains a set of components that can be used for performing diagnostics. The DIAG module is located in the Uniboard RadioNet FP7 SVN repository at $UNB/Firmware/modules/Lofar/diag/.

# DIAG components

## mms_diag_block_gen component

The mms_diag_block_gen component is a packet generator that can send parameterized packets in the DP streaming format as defined in [1]. It is also possible to specify the period between two consecutive packets. The waveform data is stored in memory and can be uploaded via a mm interface.

### mms_diag_block_gen parameters

The mms_diag_block_gen module has a list of generics that determine the number of outputs and the size of the waveform memory.  The generics are listed in Table 1.

| Generic | Type | Value | Description |
| --- | --- | --- | --- |
| nof_output_streams | POSITIVE | 16 | The number of data streams that the packet generator has to provide. |
| buf_dat_w | POSITIVE | 32 | Specifies the bitwidth of the memory that holds the waveform data. |
| buf_addr_w | POSITIVE | 7 | This number defines the address width of each waveform memory. |
| sim | BOOLEAN | FALSE | A generic to determine if the mms_diag_block_gen unit is used in simulation or used in synthesis. The sim generic is used to locate the files that contain the initial content for the waveform memories. |

Table 1 Mms_diag_block_gen parameters

### mms_diag_block_gen interface signals

The interface signals of the mms_diag_block_gen module are shown in Figure 1 and Table 2 lists a detailed overview of each signal.

Figure 1 mms_diag_block_gen interface signals

| Interface | Type | Size or Span | Description |
| --- | --- | --- | --- |
| mm_bg_data_mosi | t_mm_mosi | nof_output_streams*2 buf_addr_w-1 | The mosi part of the mm interface that contains the memory locations for the waveform data of every stream. |
| mm_bg_data_miso | t_mm_miso | nof_output_streams*2 buf_addr_w-1 | The miso part of the mm interface that contains the memory locations for the waveform data of every stream. |
| mm_bg_ctrl_mosi | t_mm_mosi | 0x8 | The mosi part of the mm interface that contains the control registers for the mms_diag_block_gen unit. |
| mm_bg_ctrl_miso | t_mm_miso | 0x8 | The miso part of the mm interface that contains the control registers for the mms_diag_block_gen unit. |
| out_sosi_arr | t_dp_sosi_arr | nof_output_streams | Array of data streams that contain the configured blocks with the waveform data. |
| dp_clk | std_logic | na | Datapath clock |
| dp_rst | std_logic | na | Datapath reset |
| mm_clk | std_logic | na | Memory mapped interface clock |
| mm_rst | std_logic | na | Memory mapped interface reset |

Table 2 mms_diag_block_gen interface signals

### mms_diag_block_gen software interface

The software interface of the mms_diag_block_gen module consists of two address spans: a control span and a data span. The control span is used to configure and control the streams and the data span is used to upload the data that represents the waveform.

#### mms_diag_block_gen control span

The control registers of the mms_diag_block_gen apply to all output streams. The mms_diag_block_gen control span contains the following registers:

| Name | Address (words) | Bit Width | Read/ Write | Description |
| --- | --- | --- | --- | --- |
| enable | 0x0 | 1 | r/w | The waveform enable register. A logic ‘1’ will enable the data streams, starting with the assertion of sop. A logic ‘0’ will stop the outgoing data streams after the current packet is finished. The stream will end with the assertion of eop. |
| samples_per_packet | 0x1 | 16 | r/w | This register specifies the number of samples in a packet. |
| gapsize | 0x2 | 16 | r/w | Register represents the time between two consecutive packets. Time is expressed in dp_clk cycles. |
| blocks_per_sync | 0x3 | 16 | r/w | This register specifies the sync interval in number of blocks. |
| mem_low_address | 0x4 | 8 | r/w | Register contains the pointer to the first address of the memory from which the waveform generator reads the data. |
| mem_high_address | 0x5 | 8 | r/w | Register contains the pointer to the last address of the memory from which the waveform generator reads the data. |
| bsn_init_low | 0x6 | 32 | r/w | The 32 lsb’s of the initial value of the bsn. |
| bsn_init_high | 0x7 | 16 | r/w | The 16 msb’s of the initial value of the bsn. |

Table 3 mms_diag_block_gen control span

#### mms_diag_block_gen data span

The mms_diag_block_gen data span contains the entrance to the memory locations that hold the data that represent the waveforms. The number of registers is defined by nof_output_streams*2buf_addr_w. The mms_diag_block_gen module provides complex data to the output-streams and therefore complex data is stored in the memory. The format is shown in Figure 2.

Figure 2 Waveform data format

The registers of the mms_diag_block_gen data span are listed in Table 4. The shown registers are based on a mms_diag_block_gen instantiation where nof_output_streams = 16, buf_dat_w = 32 and buf_addr_w = 7. Note that not all registers are listed in order to save space.

| Name | Address (words) | Bit Width | Read/ Write | Description |
| --- | --- | --- | --- | --- |
| bg_str0_d0 | 0x0 | 32 | r/w | Complex data word 0 of stream 0. |
| bg_str0_d1 | 0x1 | 32 | r/w | Complex data word 1 of stream 0. |
| bg_str0_d2 | 0x2 | 32 | r/w | Complex data word 2 of stream 0. |
| ----------------------- | ------ | --- | ----- | ------------------------------------------------------------ |
| bg_str0_d126 | 0x7E | 32 | r/w | Complex data word 126 of stream 0. |
| bg_str0_d127 | 0x7F | 32 | r/w | Complex data word 127 of stream 0. |
| bg_str1_d0 | 0x80 | 32 | r/w | Complex data word 0 of stream 1. |
| bg_str1_d1 | 0x81 | 32 | r/w | Complex data word 1 of stream 1. |
| bg_str1_d2 | 0x82 | 32 | r/w | Complex data word 2 of stream 1. |
| ----------------------- | ------ | --- | ----- | ------------------------------------------------------------ |
| bg_str1_d126 | 0xFE | 32 | r/w | Complex data word 126 of stream 1. |
| bg_str1_d127 | 0xFF | 32 | r/w | Complex data word 127 of stream 1. |
| bg_str2_d0 | 0x100 | 32 | r/w | Complex data word 0 of stream 2. |
| bg_str2_d1 | 0x101 | 32 | r/w | Complex data word 1 of stream 2. |
| ----------------------- | ------ | --- | ----- | ------------------------------------------------------------ |
| ----------------------- | ------ | --- | ----- | ------------------------------------------------------------ |
| ----------------------- | ------ | --- | ----- | ------------------------------------------------------------ |
| bg_str14_d126 | 0x77E | 32 | r/w | Complex data word 126 of stream 14. |
| bg_str14_d127 | 0x77F | 32 | r/w | Complex data word 127 of stream 14. |
| bg_str15_d0 | 0x780 | 32 | r/w | Complex data word 0 of stream 15. |
| bg_str15_d1 | 0x781 | 32 | r/w | Complex data word 1 of stream 15. |
| bg_str15_d2 | 0x782 | 32 | r/w | Complex data word 2 of stream 15. |
| ----------------------- | ------ | --- | ----- | ------------------------------------------------------------ |
| bg_str15_d126 | 0x7FE | 32 | r/w | Complex data word 126 of stream 15. |
| bg_str15_d127 | 0x7FF | 32 | r/w | Complex data word 127 of stream 15. |

Table 4 mms_diag_block_gen data span

### mms_diag_block_gen mode of operation

The way to operate the mms_diag_block_gen module is straight forward. First upload the data of the waveforms, then configure the stream and finally enable the streams by writing a logic ‘1’ to the “enable” register. Figure 3 shows a diagram that shows the timing of a single output stream. The stream is configured with the following values:

enable = 1

samples_per_packet = 8

blocks_per_sync = 2

gapsize = 8

mem_low_adrs = 0

mem_high_adrs = 11

bsn_init_low = 41

bsn_init_high = 0

Figure 3 Timing diagram mms_diag_block_gen

### mms_diag_block_gen design

A schematic overview of the mm_diag_block_gen design is shown in Figure 4 (nof_output_streams = 4). The common_mem_mux (common_lib) module is used to connect multiple dual port rams in case the mms_diag_block_gen module is configured to have more than one output stream (in Figure 4 only the first element of the array is drawn). A dual ported ram instance (also from the common_lib) functions as memory buffer for the data that will be send in the packets.

Figure 4 mms_diag_block_gen schematic overview

The diag_block_gen (diag_lib) responses to the settings of the bg_ctrl input port and when enabled it reads data from the dual ported ram and composes blocks on the out_sosi_arr that complies with the DP streaming format.  The diag_block_gen_reg unit receives the control values via the mm interface and provides these values to the bg_ctrl record. The bg_ctrl record is defined in the diag_pkg (diag_lib).