Data Path Packet Interface Specification

|  | Organisatie / Organization | Datum / Date |
| --- | --- | --- |
| Auteur(s) / Author(s): |  |  |
| Eric Kooistra | ASTRON |  |
| Controle / Checked: |  |  |
| Andre Gunst | ASTRON |  |
| Goedkeuring / Approval: |  |  |
| Andre Gunst | ASTRON |  |
| Autorisatie / Authorisation: |  |  |
| Handtekening / Signature Andre Gunst | ASTRON |  |
| © ASTRON 2011 All rights are reserved. Reproduction in whole or in part is prohibited without written consent of the copyright owner. |  |  |

Distribution list:

| Group: | Others: |
| --- | --- |
| Andre Gunst (AG, ASTRON) Eric Kooistra (EK, ASTRON) Daniel van der Schuur (DS, ASTRON) Harm-Jan Pepping (HJP, ASTRON) | Gijs Schoonderbeek (GS, ASTRON) Jonathan Hargreaves (JH, JIVE) Salvatore Pirrucci (SP, JIVE) |

Document history:

| Revision | Date | Author | Modification / Change |
| --- | --- | --- | --- |
| 0.1 | 2011-11-28 | Eric Kooistra | Draft. |
| 0.2 | 2012-06-29 | Eric Kooistra | Clarified definition of SOSI frame sync in BSN field. |

Table of contents:

1	Introduction	4

2	DP packet structure	5

2.1	Data width	5

2.2	Packet length	6

2.3	CHAN field	6

2.4	BSN field	6

2.5	DATA field	6

2.6	ERR field	6

3	Transporting DP packets	7

4	Appendix : Streaming interface record definition	8

Terminology:

BSN		Block Sequence Number

CHAN		Channel

CRC		Cyclic Redundancy Check

DP		Data Path

eop		end of packet

ERR		Error

ETH		Ethernet

Im		Imaginary

LSBit		Least Significant bit

LVDS		Low Voltage Differential Signaling

MSBit		Most Significant bit

PHY		Physical interface

Re		Real

SISO		Sink In Source Out

sop		start of packet

SOSI		Source Out Sink In

UDP		User Datagram Protocol

References:

- “Specification for module interfaces using VHDL records”, ASTRON-RP-380, E. Kooistra

- “Data Path Interface Description”, ASTRON-RP-394, E. Kooistra

- “Uthernet Interface Specification”, ASTRON-SP-041, E. Kooistra

- http://en.wikipedia.org/wiki/Ethernet_frame

# Introduction

This document specifies the Data Path (DP) Packet Interface. The DP packet interface supports the transfer of all DP streaming interface signals in a DP packet. The DP streaming interface signals are defined in [1] and listed in appendix 4 for convenience. A DP packet can for example be transported over a point-to-point link by putting it in the payload of an Uthernet packet [2] or it can for example be transported via a network as UDP payload in an Ethernet frame [4].

This DP Packet specification makes the DP frame interface that was defined in [2] obsolete.

# DP packet structure

Figure 1 shows the complete DP packet. The header contains 2 fields (CHAN and BSN), the payload DATA field contains at least 1 data word and the tail contains 1 field (ERR).

Figure 1: DP packet structure

The widths of the DP packet CHAN, BSN and ERR fields are fixed and defined in Table 1. The DP packet DATA field word width is equal to the packet data word width w.

| Field | Width [bits] |
| --- | --- |
| CHAN | 16 |
| DATA | ≥ 1 |
| BSN | 48 |
| ERR | 16 |

Table 1: DP packet field widths definition

The number of data words per DP packet field depends on the data word width w and equals ceil(DP packet field width / data word width), as shown in Table 2. For example if the data width w is 11 then the CHAN field of 16 bits uses 2 data words and CHAN[15:11] is encoded first in data[4:0] and CHAN[10:0] is encoded in the next data[10:0]. The unused MSbits in the data are ‘0’. Similar for the BSN field and the ERR field.

| Data word width w [bits] | CHAN | BSN | DATA | ERR | Comment |
| --- | --- | --- | --- | --- | --- |
| 0 | - | - | - | - | Illegal data width |
| ≥ 1 | ceil(16/w) | ceil(48/w) | N * 1 | ceil(16/w) |  |
| 8, 9 | 2 | 6 | N * 1 | 2 |  |
| 10, 11 | 2 | 5 | N * 1 | 2 |  |
| 12 : 15 | 2 | 4 | N * 1 | 2 |  |
| 16 : 23 | 1 | 3 | N * 1 | 1 |  |
| 24 : 47 | 1 | 2 | N * 1 | 1 |  |
| ≥ 48 | 1 | 1 | N * 1 | 1 |  |

Table 2: DP packet field lengths for the supported data widths

The DP packet header contains the CHAN field and the BSN field. The packet DATA field must contain at least 1 data word, so N ≥ 1. The DP packet tail contains the ERR field. The total DP packet overhead depends on the data width w, and is at least 3 words.

## Data width

The DP packet data width can have any width w ≥ 1. The data bits are indexed as [w-1:0] and the MSbit [w-1] is transferred first. In case the data word contains multiple bytes then the data word is transferred as big-endian, so MSByte first.

## Packet length

The DP packet transports the DP data information of one or multiple data streams. All these data streams all have a constant and known data length and therefore the sosi.channel number or the mere allocation of resources for a DP packet implicitly represent the sosi.sop, sosi.eop and sosi.empty signals.

## CHAN field

The CHAN field represents the sosi.channel that identifies a data stream.

The MSBit [15] of the CHAN field is reserved for future use. It may be used in future for flow control at DP packet level.

## BSN field

The BSN field represents the sosi.sync and sosi.bsn. The sosi.sync bit is transported as MSBit [w-1] of the first data word of the BSN field. If the BSN field of 48 bits just fits in an integer number of data words (i.e. for w = 1, 2, 3, 4, 6, 8, 12, 16, 24 or 48), then the sosi.sync will replace sosi.bsn[47]. A frame with an active sosi.sync is the last frame in a sync interval, so the next frame will be the first frame of a sync interval.

## DATA field

The DATA field transports the sosi.data or the sosi.re and sosi.im complex data.

## ERR field

The ERR field transports the sosi.err.

# Transporting DP packets

For internal handling of the DP packets it is necessary to also define the valid, start of packet (sop) and end of packet (eop) signals [1] as shown in Figure 2. The DP packet may have not-valid data words between the sop and the eop during the packet.

Figure 2: DP packet with control signals

Multiple different DP data streams can be grouped based on the data length (defined by the combination of sosi.sop, sosi.eop and sosi.empty). Within such a group there can be one or multiple DP data streams. The groups and the streams within a group can be represented as two levels of sosi.channel numbering. Dependent on the properties of the PHY interface packet structure the DP packet CHAN field may represent only the streams within a group or the different groups as well. The sosi.channel part for identifying streams within a group is always transported via the DP packet CHAN field. If the sosi.channel part for identifying a group is not part of the DP packet CHAN field then it is present as a sosi.channel signal as shown in Figure 2.

- For UDP/IP over Ethernet [3] the group part of the sosi.channel can be transported via the CHAN field, because UDP can transport different payload lengths per UDP port. Alternatively the group part of the sosi.channel field may be used to map on different UDP port numbers.

- For Uthernet [4] the group part of the sosi.channel field can not be transported via the DP packet CHAN field, because Uthernet only supports a single payload length per TLEN type. Therefore for Uthernet it is necessary to map the group part of the sosi.channel number on the Uthernet header TLEN field.

If the DP packet gets corrupted during transport over the PHY link then that gets reported via the sosi.err signal as shown in Figure 2. The application can use this sosi.err signal to flag this error in the DP packet ERR field.

# Appendix : Streaming interface record definition

TYPE t_dp_siso IS RECORD  -- Source In or Sink Out

ready    : STD_LOGIC;

nc       : STD_LOGIC;   -- not connect

END RECORD;

TYPE t_dp_sosi IS RECORD  -- Source Out or Sink In

sync     : STD_LOGIC;

bsn      : STD_LOGIC_VECTOR(c_dp_stream_bsn_w-1 DOWNTO 0);      -- 48

data     : STD_LOGIC_VECTOR(c_dp_stream_data_w-1 DOWNTO 0);     -- 256

re       : STD_LOGIC_VECTOR(c_dp_stream_dsp_data_w-1 DOWNTO 0); -- 64

im       : STD_LOGIC_VECTOR(c_dp_stream_dsp_data_w-1 DOWNTO 0); -- 64

valid    : STD_LOGIC;

sop      : STD_LOGIC;

eop      : STD_LOGIC;

empty    : STD_LOGIC_VECTOR(c_dp_stream_empty_w-1 DOWNTO 0);    -- 8

channel  : STD_LOGIC_VECTOR(c_dp_stream_channel_w-1 DOWNTO 0);  -- 16

err      : STD_LOGIC_VECTOR(c_dp_stream_error_w-1 DOWNTO 0);    -- 16

END RECORD;